Рефераты статей, 
опубликованных в № 2 журнала

УДК 628.335.1

Разработка и применение отстойников-флокуляторов для очистки оборотной воды металлургических заводов России и Украины. Галкин Ю. А. – С. 5.

Рассказано о разработке научно-проектной фирмой «ЭКО-ПРОЕКТ» и внедрении на металлургических предприятиях России и Украины серии компактных высокоэффективных отстойников-флокуляторов, предназначенных для очистки оборотной воды технологических агрегатов, промышленно-ливневых сточных вод и водоподготовки. Применение одноступенчатых схем глубокой очистки сточных вод вместо двухступенчатых на отстойниках-флокуляторах в комплексе с новой технологией гравитационного обезвоживания получаемого в них осадка позволяет принципиально улучшить все технико-экономические и экологические показатели водного хозяйства металлургических заводов.

Ключевые слова: заводы черной металлургии, водное хозяйство, отстойник-флокулятор, глубокая очистка оборотной воды, гравитационное обезвоживание осадка.

УДК 696.4:662.997

Солнечные мобильные модульные установки горячего водоснабжения. Семенов И. Е., Рыженко С. Н. – С. 12.

Использование солнечной энергии для горячего водоснабжения является наиболее простым в технологическом и наиболее прогрессивным в практическом плане направлением. Предложены новые конструкции плоских солнечных коллекторов и баков-аккумуляторов для установок солнечного горячего водоснабжения. Разработана и изготовлена высокоэффективная мобильная модульная установка солнечного горячего водоснабжения.

Ключевые слова: солнечный коллектор, бак-аккумулятор, солнечное излучение, модуль коллекторный, мобильная установка.

УДК 628.162:662.997

Повышение эффективности использования геотермальной воды для горячего водоснабжения. Ахмедов Г. Я. – С. 18.

Представлены результаты исследований фракционного состава песка, поступающего попутно с геотермальной водой с апшеронского, караганского и чокракского водоносных горизонтов, а также его влияния на кинетику выделения твердой фазы карбоната кальция из растворов геотермальных вод. Приводится проектная схема эффективного использования геотермальных вод для горячего водоснабжения и отопления.

Ключевые слова: геотермальная вода, песок, обработка воды, водоснабжение, солеотложение, взвешенные вещества, карбонат кальция.

УДК 628.32

Прогрессивные технологии очистки сточных вод в стесненных условиях городской застройки и ограничения энергопотребления. Чурбанова И. Н., Певнев С. Г., Дятлова Т. В. – С. 25.

Представлена технологическая схема глубокой биологической очистки сточных вод в условиях ограниченности территории застройки и минимизации энергопотребления на технологические нужды. Предложенная схема очистки позволяет проектировать и модернизировать станции любой производительности с гарантией достижения нормативных показателей очищенных сточных вод.

Ключевые слова: механическая очистка, глубокая биологическая очистка, биофильтр, биореактор, гидроциклон, регулирование расхода сточных вод.

УДК 628.162.8

Внедрение двухступенчатой схемы обеззараживания воды на водопроводных станциях Санкт-Петербурга. Кинебас А. К., Нефедова Е. Д., Бекренев А. В., Яковлев В. Ю. – С. 36.

Рассказано о модернизации систем обеззараживания воды на водопроводных станциях Санкт-Петербурга с использованием гипохлорита натрия и сульфата аммония. Это позволило полностью ликвидировать угрозу поражения территории и населения города высокотоксичными веществами (газообразными хлором и аммиаком) вследствие возможных аварий на промышленных площадках ГУП «Водоканал Санкт-Петербурга» и при транспортировке сжиженного хлора и растворов аммиака, а также исключить воздействие хлора и аммиака на окружающую среду при разгерметизации оборудования. Внедрение технологии обеззараживания воды ультрафиолетовым облучением способствовало реализации концепции множественных барьеров при обеззараживании питьевой воды на водопроводных станциях города.

Ключевые слова: обеззараживание, питьевая вода, сжиженный хлор, гипохлорит натрия, аммиачная вода, сульфат аммония, ультрафиолетовое облучение.

УДК 628.11.004.6

Использование подземных вод для резервного водоснабжения Санкт-Петербурга при чрезвычайных ситуациях. Нефедова Е. Д., Боровицкая Е. Ю. – С. 44.

Рассказано о реализации в Санкт-Петербурге «Целевой программы по организации резервного водоснабжения Санкт-Петербурга при чрезвычайных ситуациях мирного времени и на особый период за счет эксплуатационных запасов подземных вод». Использование имеющихся на территории города подземных источников позволит ГУП «Водоканал Санкт-Петербурга» обеспечить водоснабжение жителей мегаполиса при чрезвычайных ситуациях.

Ключевые слова: подземные воды, резервное водоснабжение, чрезвычайная ситуация, скважинные водозаборы.

УДК 628.16.067.1

Конструктивные особенности и опыт эксплуатации пористых дренажных систем в скорых фильтрах. Филатов А. И., Шаповалов В. И., Райкин Л. Л. – С. 48.

Изложены конструктивные особенности и многолетний опыт эксплуатации пористых дренажных систем двух типов в скорых фильтрах. Дренажи обеспечивают высокую степень равномерности промывки по площади фильтров.

Ключевые слова: фильтры, питьевая вода, дренажная система, зернистая загрузка, гранулы, пористый слой.

УДК 628.162

Совершенствование расчетной модели функционирования кольцевой водопроводной сети. Гальперин Е. М. – С. 51.

Рассмотрены вопросы совершенствования расчетной модели функционирования кольцевой водопроводной сети с учетом среднестатистических данных о ее работе. Предложено внести коррективы в нормативы проектирования систем водоснабжения, дополнив их выполнением надежностных расчетов кольцевой водопроводной сети.

Ключевые слова: водопроводная сеть, подача воды, напор, надежность, отказ, герметичность, авария, расчетная модель.

УДК 628.16.085

Модернизация процесса вторичного обеззараживания воды в системе очистки (на примере р. Амура). Гусева О. А., Герасимов М. М., Смирнов А. Д. – С. 59.

Представлены результаты исследования различных режимов обеззараживания воды (хлорирования, ультрафиолетового облучения) на специальном пилотном испытательном комплексе. Приведены данные вирусологического и бактериологического анализа воды в условиях фонового и высокого микробиологического загрязнения р. Амура.

Ключевые слова: технологическая схема очистки, хлорирование, ультрафиолетовое обеззараживание, микробиологические показатели, вирусы, высокое микробиологическое загрязнение.

УДК 628.16.32.004.658.5

Информационные технологии в ГУП «Водоканал Санкт-Петербурга». Касаткин Ф. Ю., Костылев С. В. – С. 66.

Для реализации оперативного управления предприятием и своевременного решения производственных задач в ГУП «Водоканал Санкт-Петербурга» была создана Корпоративная цифровая сеть с интеграцией служб, представляющая собой смешанную структуру, состоящую из собственных магистральных волоконно-оптических каналов, радиорелейных линий передачи данных и корпоративной телефонии, а также арендованных каналов передачи данных ведущих операторов Санкт-Петербурга.

Ключевые слова: информационные технологии, телефония, радиосвязь, передача данных.

Abstracts of the Articles Published in Issue № 2

UDC 628.335.1

Development and the Use of Settling Tank-Flocculators for Treatment of Recirculated Water at Metallurgical Works of Russia and Ukraine. Galkin Yu. A. – P. 5.

It is told about the development by the scientific-engineering firm EKO-PROJECT and introduction at the metallurgical enterprises of Russia and Ukraine of a series of compact, highly efficient settling tank-flocculators intended for treatment of recirculated water of technological units, industrial-storm wastewater and water conditioning. Application of one-stage schemes of advanced wastewater treatment instead of two-level ones at settling tank-flocculators in a complex with new technology of gravitational dewatering of sludge produced therein allows to improve essentially all technical, economic and ecological indicators of water management of metal works.

Key words: ferrous metallurgy factories, water management, settling tank-flocculator, advanced treatment of recirculated water, gravitational dewatering of sludge.

UDC 696.4:662.997

Solar Mobile Module Plants of Hot Water Supply. Semenov I. E., Ryzhenko S. N. – P. 12.

The use of solar energy for hot water supply is the simplest trend technologically and the most progressive one practically. New designs of flat solar collectors and hot water tank-accumulators for solar hot water supply plants are proposed. A highly efficient mobile module plant of solar hot water supply has been developed and manufactured.

Key words: solar collector, tank-accumulator, solar radiation, collector module, mobile plant.

UDC 628.162:662.997

Enhancement of Efficiency of the Use of Geothermal Water for Hot Water Supply. Akhmedov G. Ya. – P. 18.

Results of the study of the fractional composition of sand coming by with geothermal water from Apsheron, Karagan and Chokrak aquifers, and also its influence on kinetics of the precipitation of the solid phase of calcium carbonate from of geothermal water solutions are presented. A design scheme of effective use of geothermal water for hot water supply and heating is resulted.

Key words: geothermal water, sand, water treatment, water supply, salt precipitation, suspended substances, calcium carbonate.

UDC 628.32

Progressive Techniques of Wastewater Treatment under Restricted Conditions of Urban Development and Power Consumption Limitation. Churbanova I. N., Pevnev S. G., Dyatlova T. V. – P. 25.

A flow chart of advanced biological treatment of wastewater under restricted conditions of a development territory and minimization of power consumption for technological needs is presented. This flow chart makes it possible to design and modernize the stations of any output ensuring the achievement of standards of wastewater treated.

Key words: mechanical treatment, advanced biological treatment, biofilter, bioreactor, hydrocyclone, regulation of wastewater flow rate.

UDC 628.162.8

Introduction of Two-Stage Scheme of Water Disinfection at Waterworks of St. Petersburg. Kinebas A. K., Nefedova Е. D., Bekrenev A. V., Yakovlev V. Yu. – P. 36.

It is told about the modernization of systems of water disinfection at waterworks of St. Petersburg using sodium hypochlorite and ammonium sulphate. It has allowed to liquidate completely the threat of affecting the city’s territory and population with highly toxic substances (gaseous chlorine and ammonia) owing to possible failures at industrial sites of the State Unitary Enterprise Vodokanal Sankt-Peterburga and at transportation of liquefied chlorine and ammonia solutions, and also to exclude chlorine and ammonia influence on the environment in the case of equipment depressurization. Introduction of the technology of water disinfection with ultra-violet irradiation has favored the realization of the concept of multiple barriers at potable water disinfecting at the city’s waterworks.

Key words: disinfection, potable water, liquefied chlorine, sodium hypochlorite, ammonia water, ammonium sulphate, ultra-violet irradiation.

UDC 628.11.004.6

The Use of Ground Water for Reserve Water Supply of St. Petersburg at Emergency Situations. Nefedova E. D., Borovitskaya E. Yu. – P. 44.

It is told about the realization of The Target Program of Organization of Reserve Water Supply of St. Petersburg at Emergency Situations in Peacetime and for a Special Period at the Cost of Operation Reserves of Ground Water. The use of underground sources available on the city’s territory will make it possible for the State Unitary Enterprise Vodokanal Sankt-Peterburga to guarantee the water supply of the megalopolis population at emergency situations.

Key words: ground water, reserve water supply, emergency situation, borehole water intakes.

UDC 628.16.067.1

Design Features and Experience of Porous Drainage Systems Operation in High-Rate Filters. Filatov A. I., Shapovalov V. I., Raykin L. L. – P. 48.

Design features and multiyear experience in operation of porous drainage systems of two types in high-rate filters are expounded. Drainages ensure a high level of uniformity of washing across the filters’ area.

Key words: filters, potable water, drainage system, granular media, granules, porous layer.

UDC 628.162

Improvement of a Design Model of Operation of a Ring Water Supply Network. Galperin E. M. – P. 51.

Issues of improvement of the design model of operation of the ring water supply network taking into account statistical data on its operation are considered. It is proposed to make corrections in the standards of water supply systems designing and supplement them with execution of reliability calculations of the ring water supply network.

Key words: water supply network, water supply, head, reliability, failure, air-tightness, damage, designed model.

UDC 628.16.085

Modernization of the Process of Secondary Water Disinfecting in the Treatment System (on the Example of the Amur). Guseva O. A., Gerasimov M. M., Smirnov A. D. – P. 59.

Results of the research of various modes of water disinfecting (chlorination, the UV-irradiation) at a special pilot test complex are submitted. Data on the virological and bacteriological analysis of water under the conditions of background and high microbiological pollution of the Amur are presented.

Key words: flow chart of treatment, chlorination, ultra-violet disinfecting, microbiological characteristics, viruses, high microbiological pollution.

UDC 628.16.32.004.658.5

Information Technologies at the State Unitary Enterprise Vodokanal Sankt-Peterburga. Kasatkin F. Yu., Kostylev S. V. – Р. 66.

A Corporative digital network with integrated services representing a mixed structure of own main fiber-optical channels, radio-relay lines of data transfer and corporative telephony as well as rented data transfer lines of leading operators of St. Petersburg has been created at the SUE Vodokanal Sankt-Peterburga to realize  operative control over the enterprise and timely solution of production tasks.

Key words: information technology, telephony, radio communication, data transfer.