РЕФЕРАТЫ № 11, 2013

РОССИЙСКАЯ АССОЦИАЦИЯ ВОДОСНАБЖЕНИЯ И ВОДООТВЕДЕНИЯ

Итоги VI Межрегиональной конференции водоканалов России «Водный комплекс регионов России: консолидация или автономия» 4

RUSSIAN WATER AND WASTEWATER ASSOCIATION

Results of «Water Industry of the Russian Regions: Consolidation or Autonomy» VI Interregional Conference of the Russian Vodokanals 4

КОНТРОЛЬ КАЧЕСТВА ВОДОИСТОЧНИКОВ

Тымчук С. Н., Ларин В. Е., Соколов Д. М. Наиболее значимые санитарно-микробиологические показатели оценки качества питьевой воды 8

С. Н. ТЫМЧУК1, В. Е. ЛАРИН2, Д. М. СОКОЛОВ3

1 Тымчук Сергей Николаевич, кандидат медицинских наук, ведущий бактериолог отдела биологических методов анализа, ЗАО «РОСА»

119297, Россия, Москва, Родниковая ул., 7, стр. 35, тел.: (495) 439-17-74, e-mail: tsnsergtsn@yandex.ru

2 Ларин Владимир Евгеньевич, кандидат биологических наук, начальник отдела биологических методов анализа, ЗАО «РОСА»

119297, Россия, Москва, Родниковая ул., 7, стр. 35, тел.: (495) 439-17-74, e-mail: larin@rossalab.ru

3 Соколов Дмитрий Михайлович, кандидат биологических наук, директор ООО «МикроБио»

123060, Россия, Москва, 1-й Волоколамский проезд, 10, тел.: (495) 221-20-26, e-mail: info@mibio.ru

Санитарный контроль качества воды – сложный, многоуровневый процесс. Он сопровождает водопользование на всех этапах – от выбора и мониторинга источника водоснабжения, подготовки воды до обеззараживания и отведения использованной воды в приемник сточных вод. Санитарно-микробиологическая оценка качества воды подразумевает определение совокупности санитарных показателей – критериев, отражающих соответствие или несоответствие санитарного состояния исследуемого водного объекта требованиям нормативных документов. Определяемые показатели и кратность их определения зависят от назначения исследуемого объекта и жестко регламентируются соответствующей нормативной базой. Санитарные показатели разделяются на индексные – отражающие степень фекального загрязнения, и индикаторные – отражающие качество водоподготовки. К индексным показателям относятся Escherichia сoli, термотолерантные колиформные бактерии и колифаги. Индикаторными показателями являются общее микробное число, общие колиформные бактерии, споры сульфитредуцирующих клостридий и другие. Важным аспектом контроля водных объектов является определение некоторых патогенных и условно патогенных бактерий, таких как сальмонеллы, патогенные стафилококки, синегнойная палочка и другие. В странах Евросоюза произошел переход от определения групп санитарно-показательных микроорганизмов (термотолерантных колиформных бактерий, фекальных стрептококков, спор сульфитредуцирующих клостридий) к определению непосредственно самих санитарно-показательных микроорганизмов (Escherichia coli, Enterococcus faecalis, Clostridium perfringens), что делает оценку качества воды более стандартизованной и достоверной. В мировой практике санитарных исследований постоянно появляются и внедряются новые методы и подходы к оценке санитарного состояния водных объектов.

Ключевые слова: качество воды, питьевая вода, санитарный показатель, индексный показатель, индикаторный показатель, Escherichia coli, термотолерантные колиформные бактерии, общие колиформные бактерии, колифаги, общее микробное число, сальмонеллы, P. aeruginosa, S. aureus, санитарно-микробиологическая оценка, санитарно-показательный микроорганизм, хромогенная питательная среда.

СПИСОК ЛИТЕРАТУРЫ

- СанПиН 2.1.2.1188-03. Плавательные бассейны. Гигиенические требования к устройству, эксплуатации и качеству воды. Контроль качества.

- СанПиН 2.1.4.1074-01. Питьевая вода. Гигиенические требования к качеству централизованных систем питьевого водоснабжения. Контроль качества: санитарно-эпидемиологические правила и нормативы.

- СанПиН 2.1.4.1116-02. Питьевая вода. Гигиенические требования к качеству воды, расфасованной в емкости. Контроль качества.

- СанПиН 2.1.4.1175-02. Гигиенические требования к качеству воды нецентрализованного водоснабжения.

- СанПиН 2.1.5.980-00. Водоотведение населенных мест, санитарная охрана водных объектов. Гигиенические требования к охране поверхностных вод.

- МУ 2.1.4.1057-01. Организация внутреннего контроля качества санитарно-микробиологических исследований воды.

- МУК 4.2.2884-11. Методы микробиологического контроля объектов окружающей среды и пищевых продуктов с использованием петрифильмов.

- Соколов Д. М., Соколов М. С. Микробиологический контроль с использованием петрифильмов // Молочная промышленность. 2012. № 2. С. 36–37.

- МУК 4.2.1018-01. Санитарно-микробиологический анализ питьевой воды.

- Guidelines for drinking-water quality [electronic resource]: Recommendations. World Health Organization. 2006. Vol. 1.

- ГОСТ Р 52426-2005 (ISO 9308-1:2000). Вода питьевая. Обнаружение и количественный учет Escherichia coli и колиформных бактерий. Часть 1. Метод мембранной фильтрации.

- МР № 24 ФЦ 513. Определение колиформных бактерий и E. coli c использованием хромогенных и флюорогенных индикаторных сред производства Мerck (Германия).

WATER SOURCES QUALITY CONTROL

Tymchuk S. N., Larin V. E., Sokolov D. M. Most significant sanitary microbiological parameters of drinking water quality assessment 8

S. N. TYMCHUK1, V. E. LARIN2, D. M. SOKOLOV3

1 Tymchuk Sergei Nikolaevich, PhD (Medicine), Leading bacteriologist, department of biological methods of analysis, ROSSA CJSC

Build. 35, 7 Rodnikovaia str., 119297 Moscow, Russia, tel.: +7 (495) 439-17-74, e-mail: tsnsergtsn@yandex.ru

2 Larin Vladimir Evgen'evich, PhD (Biology), Head of department of biological methods of analysis, ROSSA CJSC

Build. 35, 7 Rodnikovaia str., 119297 Moscow, Russia, tel.: +7 (495) 439-17-74, e-mail: larin@rossalab.ru

3 Sokolov Dmitrii Mikhailovich, PhD (Biology), Director of «MicroBio» LLC

10 1st Volokolamskii passway, 123060 Moscow, Russia, tel.: +7 (495) 221-20-26, e-mail: info@mibio.ru

Sanitary control of water is a complex and multilevel process. It is associated with every stage of water use, starting from the water supply source selection, monitoring, and water treatment to disinfection and wastewater disposal to collection facilities. Sanitary and microbiological water assessment implies determining sanitary profile, i. e. criteria that reflect compliance or noncompliance of the object under investigation with the regulatory documents requirements. The determined parameters and the frequency of their measurement depend on the assignment of the project under investigation and are strictly regulated by the respective regulatory framework. Sanitary parameters are divided into index parameters that reflect the level of fecal pollution and indicator parameters that reflect the quality of water treatment. Index parameters include: Escherichia сoli, thermotolerant coliforms and coliphages. Indicator parameters are: total microbial count, total coliforms, sulfite-reducing clostridia spores etc. Determination of some pathogenic and opportunistic pathogenic bacteria like salmonella, pathogenic staphylococcus, blue pus bacillus etc. is an important aspect of water bodies monitoring. EC countries moved from determining groups of sanitary indicator microorganisms (thermotolerant coliforms, fecal staphylococcus, sulfite-reducing clostridia spores) to determining directly sanitary indicator microorganisms (Escherichia coli, Enterococcus faecalis, Clostridium perfringens), which makes water quality assessment more standardized and reliable. Advanced methods and approaches to the assessment of the sanitary state of the water bodies are continuously emerging and introduced in the international practice of sanitary surveys.

Key words: water quality, drinking water, sanitary parameter, index parameter, indicator parameter, Escherichia coli, thermotolerant coliforms, total coliforms, coliphages, total microbial count, salmonella, P. aeruginosa, S. aureus, sanitary microbiological assessment, sanitary indicator microorganism, chromogenic substratum.

REFERENCES

- SanPiN 2.1.2.1188-03. Plavatel'nye basseiny. Gigienicheskie trebovaniia k ustroistvu, ekspluatatsii i kachestvu vody. Kontrol' kachestva [SanPiN 2.1.2.1188-03. Swimming pools. Hygienic requirements to the design, operation and water quality. Quality control]. (In Russian).

- SanPiN 2.1.4.1074-01. Pit'evaia voda. Gigienicheskie trebovaniia k kachestvu tsentralizovannykh sistem pit'evogo vodosnabzheniia. Kontrol' kachestva: sanitarno-epidemiologicheskie pravila i normativy [SanPiN 2.1.4.1074-01. Drinking water. Hygienic requirements to the quality of public drinking water supply. Quality control: sanitation-and-epidemiological rules and regulations]. (In Russian).

- SanPiN 2.1.4.1116-02. Pit'evaia voda. Gigienicheskie trebovaniia k kachestvu vody, rasfasovannoi v emkosti. Kontrol' kachestva [SanPiN 2.1.4.1116-02. Drinking water. Hygienic requirements to bottled water quality. Quality control]. (In Russian).

- SanPiN 2.1.4.1175-02. Gigienicheskie trebovaniia k kachestvu vody netsentralizovannogo vodosnabzheniia [SanPiN 2.1.4.1175-02. Hygienic requirements to the quality of water in noncentralized water supply]. (In Russian).

- SanPiN 2.1.5.980-00. Vodootvedenie naselennykh mest, sanitarnaia okhrana vodnykh ob"ektov. Gigienicheskie trebovaniia k okhrane poverkhnostnykh vod [SanPiN 2.1.5.980-00. Community wastewater disposal, sanitary protection of water bodies. Hygienic requirements to surface water protection]. (In Russian).

- MU 2.1.4.1057-01. Organizatsiia vnutrennego kontrolia kachestva sanitarno-mikrobiologicheskikh issledovanii vody [Methodical conditions 2.1.4.1057-01. Arrangement of internal control of sanitary microbiological water survey quality]. (In Russian).

- MUK 4.2.2884-11. Metody mikrobiologicheskogo kontrolia ob"ektov okruzhaiushchei sredy i pishchevykh produktov s ispol'zovaniem petrifil'mov [Practical policies 4.2.2884-11. Methods of microbiological control of environmental projects and food products with the use of Petrifilms]. (In Russian).

- Sokolov D. M., Sokolov M. S. Mikrobiologicheskii kontrol' s ispol'zovaniem petrifil'mov [Microbiological control with the use of Petrifilms]. Molochnaia Promyshlennost', 2012, no. 2, pp. 36–37. (In Russian).

- MUK 4.2.1018-01. Sanitarno-mikrobiologicheskii analiz pit'evoi vody [Practical policies 4.2.1018-01. Sanitary-microbiological analysis of drinking water]. (In Russian).

- Guidelines for drinking-water quality [electronic resource]: Recommendations. World Health Organization, 2006, vol. 1.

- GOST R 52426-2005 (ISO 9308-1:2000). Voda pit'evaia. Obnaruzhenie i kolichestvennyi uchet Escherichia coli i koliformnykh bakterii. Chast' 1. Metod membrannoi fil'tratsii [GOST R 52426-2005 (ISO 9308-1:2000). Drinking water. Detection and quantitative accounting of Escherichia coli and coliform bacteria. Part 1. Membrane filtration method]. (In Russian).

- MR № 24 FTs 513. Opredelenie koliformnykh bakterii i E. coli c ispol'zovaniem khromogennykh i fliuorogennykh indikatornykh sred proizvodstva Merck (Germaniia) [Guidelines No. 24 FTS 513. Detection of coliform bacteria and E. coli with the use of Мerck (Germany) chromogenic and fluorogenic indicator substrates]. (In Russian).

Василенко С. Л., Кобылянский В. Я. Система раннего оповещения об ухудшении качества воды на основе биотестирования ее токсичности 17

С. Л. ВАСИЛЕНКО1, В. Я. КОБЫЛЯНСКИЙ2

1 Василенко Сергей Леонидович, кандидат технических наук, главный гидролог, Коммунальное предприятие «Харьковводоканал»

61013, Украина, г. Харьков, ул. Шевченко, 2, тел.: +38 (057) 712-15-21, e-mail: texvater@rambler.ru

2 Кобылянский Владимир Ярославович, кандидат технических наук, директор ООО «Научно-исследовательский центр водоснабжения и качества воды»

61183, Украина, г. Харьков, ул. Дружбы Народов, 255, тел.: +38 (057) 756-26-70, e-mail: vkob@list.ru

Для совершенствования мониторинга качества исходной воды на питьевых водозаборах и определения ее токсичности используются методы биотестирования с целью своевременного принятия защитно-управленческих решений. Система мониторинга качества исходной воды для водоснабжения городов базируется на установлении предельно допустимых концентраций загрязняющих веществ. На Украине используются нормы охраны поверхностных вод от загрязнения (СанПиН 4630-88), в которых только нормируемые химические вещества первого и второго класса опасности исчисляются сотнями. Разработанные ранее методики контроля загрязнений содержат противоречия из-за несоответствия растущей номенклатуры загрязняющих веществ и устаревшего технического и методического оснащения контролирующих организаций. Предложено использовать в качестве тактического контроля загрязнений метод биотестирования, который основан на регистрации активности фотосинтезирующей способности и дыхания зеленых микроводорослей Chlorella vulgaris Beijer или Scenedesmus quadricauda. Разработана функциональная схема автоматического поста непрерывного наблюдения и алгоритм оценки качества природных вод. Показана корреляция показателя токсичности воды с традиционными интегральными и физико-химическими показателями качества воды. Описана функциональная схема автоматического поста непрерывного наблюдения и его технические возможности, позволяющие подключиться к региональной системе экологического мониторинга. Опытный образец автоматического поста непрерывного наблюдения прошел испытания на Печенежском водохранилище. Было установлено, что параметр токсичности повышался в периоды роста БПК и снижения содержания кислорода, а также увеличения концентрации аммиака. Окисляемость в это время почти не изменялась. На основании исследований состава природных вод и экспериментальных измерений параметра их токсичности рекомендуется установить для питьевого водоснабжения допустимое предельное значение параметра токсичности 40%.

Ключевые слова: качество воды, биотестирование, экологическая безопасность, мониторинг.

СПИСОК ЛИТЕРАТУРЫ

- Филенко О. Ф. Биологические методы в контроле качества окружающей среды // Экологические системы и приборы. 2007. № 6. С. 18–20.

- Чеснокова С. М., Чугай Н. В. Биологические методы оценки качества объектов окружающей среды. Методы биотестирования. – Владимир, ВГУ, 2008. 91 с.

- Крайнюкова А. Н. Система интегральной токсикологической оценки природных и сточных вод // Східно-Європейський журнал передових технологій. 2009. Т. 1. № 4 (37). С. 30–33.

- Єфремова О. О. Біотестування питної води у моніторингу стану екологічної безпеки: Автореф. дис. … канд. техн. наук. – Киев, 2009. 22 с.

- Петросов В. А. Устойчивость водоснабжения. – Харьков: Фактор, 2008. 360 с.

- Остроумов С. А. Элементы теории биоконтроля качества воды: фактор экологической безопасности источников водоснабжения // Химическая и биологическая безопасность. 2008. № 5–6. С. 36–39.

- Гончарук В. В., Архипчук В. В., Терлецька Г. В., Корчак Г. І.  Комплексна оцінка якості фасованих вод // Вісник НАН України. 2005. № 3. С. 47–58.

- Богомазов О. А., Кобылянский В. Я., Панасенко А. А., Шавлов А. Т. Автоматизация контроля качества природных и сточных вод. – Екатеринбург: Уральский рабочий, 1997. 239 с.

- Стародубов А. Г., Колотило В. Д., Кобылянский В. Я.  Научно-методическое и техническое оснащение ведомственных лабораторий предприятий водоснабжения / Актуальні питання якості води в Україні: Матер. наук.-практ. семінару, 2–3 червня 2005 р. – Киев: ДП «УНДНЦ» Держспоживстандарт, 2005. С. 156–158.

- Джерела централізованого питного водопостачання. Гігієнічні та екологічні вимоги щодо якості води і правила вибирання. ДСТУ 4808:2007. – Киев: Держспоживстандарт України, 2007. 36 с.

- Василенко С. Л. Синтез и логический анализ аксиоматики в сфере водоснабжения: системно-экологический аспект / Екологічна безпека: проблеми і шляхи вирішення: Зб. наук. ст. у 2-х т. Т. 1 / УкрНДІЕП. – Харьков: Райдер, 2011. С. 246–251.

- Василенко С. Л. Экологическая безопасность систем питьевого водоснабжения: систематизация методологических принципов // Водное хозяйство России. 2009. № 2. С. 68–77.

- Васюков А. Е., Бланк А. Б. Химические аспекты экологической безопасности поверхностных водных объектов. – Харьков: Институт монокристаллов, 2007. 256 с.

- Kajiwara Y., Kashiwagi N., Kadokami K. Nationwide study of dioxins in the freshwater fish Carassius auratus (gibelio) langsdorfii (crucian carp) in Japan: Concentrations and estimation of source contribution ratios // Chemosphere. 2007. Vol. 69. № 8. PP. 1177–1187.

- Кобылянский В. Я., Василенко С. Л.  Автономные посты контроля качества природных и сточных вод / Проблемы обеспечения населения качественной водой: Международная научная конференция. 8–10 октября 1997 г., Киев. С. 131–134.

Vasilenko S. L., Kobylianskii V. Ia. The system of early warning of water quality deterioration on the basis of toxicological testing 17

S. L. VASILENKO1, V. Ia. KOBYLIaNSKII2

1 Vasilenko Sergei Leonidovich, PhD (Engineering), Chief Hydrologist, «Khar'kovvodokanal» Public Utility

2 Shevchenko str., Kharkov 61013, Ukraine, tel.: +38 (057) 712-15-21, e-mail: texvater@rambler.ru

2 Kobylianskii Vladimir Iaroslavovich, PhD (Engineering), Director, Research Centre of water supply and water quality LLC

255 Druzhby Narodov str., Kharkov 61183, Ukraine, tel.: +38 (057) 756-26-70, e-mail: vkob@list.ru

To improve raw water quality monitoring at water intakes and determine water toxicity the methods of biotesting are used to provide for making timely protecting management decisions. The system of raw water quality monitoring in municipal water supply is based on setting maximum permissible concentrations of pollutants. In Ukraine the normative standards of surface water protection from pollution (SanPiN 4630-88), where critical chemicals of the first and second classes of hazard alone amount to hundreds. The techniques developed earlier are inconsistent because of the discrepancies between the enlarging list of pollutants and out-of-date technical and methodological capacities of the supervisory agencies. The use of the biotesting method based on recording the photosynthetic capacity and respiration of Chlorella vulgaris Beijer or Scenedesmus quadricauda green algae for strategic pollution monitoring has been suggested. The flowchart of an automatic continuous monitoring station and an algorythm of natural water quality assessment were designed. The correlation between water toxicity index and traditional integral, physical and chemical water quality indices is shown. The flowchart of an automatic continuous monitoring station and its technical capacbilities that provide for tapping into the regional environmental monitoring system are described. A pilot model of the automatic continuous monitoring station was successfully tested at the Petchenezhskoye water reservoir. It was found that the toxicity index increased when BOD increased and the oxygen concentration decreased. The oxidation characetristic at that time did not change practically. On the basis of investigating natural water composition and experimental measuring water toxicity setting 40% threshold toxicity value for drinking water supply is recommended.

Key words: water quality, biotesting, environmental safety, monitoring.

REFERENCES

- Filenko O. F. Biologicheskie metody v kontrole kachestva okruzhaiushchei sredy [Biological methods in environmental quality control]. Ekologicheskie Sistemy i Pribory, 2007, no. 6, pp. 18–20. (In Russian).

- Chesnokova S. M., Chugai N. V. Biologicheskie metody otsenki kachestva ob"ektov okruzhaiushchei sredy. Metody biotestirovaniia [Biological methods of evaluating quality of environmental objects. Methods of biotesting]. Vladimir, VSU, 2008, 91 p. (In Russian).

- Krainiukova A. N. Sistema integral'noi toksikologicheskoi otsenki prirodnykh i stochnykh vod [The system of integral toxicologic estimation of natural and waste water]. Shidno-Jevropejs'kyj Zhurnal Peredovyh Tehnologij, 2009, vol. 1, no. 4 (37), pp. 30–33. (In Russian).

- Jefremova O. O. Biotestuvannja pytnoi' vody u monitoryngu stanu ekologichnoi' bezpeky [Biotesting of potable water and monitoring of ecological safety conditions]. Synopsis of a thesis for PhD degree in Engineering. Kiev, 2009, 22 p. (In Ukrainian).

- Petrosov V. A. Ustoichivost' vodosnabzheniia [Water supply sustainability]. Kharkov, Faktor Publ., 2008, 360 p. (In Russian).

- Ostroumov S. A. Elementy teorii biokontrolia kachestva vody: faktor ekologicheskoi bezopasnosti istochnikov vodosnabzheniia [Elements of the theory of water quality biological monitoring: the factor of water sources environmental safety]. Khimicheskaia i Biologicheskaia Bezopasnost', 2008, no. 5–6, pp. 36–39. (In Russian).

- Goncharuk V. V., Arhypchuk V. V., Terlec'ka G. V., Korchak G. I. Kompleksna ocinka jakosti fasovanyh vod [Complex assessment of quality of packaged water]. Visnyk NAN Ukrai'ny, 2005, no. 3, pp. 47–58. (In Ukrainian).

- Bogomazov O. A., Kobylianskii V. Ia., Panasenko A. A., Shavlov A. T. Avtomatizatsiia kontrolia kachestva prirodnykh i stochnykh vod [Automatic control of natural and waste water quality]. Ekaterinburg, Ural'skii Rabochii Publ., 1997, 239 p. (In Russian).

- Starodubov A. G., Kolotilo V. D., Kobylianskii V. Ia. Nauchno-metodicheskoe i tekhnicheskoe osnashchenie vedomstvennykh laboratorii predpriiatii vodosnabzheniia [Scientific-methodological and technical equipment of departmental laboratories of water supply utilities]. Actual issues of water quality in Ukraine: materials of scientific-practical seminar, June 2–3, 2005, Kiev, DP «UNDNTs» Derzhspozhyvstandart Publ., pp. 156–158. (In Russian).

- Dzherela centralizovanogo pytnogo vodopostachannja. Gigijenichni ta ekologichni vymogy shhodo jakosti vody i pravyla vybyrannja [Sources of centralized potable water supply. Hygienic and ecological requirements for water quality and rules of selection]. DSTU 4808:2007. Kiev, Derzhspozhyvstandart Ukrai'ny Publ., 2007, 36 p. (In Ukrainian).

- Vasilenko S. L. Sintez i logicheskii analiz aksiomatiki v sfere vodosnabzheniia: sistemno-ekologicheskii aspekt [Synthesis and logic analysis of axiomatic in water supply: systematic and ecologic aspect]. Ekologichna bezpeka: problemy i shljahy vyrishennja: Zb. nauk. st. u 2-h t. UkrNDIEP. Kharkov, Rajder Publ., 2011, vol. 1, pp. 246–251. (In Russian).

- Vasilenko S. L. Ekologicheskaia bezopasnost' sistem pit'evogo vodosnabzheniia: sistematizatsiia metodologicheskikh printsipov [Environmental safety of drinking water supply systems: guidance systematizing]. Vodnoe Khoziaistvo Rossii, 2009, no. 2, pp. 68–77. (In Russian).

- Vasiukov A. E., Blank A. B. Khimicheskie aspekty ekologicheskoi bezopasnosti poverkhnostnykh vodnykh ob"ektov [Chemical aspects of environmental safety of surface water projects]. Kharkov, Institute of monocrystals, 2007, 256 p. (In Russian).

- Kajiwara Y., Kashiwagi N., Kadokami K. Nationwide study of dioxins in the freshwater fish Carassius auratus (gibelio) langsdorfii (crucian carp) in Japan: Concentrations and estimation of source contribution ratios. Chemosphere, 2007, vol. 69, no. 8, pp. 1177–1187.

- Kobylianskii V. Ia., Vasilenko S. L. Avtonomnye posty kontrolia kachestva prirodnykh i stochnykh vod [Self-contained stations of natural and waste water quality monitoring]. Problemy obespecheniia naseleniia kachestvennoi vodoi [Problems of supplying high-quality water to the population]. International scientific conference, October 8–10, 1997, Kiev, pp. 131–134. (In Russian).

ПИТЬЕВОЕ ВОДОСНАБЖЕНИЕ

Башкетова Н. С., Нефедова Е. Д. Актуальные вопросы обеспечения населения Санкт-Петербурга доброкачественной питьевой водой 26

Н. С. БАШКЕТОВА1, Е. Д. НЕФЕДОВА2

1 Башкетова Наталия Семеновна, заместитель руководителя Управления Роспотребнадзора по городу Санкт-Петербургу

191025, Россия, Санкт-Петербург, Стремянная ул., 19, тел.: (812) 764-58-77, e-mail: bashketova-natal@mail.ru

2 Нефедова Елена Дмитриевна, кандидат технических наук, директор филиала «Инженерно-инновационный центр», ГУП «Водоканал Санкт-Петербурга»

191015, Россия, Санкт-Петербург, Кавалергардская ул., 42, тел.: (812) 326-52-06, e-mail: Nefedova_ED@vodokanal.spb.ru

Приведены результаты исследований, выполненных специалистами ГУП «Водоканал Санкт-Петербурга» и Центра гигиены и эпидемиологии в рамках Рабочей программы производственного контроля качества воды в целях санитарно-гигиенического мониторинга. Контроль качества питьевой воды, подаваемой населению Санкт-Петербурга и пригородов, на всех этапах водоподготовки осуществляется в 174 контрольных точках по 86 показателям с периодичностью от одного раза в сутки до одного раза в месяц. Аналитический контроль качества питьевой воды выполнялся с использованием современных методов и оборудования. Для оценки эффективности мероприятий по обеспечению населения эпидемиологически безопасной питьевой водой введен показатель «заболеваемость гепатитом А». Результаты социально-гигиенического мониторинга свидетельствуют о стабильном улучшении показателей качества питьевой воды в многолетней динамике и тенденции к снижению заболеваемости населения. Разработаны мероприятия для обеспечения безопасного питьевого водоснабжения: внедрение системы ультрафиолетового обеззараживания питьевой воды; применение двухступенчатой схемы обеззараживания питьевой воды при сочетании химического и физического методов; переход на технологии обеззараживания воды гипохлоритом натрия и сульфатом аммония; внедрение системы биомониторинга качества воды водоисточника и системы дозирования сорбента; строительство нового блока очистки воды. Комплексный подход к контролю качества питьевой воды позволил сформулировать «Концепцию обеспечения населения Санкт-Петербурга физиологически полноценной питьевой водой» и дать объективную оценку качества услуги водоснабжения, в том числе с учетом оценки риска потребления и полноценности питьевой воды.

Ключевые слова: питьевая вода, система контроля, санитарно-эпидемиологический надзор, физиологическая полноценность воды.

СПИСОК ЛИТЕРАТУРЫ

- Федеральный закон от 30 марта 1999 г. № 52-ФЗ «О санитарно-эпидемиологическом благополучии населения».

- Федеральный закон от 7 декабря 2011 г. № 416-ФЗ «О водоснабжении и водоотведении».

- СанПиН 2.1.4.1074-01. Питьевая вода. Гигиенические требования к качеству воды централизованных систем питьевого водоснабжения. Контроль качества. Гигиенические требования к обеспечению систем горячего водоснабжения (в редакции постановления Главного государственного санитарного врача РФ от 7 апреля 2009 г. № 20).

- Программа комплексного развития систем коммунальной инфраструктуры Санкт-Петербурга по разделам электро-, тепло-, водоснабжение, водоотведение и очистка сточных вод до 2015 года. Утверждена постановлением Правительства Санкт-Петербурга от 21 октября 2008 г. № 1270 «О Программе комплексного развития систем коммунальной инфраструктуры Санкт-Петербурга до 2015 года».

- Программа «Чистая вода Санкт-Петербурга» на 2011–2025 годы. Утверждена постановлением Правительства Санкт-Петербурга от 24 мая 2011 г. № 625 «Об утверждении Программы «Региональная программа «Чистая вода Санкт-Петербурга» на 2011–2025 годы».

- СанПиН 2.1.4.1116-02. Питьевая вода. Гигиенические требования к качеству воды, расфасованной в емкости. Контроль качества.

DRINKING WATER SUPPLY

Bashketova N. S., Nefedova E. D. Current problems of delivering high quality drinking water to the population of St. Petersburg 26

N. S. BASHKETOVA1, E. D. NEFEDOVA2

1 Bashketova Nataliia Semenovna, Deputy Head of St. Petersburg Rospotrebnadzor (Federal Service on Customer's Rights, Protection and Human Well-Being Surveillance)

19 Stremiannaia str., 191025 St. Petersburg, Russia, tel.: +7 (812) 764-58-77, e-mail: bashketova-natal@mail.ru

2 Nefedova Elena Dmitrievna, PhD (Engineering), Director of Engineering Innovation Centre branch, SUE «Vodokanal of St. Petersburg»

42 Kavalergardskaia str., 191015 St. Petersburg, Russia, tel.: +7 (812) 326-52-06, e-mail: Nefedova_ED@vodokanal.spb.ru

The results of studies carried out by the experts of SUE «Vodokanal of St. Petersburg» and Centre for hygiene and epidemiology within the frames of Work program of water quality process inspection for the purpose of public health monitoring are presented. Monitoring the quality of drinking water delivered to the population of St. Petersburg and its surroundings is carried out for 86 parameters in 174 control points at every stage of water treatment with regularity from daily to monthly. Analytical drinking water quality control was carried out with the use of advanced methods and equipment. To evaluate the efficiency of the measures for delivering epidemiologically safe drinking water to the population an «infectious hepatitis» morbidity rate was introduced. The results of social-hygienic monitoring show sustainable long-term improvement of water quality parameters and the tendency for population morbidity decrease. The following measures for ensuring safe drinking water supply have been developed: introduction of UV drinking water disinfection; use of two-stage drinking water disinfection with the combination of physical and chemical methods; conversion to disinfection with sodium hypochlorite and ammonia sulphate; introduction of water source quality biomonitoring and sorbent dosing systems; construction of an advanced water treatment line. The comprehensive approach to drinking water quality monitoring allowed to develop the Concept of delivering physiologically adequate drinking water to the St. Petersburg population and give an objective estimate of the water supply services with account of the assessment of consumer's risk and full-value of water.

Key words: drinking water, monitoring system, disease control, physiological full-value of water.

REFERENCES

- Federal'nyi zakon ot 30 marta 1999 g. № 52-FZ «O sanitarno-epidemiologicheskom blagopoluchii naseleniia» [52-FZ Federal Law of March 30, 1999 «On sanitary-epidemiological population welfare»]. (In Russian).

- Federal'nyi zakon ot 7 dekabria 2011 g. № 416-FZ «O vodosnabzhenii i vodootvedenii» [416-FZ Federal Law of December 7, 2011 «On water supply and wastewater disposal»]. (In Russian).

- SanPiN 2.1.4.1074-01. Pit'evaia voda. Gigienicheskie trebovaniia k kachestvu vody tsentralizovannykh sistem pit'evogo vodosnabzheniia. Kontrol' kachestva. Gigienicheskie trebovaniia k obespecheniiu sistem goriachego vodosnabzheniia (v redaktsii postanovleniia Glavnogo gosudarstvennogo sanitarnogo vracha RF ot 7 aprelia 2009 g. № 20) [SanPiN 2.1.4.1074-01. Drinking water. Hygienic requirements to the quality of public drinking water supply. Quality control. Hygienic requirements to hot water supply systems (as defined in the Resolution No. 20 of the RF Chief medical sanitation officer of April 7, 2009)]. (In Russian).

- Programma kompleksnogo razvitiia sistem kommunal'noi infrastruktury Sankt-Peterburga po razdelam elektro-, teplo-, vodosnabzhenie, vodootvedenie i ochistka stochnykh vod do 2015 goda [Program of integrated development of the St. Petersburg municipal infrastructure sectors of power-, heat-, and water supply and wastewater disposal till 2015]. Approved by Resolution No. 1270 of October 21, 2008 «On the Program of integrated development of the St. Petersburg municipal infrastructure till 2015». (In Russian).

- Programma «Chistaia voda Sankt-Peterburga» na 2011–2025 gody [«Clean water of St. Petersburg» Program for 2011–2025]. Approved by Resolution No. 625 of May 24, 2011 «On the approval of the «Clean water of St. Petersburg» Regional Program for 2011–2025». (In Russian).

- SanPiN 2.1.4.1116-02. Pit'evaia voda. Gigienicheskie trebovaniia k kachestvu vody, rasfasovannoi v emkosti. Kontrol' kachestva [SanPiN 2.1.4.1116-02. Drinking water. Hygienic requirements to bottled water quality. Quality control]. (In Russian).

Беляк А. А., Герасимов М. М., Гусева О. А., Смирнов А. Д.

Извлечение аммонийного азота из воды поверхностного водоисточника с использованием порошкообразных цеолитов 32

А. А. БЕЛЯК1, М. М. ГЕРАСИМОВ2, О. А. ГУСЕВА3, А. Д. СМИРНОВ4

1 Беляк Андрей Анатольевич, старший научный сотрудник, ОАО «НИИ ВОДГЕО»

119435, Россия, Москва, Б. Саввинский пер., 9, стр. 1, тел.: (499) 272-47-53, e-mail: reddbear2002@yahoo.com

2 Герасимов Михаил Михайлович, кандидат технических наук, заместитель заведующего лабораторией, ОАО «НИИ ВОДГЕО»

119435, Россия, Москва, Б. Саввинский пер., 9, стр. 1, тел.: (499) 272-47-56, e-mail: vodgeo@mail.ru

3 Гусева Оксана Анатольевна, инженер, ОАО «НИИ ВОДГЕО»

119435, Россия, Москва, Б. Саввинский пер., 9, стр. 1, тел.: (499) 272-47-53, e-mail: reddbear2002@yahoo.com

4 Смирнов Александр Дмитриевич, доктор технических наук, заведующий лабораторией глубокой очистки воды, ОАО «НИИ ВОДГЕО»

119435, Россия, Москва, Б. Саввинский пер., 9, стр. 1, тел.: (499) 272-47-56, e-mail: vodgeo@mail.ru

В паводковый период на очистных сооружениях водопровода г. Кирова фиксируется повышение концентрации аммонийного азота в речной воде до 2–2,5 ПДК. Дана оценка возможности использования клиноптилолитсодержащих цеолитов в порошкообразной форме для удаления ионов аммония из природных вод в существующей технологической схеме очистки воды. Испытания проводились на воде р. Вятки. После ввода порошка цеолитов в очищаемую воду дозой 0,1–1,3 г/л и перемешивания в течение 30 минут (200 об/мин) образовывалась тонкая взвесь. При отстаивании часть порошка цеолита выпадала в осадок в течение 2–3 часов, но высокая мутность сохранялась на протяжении суток. После ввода коагулянта (сульфата алюминия) дозой 10 мг/л по Al2O3 и флокулянта (Праестол 650TR) осадок выпадал полностью в виде крупных хлопьев в течение 5–15 минут. Определена ионообменная емкость образцов порошкообразных цеолитов – от 1,35 до 2,25 мг/г (по аммонийному азоту). Наибольшую емкость показал природный цеолит «Сокирнит» (фракция 0,06–0,1 мм). Доказано, что обменная емкость по ионам аммония зависит от содержания клиноптилолита в материале (его содержание может меняться при измельчении и фракционировании природного цеолита). На Кировских очистных сооружениях водопровода была смонтирована и опробована первая опытная линия комплекса производительностью по очищенной воде 58 тыс. м3/сут, позволившая снизить содержание аммонийного азота до значений ниже ПДК (2 мг/л).

Ключевые слова: очистные сооружения водопровода, технологическая схема, клиноптилолитсодержащий цеолит, ионы аммония, обменная емкость.

СПИСОК ЛИТЕРАТУРЫ

- Орлов В. А. Озонирование воды. – М.: Стройиздат, 1984. C. 16–18.

- Bischof H., Hofl C., Schonweitz C., Sigl G., Wimmer B., Wabner D. UV-activated hydrogen peroxide for ground and drinking water treatment. Developing of a technical process: Regional Conference on Ozone, Ultraviolet Light, Advanced Oxidation Processes in Water Treatment. Amsterdam, Netherlands, September 24–26, 1996. P. 117–131.

- Поляков В. Е., Тарасевич Ю. И., Медведев М. И., Розенфельд А. Л., Смирнов О. П. Ионообменная сорбция аммония и калия клиноптилолитом и разработка технологии их извлечения из сточных вод // Химия и технология воды. 1979. Т. 1. № 2. С. 19–24.

- Ершов А. В., Яременко Л. В., Лебеда Л. В., Алекберова В. В. Очистка городских сточных вод от аммонийного азота клиноптилолитовой породой Закарпатья // Химия и технология воды. 1984. Т. 6. № 1. С. 71–74.

- Ершова А. Т., Кирпиченко Е. Н., Михайлов В. И. Использование клиноптилолита Холинского месторождения для удаления ионов аммония из техногенных вод // Химия и технология воды. 1991. Т. 13. № 1. С. 65–67.

- Перечень материалов, реагентов и малогабаритных очистных устройств, разрешенных Государственным комитетом санитарно-эпидемиологического надзора РФ для применения в практике хозяйственно-питьевого водоснабжения. Утвержден 23 октября 1992 г. № 01-19/32-11.

- Лобанов Р. Цеолиты против азота // ВодаMagazine. 2012. № 8 (60). C. 4–5.

Beliak A. A., Gerasimov M. M., Guseva O. A., Smirnov A. D. Removal of ammonia nitrogen from surface water source with the use of powdered zeolite 32

A. A. BELIAK1, M. M. GERASIMOV2, O. A. GUSEVA3, A. D. SMIRNOV4

1 Beliak Andrei Anatol'evich, Senior researcher, «NII VODGEO» OJSC

Build. 1, 9 B. Savvinskii lane, 119435 Moscow, Russia, tel.: +7 (499) 272-47-53, e-mail: reddbear2002@yahoo.com

2 Gerasimov Mikhail Mikhailovich, PhD (Engineering), deputy laboratory chief, «NII VODGEO» OJSC

Build. 1, 9 B. Savvinskii lane, 119435 Moscow, Russia, tel.: +7 (499) 272-47-56, e-mail: vodgeo@mail.ru

3 Guseva Oksana Anatol'evna, engineer, «NII VODGEO» OJSC

Build. 1, 9 B. Savvinskii lane, 119435 Moscow, Russia, tel.: +7 (499) 272-47-53, e-mail: reddbear2002@yahoo.com

4 Smirnov Aleksandr Dmitrievich, Doctor of Engineering, Chief of advanced water treatment laboratory, «NII VODGEO» OJSC

Build. 1, 9 B. Savvinskii lane, 119435 Moscow, Russia, tel.: +7 (499) 272-47-56, e-mail: vodgeo@mail.ru

During flood periods increased concentrations of ammonia nitrogen in river water to 2–2.5 maximum permissible concentration are recorded at the Kirov water treatment plant. The assessment of possible use of powdered clinoptilolit containing zeolites in the existing water treatment process flow scheme to eliminate ammonium ions from natural water is given. The tests were carried out with the Viatka river water. After zeolite powder dosage of 0.1–1.3 mg/l addition into the water and 30 minutes mixing (200 rpm) slick was formed. During sedimentation a part of zeolite powder settled for 2–3 hours; however high turbidity remained for 24 hours. After adding 10 mg/l dosage of coagulant (aluminium sulfate) (as Al2O3) and Praestol 650TR flocculant complete precipitation of big flocks was achieved within 5–15 minutes. The ion-exchange capacity of powdered zeolite specimen – from 1.35 to 2.25 (as ammonia nitrogen) was determined. The highest ion-exchange capacity was recorded for Sokirnit natural zeolite (0.06–0.1 mm particle size). It was proved that ammonium ions exchange capacity depends on the concentration of clinoptilolit in the material (its concentration can change in the process of natural zeolite disintegrating and fractionating). The first pilot line of the plant with a capacity of 58 thousand m3 of water per day that provided for reducing ammonia nitrogen concentration to the level lower than MPC (2 mg/l) was assembled and tested at the Kirov water treatment plant.

Key words: water treatment plant, process flow scheme, clinoptilolit containing zeolite, ammonium ions, exchange capacity.

REFERENCES

- Orlov V. A. Ozonirovanie vody [Ozonation of water]. Moscow, Stroiizdat Publ., 1984, pp. 16–18. (In Russian).

- Bischof H., Hofl C., Schonweitz C., Sigl G., Wimmer B., Wabner D. UV-activated hydrogen peroxide for ground and drinking water treatment. Developing of a technical process: Regional Conference on Ozone, Ultraviolet Light, Advanced Oxidation Processes in Water Treatment. Amsterdam, Netherlands, September 24–26, 1996, pp. 117–131.

- Poliakov V. E., Tarasevich Iu. I., Medvedev M. I., Rozenfel'd A. L., Smirnov O. P. Ionoobmennaia sorbtsiia ammoniia i kaliia klinoptilolitom i razrabotka tekhnologii ikh izvlecheniia iz stochnykh vod [Ion exchange sorption of ammonium and potassium by clinoptilolit and development of the technology of their removal from wastewater]. Khimiia i Tekhnologiia Vody, 1979, vol. 1, no. 2, pp. 19–24. (In Russian).

- Ershov A. V., Iaremenko L. V., Lebeda L. V., Alekberova V. V. Ochistka gorodskikh stochnykh vod ot ammoniinogo azota klinoptilolitovoi porodoi Zakarpat'ia [Removing ammonia nitrogen from wastewater with the use of clinoptilolit rock from Zakarpatie]. Khimiia i Tekhnologiia Vody, 1984, vol. 6, no. 1, pp. 71–74. (In Russian).

- Ershova A. T., Kirpichenko E. N., Mikhailov V. I. Ispol'zovanie klinoptilolita Kholinskogo mestorozhdeniia dlia udaleniia ionov ammoniia iz tekhnogennykh vod [The use of clinoptilolit from Kholinsk deposit for the removal of ammonium ions from technogenic waters]. Khimiia i Tekhnologiia Vody, 1991, vol. 13, no. 1, pp. 65–67. (In Russian).

- Perechen' materialov, reagentov i malogabaritnykh ochistnykh ustroistv, razreshennykh Gosudarstvennym komitetom sanitarno-epidemiologicheskogo nadzora RF dlia primeneniia v praktike khoziaistvenno-pit'evogo vodosnabzheniia [List of materials, chemicals and small treatment units released by the RF State Committee on disease control for use in domestic water supply]. Approved on October 23, 1992, no. 01-19/32-11. (In Russian).

- Lobanov R. Tseolity protiv azota [Zeolites against nitrogen]. VodaMagazine, 2012, no. 8 (60), pp. 4–5. (In Russian).

Эффективное водоснабжение с Grundfos – правильный путь к экономии ресурсов 39

Efficient water supply with Grundfos – the key to resource conservation 39

Консигнационный склад – оптимальное решение для водоканалов 40

Consignment warehouse – optimal solution for vodokanals 40

ОЧИСТКА СТОЧНЫХ ВОД

Швецов В. Н., Морозова К. М. Расчет сооружений биологической очистки сточных вод с удалением биогенных элементов 42

В. Н. ШВЕЦОВ1, К. М. МОРОЗОВА2

1 Швецов Валерий Николаевич, доктор технических наук, профессор, руководитель отдела, ОАО «НИИ ВОДГЕО»

119435, Россия, Москва, Б. Саввинский пер., 9, стр. 1, тел.: (499) 245-96-33, e-mail: vst@aha.ru

2 Морозова Ксения Михайловна, кандидат технических наук, ведущий научный сотрудник, ОАО «НИИ ВОДГЕО»

119435, Россия, Москва, Б. Саввинский пер., 9, стр. 1, тел.: (499) 272-47-54, e-mail: vodgeo@inbox.ru

Представлена методика расчета сооружений биологической очистки сточных вод с нитри-денитрификацией. Расчет сооружений биологической очистки должен производиться на основе экспериментально полученных кинетических констант, коэффициентов процессов нитрификации и денитрификации и по каждому лимитирующему показателю в зависимости от требований, предъявляемых к качеству очистки. Приведены кинетические константы и коэффициенты, необходимые для расчета очистных сооружений городских сточных вод. Изложен алгоритм расчета, включающий следующие этапы: обоснование исходных данных по расходам и качественному составу сточных вод в соответствии с требуемой степенью надежности (обеспеченностью не менее 85–90%); выбор технологической схемы работы сооружений – количество ступеней и (или) стадий, порядок и схема их работы; определение кинетических констант уравнений ферментативной кинетики трансформации каждого из основных компонентов загрязнений по экспериментальным данным или на основе имеющейся базы данных; предварительный расчет количества избыточного активного ила и уточнение материального баланса по азоту и фосфору; определение лимитирующего компонента загрязнений, на окисление которого потребуется наибольшее время; расчет объема сооружений по лимитирующему компоненту с определением степени очистки по другим компонентам загрязнений. Результаты длительных экспериментальных исследований процессов нитри-денитрификации с различными видами городских и производственных сточных вод позволяют дополнить метод расчета аэротенков, заложенный в СНиП 2.04.03-85, формулами и данными для расчета и оптимизации работы аэротенков с удалением соединений азота и фосфора.

Ключевые слова: сточные воды, биологическая очистка, нитрификация, денитрификация, кинетические константы, активный ил.

СПИСОК ЛИТЕРАТУРЫ

- СНиП 2.04.03-85. Канализация. Наружные сети и сооружения.

- Пат. 2210549, РФ. МПК C 02 F. Способ очистки сточных вод от органических соединений и азота / Швецов В. Н., Морозова К. М., Нечаев И. А., Пушников М. Ю. // Изобретения. Полезные модели. 2003. № 23.

- Пат. 2351551, РФ. МПК C 02 F. Способ очистки сточных вод от органических соединений, азота и фосфора / Швецов В. Н., Морозова К. М., Киристаев А. В., Смирнова И. И. // Изобретения. Полезные модели. 2009. № 10.

- Яковлев С. В., Скирдов И. В., Швецов В. Н. и др. Биологическая очистка производственных сточных вод: Процессы, аппараты и сооружения. – М.: Стройиздат, 1985. 208 с.

- Морозова К. М. Принципы расчета систем биологической очистки сточных вод // Водоснабжение и санитарная техника. 2009. № 1. С. 26–31.

WASTEWATER TREATMENT

Shvetsov V. N., Morozova K. M. Procedure of designing wastewater biological treatment facilities with nutrients removal 42

V. N. SHVETSOV1, K. M. MOROZOVA2

1 Shvetsov Valerii Nikolaevich, Doctor of Engineering, Professor, Chief of department, «NII VODGEO» OJSC

Build. 1, 9 B. Savvinskii lane, 119435 Moscow, Russia, tel.: +7 (499) 245-96-33, e-mail: vst@aha.ru

2 Morozova Kseniia Mikhailovna, PhD (Engineering), Chief research officer, «NII VODGEO» OJSC

Build. 1, 9 B. Savvinskii lane, 119435 Moscow, Russia, tel.: +7 (499) 272-47-54, e-mail: vodgeo@inbox.ru

The procedure of designing wastewater biological treatment facilities with nitrification-denitrification is presented. Biological treatment facilities shall be designed on the basis of experimentally determined kinetic constants, nitrification and denitrification process factors, and for every limiting value depending on the requirements to the quality of treatment. Kinetic constants and factors required for designing municipal wastewater treatment facilities are given. Design algorithm is set out including the following stages: substantiation of the initial data on flow rates and qualitative wastewater composition according to the required level of reliability (more than 85–90% probability); selection of the process flow scheme – number of steps and(or) stages, operation procedure and lay out; determination of kinetic constants of equations of enzyme kinetics of every basic pollution component transformation from experimental data or from the available data base; preliminary calculation of excess activated sludge volume and adjustment of nitrogen and phosphorus mass balance; determination of limiting pollution component that needs maximum time to be oxidized; calculation of the facility capacity by limiting component with determination of the treatment level for other pollution components. The results of the long-term experimental studies of nitrification-denitrification processes with different types of municipal and industrial wastewater provide for supplementing the method of aeration tank design specified in SNiP 2.04.03-85 with formulae and data for design and optimization of aeration tank operation with nitrogen and phosphorus removal.

Key words: wastewater, biological treatment, nitrification, denitrification, kinetic constants, activated sludge.

REFERENCES

- SNiP 2.04.03-85. Kanalizatsiia. Naruzhnye seti i sooruzheniia [SNiP 2.04.03-85. Sewerage. Public Utilities]. (In Russian).

- Pat. 2210549, RF. MPK C 02 F. Sposob ochistki stochnykh vod ot organicheskikh soedinenii i azota [Patent 2210549, RF. IPC C 02 F. Method of eliminating organic compounds and nitrogen from wastewater] Shvetsov V. N., Morozova K. M, Nechaev I. A., Pushnikov M. Iu. Izobreteniia. Poleznye Modeli, 2003, no. 23. (In Russian).

- Pat. 2351551, RF. MPK C 02 F. Sposob ochistki stochnykh vod ot organicheskikh soedinenii, azota i fosfora [Patent 2351551, RF. IPC C 02 F. Method of eliminating organic compounds, nitrogen and phosphorus from wastewater] Shvetsov V. N., Morozova K. M., Kiristaev A. V., Smirnova I. I.  Izobreteniia. Poleznye Modeli, 2009, no. 10. (In Russian).

- Iakovlev S. V., Skirdov I. V., Shvetsov V. N., et al. Biologicheskaia ochistka proizvodstvennykh stochnykh vod: Protsessy, apparaty i sooruzheniia [Biological treatment of industrial wastewater: Processes, equipment and facilities]. Moscow, Stroiizdat Publ., 1985, 208 p. (In Russian).

- Morozova K. M. Printsipy rascheta sistem biologicheskoi ochistki stochnykh vod [Principles of calculation of biological treatment systems]. Vodosnabzhenie i Sanitarnaia Tekhnika, 2009, no. 1, pp. 26–31. (In Russian).

ОБРАБОТКА ОСАДКОВ СТОЧНЫХ ВОД

Данилович Д. А., Ванюшина А. Я. Анаэробное сбраживание – ключевая технология обработки осадков городских сточных вод (часть 2)* 50

Д. А. ДАНИЛОВИЧ1, А. Я. ВАНЮШИНА2

1 Данилович Дмитрий Александрович, кандидат технических наук, заместитель исполнительного директора по инженерно-экологической работе, Российская ассоциация водоснабжения и водоотведения

119334, Россия, Москва, Ленинский проспект, 38, корп. 2, тел.: (495) 939-19-36, e-mail: da_danilovich@mail.ru

2 Ванюшина Анастасия Ярославна, кандидат биологических наук, ведущий инженер-технолог, Группа компаний «Экополимер»

115054, Россия, Москва, Б. Строченовский переулок, 7, тел.: (495) 989-85-04, e-mail: vanyushina@ecopolymer.com

Проанализированы варианты технологий обработки осадка городских сточных вод, различающиеся подходом к стабилизации органического вещества. Описаны негативные технологические и экологические последствия размещения сырых осадков сточных вод на полигонах. Рассмотрены процессы аэробной стабилизации осадка. Сделан вывод, что их применение экономически ограничено ситуациями, в которых возможна почвенная утилизация всего количества осадка. Показано, что классическая технология аэробной стабилизации, активно использовавшаяся в прошлом веке, не соответствует современным требованиям как по энергопотреблению, так и по глубине распада органического вещества. Изложены проблемы отечественного подхода к сбраживанию, заключающегося в использовании термофильного режима с крайне малым временем пребывания осадка в метантенке. Описаны пути решения этих проблем на Курьяновских и Люберецких очистных сооружениях г. Москвы. Сделан вывод, что анаэробное сбраживание осадка сточных вод в метантенках в XXI веке не имеет сравнимых альтернатив для крупных и средних канализационных очистных сооружений. Изложены основные тенденции в применении процесса сбраживания, конструктивного исполнения метантенков, методы интенсификации процесса. Обобщена информация по свойствам биогаза и рассмотрены варианты утилизации. Приведены технологические схемы с использованием сбраживания и его сочетания с другими технологическими процессами обработки осадка (сушка, сжигание), методы утилизации сброженного осадка. Дана информация по экологическим аспектам сбраживания – снижение выбросов парниковых газов, уменьшение «углеродного следа». Рассмотрены технологии очистки возвратных потоков от сооружений обезвоживания сброженного осадка. Приведена рекомендуемая для крупных сооружений современная схема обработки осадка на основе сбраживания.

Ключевые слова: осадок сточных вод, анаэробное сбраживание, метантенк, стабилизация, утилизация биогаза, возвратные потоки, азот, фосфор, энергетический баланс.

СПИСОК ЛИТЕРАТУРЫ

- Данилович Д. А. Энергосбережение и альтернативная энергетика на очистных сооружениях канализации // Водоснабжение и санитарная техника. 2011. № 1. С. 9–20.

- Хамидов М. Г., Стрельцов С. А., Данилович Д. А. Отходы на службе энергетики города // Коммунальный комплекс России. 2009. № 2 (56). С. 46–49.

- Гусева Т. В., Бегак М. В., Молчанова Я. П. Углеродный след коммунальных очистных сооружений: оценка, сокращение, сертификация // Техническое регулирование. 2012. № 9–10. С. 48–54.

- Данилович Д. А., Козлов М. Н., Мойжес О. В., Николаев Ю. А., Казакова Е. А., Грачев В. А. Анаэробное окисление аммония для удаления азота из высококонцентрированных сточных вод // Водоснабжение и санитарная техника. 2010. № 4. С. 49–54.

- Wett B. Development and implementation of a robust deammonification process // Water Science & Technology. 2007. № 56 (7). P. 81–88.

- Figdore B., Wett B., Hell M., Murthy S. Deammonification of dewatering sidestream from thermal hydrolysis-mesophilic anaerobic digestion process: Proceedings of the Water Environment Federation, WEFTEC 2011: Session 1 through Session 10. Р. 249–264 (16).

- Kevbrina M. V., Nikolaev Y. A., Danilovich D. A., Vanyushina A. Ya. Aerobic biological treatment of thermophilically digested sludge // Water Science & Technology. 2011. Vol. 63. No 10. P. 2340–2345.

- Кевбрина М. В., Николаев Ю. А., Данилович Д. А., Ванюшина А. Я. Аэробная биологическая обработка термофильно сброженного осадка: Conference Papiers. IWA Specialist Conference «Water and Wastewater Treatment Plants in Towns and Communities of the XXI Century: Technologies, Design and Operation». Moscow, 2010.

- Данилович Д. А. Переработка раздела «Очистные сооружения» при актуализации СНиП 2.04.03-85 «Канализация. Наружные сети и сооружения» / Проекты развития инфраструктуры города. – М.: Прима-пресс, 2011. Вып. 11.

WASTEWATER SLUDGE TREATMENT

Danilovich D. A., Vaniushina A. Ia. Anaerobic digestion – the key technology of municipal wastewater sludge treatment (part 2) 50

D. A. DANILOVICH1, A. Ia. VANIUSHINA2

1 Danilovich Dmitrii Aleksandrovich, PhD (Engineering), Deputy Executive Director, Chairman of Ecological Council, Russian Water & Wastewater Association

Build. 2, 38 Leninskii ave., 119334 Moscow, Russia, tel.: +7 (495) 939-19-36, e-mail: da_danilovich@mail.ru

2 Vaniushina Anastasiia Iaroslavna, PhD (Biology), Leading process engineer, Ecopolymer Group

7 B. Strochenovskii lane, 115054 Moscow, Russia, tel.: +7 (495) 989-85-04, e-mail: vanyushina@ecopolymer.com

The technologic options of municipal wastewater sludge treatment are reviewed with different approaches to stabilization of organic matter. The technologic and environmental consequences of raw wastewater sludge landfilling are described. Aerobic sludge stabilization processes are considered. It is concluded that their use is restricted to the options when land utilization of the entire sludge volume is possible. It is shown that traditional aerobic stabilization that was widely used last century does not meet the present day requirements both to power consumption and organics decomposition depth. The aspects of the domestic approach to digestion that consist in using thermophilic mode with extremely low retention time of sludge in a digester are considered. The ways of solving these problems at the Kouryanovo and Lyubertsy wastewater treatment facilities in Moscow are described. It is concluded that in the XXI century anaerobic digestion of wastewater sludge in digesters does not have any other comparable alternative for large and mid-sized wastewater treatment facilities. The main trends in using digestion process, digester design, methods of enhancing the process are considered. The information on biogas properties is consolidated and various options of its utilization are reviewed. Process flow schemes with the use of digesting and combining with other sludge treatment processes (drying, incineration), methods of digested sludge utilization are considered. The information on environmental aspects of digestion – reducing greenhouse gas emissions and carbon footprint – is given. The technologies of purification of return flows of digested sludge dewatering are described. The advanced sludge treatment flow scheme based on digestion and recommended for large-scale facilities is given.

Key words: wastewater sludge, anaerobic digestion, digester, stabilization, biogas utilization, return flows, nitrogen, phosphorus, energy budget.

REFERENCES

- Danilovich D. A. Energosberezhenie i al'ternativnaia energetika na ochistnykh sooruzheniiakh kanalizatsii [Power conservation and energy alternatives at the wastewater treatment facilities]. Vodosnabzhenie i Sanitarnaia Tekhnika, 2011, no. 1, pp. 9–20. (In Russian).

- Khamidov M. G., Strel'tsov S. A., Danilovich D. A. Otkhody na sluzhbe energetiki goroda [Waste to energy for the city]. Kommunal'nyi Kompleks Rossii, 2009, no. 2 (56), pp. 46–49. (In Russian).

- Guseva T. V., Begak M. V., Molchanova Ia. P. Uglerodnyi sled kommunal'nykh ochistnykh sooruzhenii: otsenka, sokrashchenie, sertifikatsiia [Carbon footprint of the public-owned wastewater treatment facilities: assessment, reduction, certification]. Tekhnicheskoe Regulirovanie, 2012, no. 9–10, pp. 48–54. (In Russian).

- Danilovich D. A., Kozlov M. N., Moizhes O. V., Nikolaev Iu. A., Kazakova E. A., Grachev V. A. Anaerobnoe okislenie ammoniia dlia udaleniia azota iz vysokokontsentrirovannykh stochnykh vod [Anaerobic oxidation of ammonium for the removal of nitrogen from highly concentrated wastewater]. Vodosnabzhenie i Sanitarnaia Tekhnika, 2010, no. 4, pp. 49–54. (In Russian).

- Wett B. Development and implementation of a robust deammonification process. Water Science & Technology, 2007, no. 56 (7), pp. 81–88.

- Figdore B., Wett B., Hell M., Murthy S. Deammonification of dewatering sidestream from thermal hydrolysis-mesophilic anaerobic digestion process: Proceedings of the Water Environment Federation, WEFTEC 2011: Session 1 through Session 10, pp. 249–264 (16).

- Kevbrina M. V., Nikolaev Y. A., Danilovich D. A., Vanyushina A. Ya. Aerobic biological treatment of thermophilically digested sludge. Water Science & Technology, 2011, vol. 63, no. 10, pp. 2340–2345.

- Kevbrina M. V., Nikolaev Iu. A., Danilovich D. A., Vaniushina A. Ia. Aerobnaia biologicheskaia obrabotka termofil'no sbrozhennogo osadka [Aerobic biological treatment of thermophilically digested sludge]: Conference Papers. IWA Specialist Conference «Water and Wastewater Treatment Plants in Towns and Communities of the XXI Century: Technologies, Design and Operation». Moscow, 2010. (In Russian).

- Danilovich D. A. Pererabotka razdela «Ochistnye sooruzheniia» pri aktualizatsii SNiP 2.04.03-85 «Kanalizatsiia. Naruzhnye seti i sooruzheniia». Proekty razvitiia infrastruktury goroda [Revision of «Wastewater treatment facilities» Section in the process of updating SNiP 2.04.03-85 «Sewerage. Public Utilities». Municipal infrustructure development projects]. Moscow, Prima-press Publ., 2011, issue 11. (In Russian).

ТРУБОПРОВОДНЫЕ СИСТЕМЫ

Примин О. Г. Обеспечение надежности и экологической безопасности напорных канализационных трубопроводов 59

О. Г. ПРИМИН*

* Примин Олег Григорьевич, доктор технических наук, профессор, заместитель генерального директора, ОАО «Институт МосводоканалНИИпроект»

105005, Россия, Москва, Плетешковский переулок, 22, тел.: (495) 956-93-00, e-mail: primin@mvkniipr.ru

Гидравлический удар является одним из значимых факторов, влияющих на экологическую безопасность и надежность систем централизованной канализации города. При возникновении гидравлического удара происходят ощутимые колебания напора, которые могут разрушить трубопровод и связанное с ним гидромеханическое оборудование (насосы, арматуру и проч.). Это приводит к изливу сточной жидкости в окружающую среду, материальному и экологическому ущербу. Основными причинами возникновения гидравлического удара в системе напорной канализации города являются: нарушение внешнего энергоснабжения, несовершенство схем управления основных агрегатов на насосных станциях. Для обеспечения надежной эксплуатации насосных станций предусматривается использование гидромеханических устройств, а также быстродействующих аварийных включений устройств защиты и автоматики, в том числе регулируемого электропривода, и устройств плавного пуска. Определено, что в практике эксплуатации систем водоотведения необходимо применять гидромеханическую арматуру, снижающую силу гидравлического удара. Для предотвращения гидравлического удара используется сброс воды через обводные линии с установкой предохранительного клапана и гидравлических баков. Помимо этого, напорные водоводы со сложным профилем, кроме сброса воды на насосных станциях, должны оснащаться устройствами для впуска и выпуска воздуха в промежуточных точках. Установлено, что для исключения гидравлического удара при эксплуатации канализационных насосных станций необходимо повышение надежности их внешнего энергоснабжения путем создания подлинно независимых источников энергоснабжения, создание комбинированных схем электроснабжения, быстродействующих устройств автоматического включения резерва на напряжение 6–10 кВ в распределительных устройствах крупных станций.

Ключевые слова: канализация, трубопровод, насосная станция, гидравлический удар, экологическая безопасность.

СПИСОК ЛИТЕРАТУРЫ

- Отставнов А. А., Примин О. Г., Хренов К. Е., Харькин В. А., Орлов В. А. О гидроударах в подземных трубопроводах из полиэтиленовых труб // Сантехника. Отопление. Кондиционирование (С.О.К.). 2012. № 3. С. 12–14.

- Пупырев Е. И., Примин О. Г. Методы повышения экологической безопасности трубопроводов канализационных сетей // Экология и промышленность России. 2013. № 3. С. 13–17.

- Правила устройства электроустановок. – М.: Энергоатомиздат, 1986. 124 с.

- Храменков С. В., Примин О. Г. Регламент эксплуатации канализационных насосных станций. – М.: Миклош, 2007. 216 с.

- Отчет НИР 11-278. Разработка мероприятий по повышению надежности работы канализационной сети с целью минимизации экологических рисков. – М., ОАО «МосводоканалНИИпроект», 2012. 76 с.

- Храменков С. В., Хренов К. Е., Козлов М. Н., Шушкевич Е. В., Богомолов М. В., Меньщикова О. А. Опыт внедрения инновационной техники для предотвращения гидравлического удара // Водоснабжение и санитарная техника. 2010. № 10, ч. 1. С. 6–13.

- Лезнов Б. С. Энергосбережение и регулируемый привод в насосных и воздуходувных установках. – М.: Энергоатомиздат, 2006. 306 с.

- Almeida A. T., Ferreira F. J. T. E., Both D. Technical and economical consideration in application of variable speed drives with electric motor systems // IEEE Tracsations on Industry Applications. 2005. Vol. 41. № 1. P. 188–198.

- Крылов Ю. А. Разработка энергосберегающих технологий в топливно-энергетическом хозяйстве города на основе современного электропривода: Автореф. дисс. … доктора техн. наук. – М., МЭИ (ТУ), 2008. 213 с.

PIPELINE SYSTEMS

Primin O. G. Ensuring reliable and environmentally safe operation of pressure sewers 59

O. G. PRIMIN*

* Primin Oleg Grigor'evich, Doctor of Science (Engineering), Professor, Deputy General Director, Institute «MosvodokanalNIIproject» OJSC

22 Pleteshkovskii lane, 105005 Moscow, Russia, tel.: +7 (495) 956-93-00, e-mail: primin@mvkniipr.ru

Surge in a pipeline is a significant factor that is affecting the environmental safety and reliability of the public wastewater disposal system. In case of an emerging surge noticeable pressure fluctuations occur that could cause pipeline burst and damage of the related hydraulic and mechanical equipment (pumps, valves etc.). It results in sewage spills, material and environmental losses. The main causes of an emerging surge in municipal pressure sewers can be as follows: mains failure, imperfections of the control schemes of the main pumping units at the sewage pumping stations. To ensure reliable operation of the pumping stations the use of hydromechanical devices as well as of high-speed emergency switching on automatic and protection devices including controlled electric drives and softstarters is provided. It was determined that in wastewater disposal practice hydromechanical fittings that reduce the surge effect shall be used. To prevent from surge water discharge through bypasses with installing safety valves and hydraulic tanks is used. In addition to wastewater discharge from pumping stations pressure sewers of intricate shape shall be equipped with air intake and relief devices at the intermediate points. It is found that surges in operation of sewage pumping stations can be eliminated by improving the reliability of external power supply by establishing truly independent power sources, designing combined power supply schemes, high-speed automatic switchover devices in 6–10 kV switchboards of big stations.

Key words: wastewater disposal system, pipeline, pumping station, surge, environmental safety.

REFERENCES

- Otstavnov A. A., Primin O. G., Khrenov K. E., Khar'kin V. A., Orlov V. A. O gidroudarakh v podzemnykh truboprovodakh iz polietilenovykh trub [On surges in underground polyethylene pipelines]. Santekhnika. Otoplenie. Konditsionirovanie (S.O.K.), 2012, no. 3, pp. 12–14. (In Russian).

- Pupyrev E. I., Primin O. G. Metody povysheniia ekologicheskoi bezopasnosti truboprovodov kanalizatsionnykh setei [Methods of improving environmental safety of sewage pipelines]. Ekologiia i Promyshlennost' Rossii, 2013, no. 3, pp. 13–17. (In Russian).

- Pravila ustroistva elektroustanovok [Rules of electric installation devices]. Moscow, Energoatomizdat Publ., 1986, 124 p. (In Russian).

- Khramenkov S. V., Primin O. G. Reglament ekspluatatsii kanalizatsionnykh nasosnykh stantsii [Sewage pumping station operating regulations]. Moscow, Miklosh Publ., 2007, 216 p. (In Russian).

- Otchet NIR 11-278. Razrabotka meropriiatii po povysheniiu nadezhnosti raboty kanalizatsionnoi seti s tsel'iu minimizatsii ekologicheskikh riskov [Research project report 11-278. Elaboration of measures to improve sewer reliability to minimize environmental risks]. Moscow, Institute «MosvodokanalNIIproekt» OJSC, 2012, 76 p. (In Russian).

- Khramenkov S. V., Khrenov K. E., Kozlov M. N., Shushkevich E. V., Bogomolov M. V., Men'shchikova O. A. Opyt vnedreniia innovatsionnoi tekhniki dlia predotvrashcheniia gidravlicheskogo udara [Experience of introduction of innovation technique for prevention of a water hammer]. Vodosnabzhenie i Sanitarnaia Tekhnika, 2010, no. 10, p. 1, pp. 6–13. (In Russian).

- Leznov B. S. Energosberezhenie i reguliruemyi privod v nasosnykh i vozdukhoduvnykh ustanovkakh [Power efficiency and variable speed drive in pumping units and air blowers]. Moscow, Energoatomizdat Publ., 2006, 306 p. (In Russian).

- Almeida A. T., Ferreira F. J. T. E., Both D. Technical and economical consideration in application of variable speed drives with electric motor systems. IEEE Transactions on Industry Applications, 2005, vol. 41, no. 1, pp. 188–198.

- Krylov Iu. A. Razrabotka energosberegaiushchikh tekhnologii v toplivno-energeticheskom khoziaistve goroda na osnove sovremennogo elektroprivoda [Developing power efficient technologies for the municipal fuel and energy facilities on the basis of the advanced electric drive: Dissertation for the degree of Doctor in Engineering]. Moscow, MEI (TU), 2008, 213 p. (In Russian).

МЕЖДУНАРОДНЫЕ ВЫСТАВКИ И КОНФЕРЕНЦИИ

IX Международная научно-производственная конференция «Решение проблем экологической безопасности в водохозяйственной отрасли» 66

INTERNATIONAL EXHIBITIONS AND CONFERENCES

«Environmental Safety Management in Water Services Industry» IX International Scientific and Production Conference 66

Международная научно-практическая конференция «Прогрессивные технологии и оборудование для водной отрасли» 68

«Advanced Technologies and Equipment for Water Services Industry» International Scientific and Practical Conference 68

НОВЫЕ ТЕХНОЛОГИИ И ОБОРУДОВАНИЕ

Кофман В. Я. Новые окислительные технологии очистки воды и сточных вод (часть 2) (обзор зарубежных изданий) 70

В. Я. КОФМАН*

* Кофман Владимир Яковлевич, старший научный сотрудник, Всероссийский институт научной и технической информации Российской академии наук

125190, Россия, Москва, ул. Усиевича, 20, тел.: (495) 155-43-08, e-mail: kofman16@rambler.ru; www.kofman.info

Новые окислительные технологии применяются для очистки воды и сточных вод, содержащих вещества, токсичные для микроорганизмов и не поддающиеся биодеградации. Ультразвуковая обработка обеспечивает хорошие результаты, но требует дорогостоящего оборудования и высоких энергозатрат. Ее эффективность может быть повышена использованием катализаторов (диоксида титана) и химических добавок; применяется в качестве предварительного процесса перед биологической очисткой. Мокрое окисление рассматривается как перспективный метод переработки фосфорсодержащих осадков сточных вод. Процесс проводят с использованием кислорода при 160–220 °С и давлении 12–28 бар с добавлением серной кислоты (рН 1,5). Технология окисления в суперкритической воде основана на взаимодействии органических загрязняющих веществ с окислителями в гомогенной суперкритической среде. Процесс проводят при температуре 400–650 °С и давлении 220–350 бар. Проведены опыты по инактивации бактерий Escherichia coli в речной воде с использованием импульсного коронного разряда (степень инактивации 99,8%). Технология плазменной очистки воды пока прошла лишь лабораторные испытания. Перспективным методом очистки воды является использование ферратов (VI) щелочных металлов, позволяющих удалять взвешенные вещества, фосфаты, снижать ХПК и БПК. Электрохимические процессы характеризуются гибкостью применения, безопасностью, селективностью и более высокой рентабельностью, позволяют удалять из сточных вод аммоний и нитраты. В результате применения комбинированных схем могут быть значительно снижены эксплуатационные расходы при высокой эффективности очистки воды в сравнении с индивидуальным использованием новых окислительных технологий. Полагают, что по мере развития научно-исследовательских и конструкторских работ в данной области число промышленных установок, использующих комбинированные схемы очистки сточных вод, будет расти.

Ключевые слова: новые окислительные технологии, мокрое окисление, суперкритическая вода, плазменная очистка, ферраты, электрохимический процесс.

ADVANCED TECHNOLOGIES AND EQUIPMENT

Kofman V. Ia. New advanced oxidation technologies of water and wastewater treatment (part 2) (foreign publications review) 70

V. Ia. KOFMAN*

* Kofman Vladimir Iakovlevich, Senior research assistant, All-Russian Institute of Scientific and Technical Information of the Russian Academy of Sciences

20 Usievicha str., 125190 Moscow, Russia, tel.: +7 (495) 155-43-08, e-mail: kofman16@rambler.ru; www.kofman.info

Newly developed oxidation technologies are applied in purification of water and wastewater containing toxic for microorganisms and non-biodegradable substances. Ultrasonic treatment provides for good results; however requires expensive equipment and much energy. The efficiency of this technology can be improved by using catalysts (titanium dioxide) and chemical additives; it is used as a process preceding biological treatment. Wet oxidation is considered as a perspective method of processing phosphorus-containing wastewater sludge. The process proceeds in the presence of oxygen at 160–220 °С and 12–28 bar pressure with addition of sulfuric acid (рН 1.5). The technology of oxidation in supercritical water is based on interaction of organic pollutants with oxidizers in homogenous supercritical medium. The process is carried out at 400–650 °С temperature and 220–350 bar pressure. Experiments of inactivating Escherichia coli bacteria in river water with the use of impulse corona discharge (inactivation rate 99.8%) were carried out. The technology of plasma water treatment was tested on laboratory scale so far. The use of alkali metal ferrates (VI) that provide for removing suspended solids, phosphates, and reducing COD and BOD is considered as a perspective method of water treatment. Electrochemical processes are characterized by flexibility, safety, selectivity and higher cost-effectiveness; they allow eliminating ammonium and nitrates from wastewater. The use of combined process flow schemes can result in the reduction of operating expenditures at higher water treatment efficiency compared to the use of separate oxidation technologies. It is supposed that as research and design in this area advance the number of industrial scale plants using combined wastewater treatment flow schemes will grow.

Key words: advanced oxidation technologies, wet oxidation, supercritical water, plasma treatment, ferrates, electrochemical process.

СПИСОК ЛИТЕРАТУРЫ/REFERENCES

- Кофман В. Я. Новые окислительные технологии (часть 1) // Водоснабжение и санитарная техника. 2013. № 10. С. 68–78.
Kofman V. Ia. Novye okislitel'nye tekhnologii [New advanced oxidation technologies of water and wastewater treatment (part 1)]. Vodosnabzhenie i Sanitarnaia Tekhnika, 2013, no. 10, pp. 68–78. (In Russian).

- Pang Y. L., Abdulah A. Z., Bhatia S. Review on sonochemical methods in the presence of catalysts and chemical additives for treatment of organic pollutants in waste water. Desalination, 2011, no. 277, pp. 1–14.

- Kubo M., Matsuoka K., Takahashi A., Shibasaki-Kitakawa N., Yonemoto T. Kinetics of ultrasonic degradation of phenol in the presence of TiO2 particles. Ultrasonic Sonochemistry, 2005, no. 12 (4), pp. 263–269.

- Wang J., Ma T., Zhang Z., Zhang X., Jiang Y., Zhang G., Zhao G., Zhao H., Zhang P. Investigation on transition crystal of ordinary rutile TiO2 powder and its sonocatalytic activity. Ultrasonic Sonochemistry, 2007, no. 14 (2), pp. 246–252.

- Wang Y., Zhao D., Ma W., Chen C., Zhao J. Enhanced sonocatalytic degradation of azo dyes by Au/TiO2. Environmental Science and Technology, 2008, no. 42 (16), pp. 6173–6178.

- Wang J., Liu Y., Zhang Z., Deng Y., Zhang L., Liu B., Xu R., Zhang X. Sonocatalytic degradation of azo fuchsine in the presence of the Co-doped and Cr-doped mixed crystal TiO2 powders and comparison of their sonocatalytic activity. Journal of Hazardous Materials, 2009, no. 170 (1), pp. 398–404.

- Zheng W., Maurin M., Tarr M. A. Enchancement of sonochemical degradation of phenol using hydrogen atom scavengers. Ultrasonic Sonochemistry, 2005, no. 12 (4), pp. 313–317.

- Goel M., Hongqiang H., Mujumdar A. S., Ray M. B. Sonochemical decomposition of volatile and non-volatile organic compounds – a comparative study. Water Research, 2004, no. 38 (19), pp. 4247–4261.

- Guo Z., Feng R. Ultrasonic irradiation-induced degradation of low-concentration bisphenol A in aqueous solution. Journal of Hazardous Materials, 2009, no. 163 (2–3), pp. 855–860.

- Merouani S., Hamdaoui O., Saoudi F., Chiba M. Sonochemical degradation of Rhodamine B in aqueous phase: effects of additives. Chemical Engineering Journal, 2010, no. 158 (3), pp. 550–557.

- Golash N., Gogate P. R. Degradation of dichlorvos containing wastewater using sonochemical reactors. Ultrasonic Sonochemistry, 2012, no. 19 (5), pp. 1051–1060.

- Blocher C., Niewersch C., Melin T. Phosphorus recovery from sewage sludge with a hybrid process of low pressure wet oxidation and nanofiltration. Water Research, 2012, no. 46, pp. 2009–2019.

- Loppinet-Sereani A., Aymonier C., Canseli F. Supercritical water for environmental technology. Chemical Technology and Biotechnology, 2010, no. 85, pp. 583–591.

- Xu D., Wang S., Tang X., Gong Y., Guo Y., Wang Y., Zhang J. Design of the first pilot scale plant of China for supercritical water oxidation of sewage sludge. Chemical Engineering Research and Design, 2012, no. 90 (2), pp. 288–297.

- Takahashi F., Oshima Y., Fukushi K., Yamamoto K. Enhanced oxidation of alkali metal acetate in supercritical water. Chemistry Letters, 2012, no. 41, pp. 267–269.

- Qiu K.-J, Wang Z.-Z. Experimental study on coking wastewater treatment by supercritical water oxidation. Gong Yongshui yu Feishui (Industrial Water and Wastewater), 2012, no. 43 (2), pp. 22–24, 37.

- Zhu N., Zhang F. Progress on the dehalogenation mechanisms of persistent organic halogenated compounds in subcritical water. Huangjing Huaxue (Environmental Chemistry), 2012, no. 31 (4), pp. 415–421.

- Wen X. Q., Wang M., Liu X. H. Effect of electrode configuration on the wastewater treatment by underwater electrical streamer discharge. IEEE Transactions on Plasma Science, 2012, no. 40 (4), pp. 1089–1093.

- Foster J., Sommers B. S., Gucker S. N., Blankson I. M., Adamovsky G. Perspective on the interaction of plasmas with liquid water for water purification. IEEE Transactions on Plasma Science, 2012, no. 40 (5), pp. 1311–1323.

- Li S., Hu S., Zhang H. Formation of hydroxyl radicals and hydrogen peroxide by a novel nanosecond pulsed plasma power in water. IEEE Transactions on Plasma Science, 2012, no. 40 (1), pp. 63–67.

- Перфильев Ю. Д., Куликов Л. А., Дедушенко С. К. Новая ферратная технология очистки воды. http://www.kge.msu.ru/ozone/archives/1rus_conf_pr/Presentations/Perfiliev.pdf.
Perfil'ev Iu. D., Kulikov L. A., Dedushenko S. K. Novaia ferratnaia tekhnologiia ochistki vody [Advanced ferrate water treatment technology]. http://www.kge.msu.ru/ozone/archives/1rus_conf_pr/Presentations/Perfiliev.pdf. (In Russian).

- Alsheyab M., Jiang J. Q., Stanford C. On-line production of ferrate with an electrochemical method and its potential application for wastewater treatment: A review. Journal of Environmental Management, 2009, no. 90 (3), pp. 1350–1356.

- Anquandah A. K., Sharma V. K., Knight D. A., Batchu S. R., Gardinali P. R. Oxidation of trimethoprim by ferrate (VI): Kinetics, products and antibacterial activity. Environmental Science and Technology, 2011, no. 45 (24), pp. 10575–10581.

- Sharma V. K. Oxidation of inorganic compounds by ferrate (VI) and ferrate (V): One-electron and two-electron transfer steps. Environmental Science and Technology, 2010, no. 44 (13), pp. 5148–5152.

- Yang B., Ying G.-G., Zhao J.-L., Liu S., Zhou L.-J., Chen F. Removal of selected endocrine disrupting chemicals (EDCs) and pharmaceuticals and personal care products (PPCPs) during ferrate (VI) treatment of secondary wastewater effluents. Water Research, 2012, no. 46 (7), pp. 2194–2204.

- Liu C. S. Higgins C. P., Wang F., Shin W.-H. Effect of temperature on oxidative transformation of perfluorooctanoic acid (PFOA) by persulfate activation in water. Separation and Purification Technolology, 2012, no. 91, pp. 46–51.

- Chen W.-S., Su Y.-C. Removal of dinitrotoluenes in wastewater by sono-activated persulfate. Ultrasonic Sonochemistry, 2012, no. 19 (4), pp. 921–927.

- Muhammad S., Shukla P. R., Tade O., Wang S. Heterogeneous activation of peroxymonosulphate by supported ruthenium catalysis for phenol degradation in water. Journal of Hazardous Materials, 2012, no. 215–216, pp. 183–190.

- Liang C., Lin Y.-T., Shih W.-H. Treatment of trichloroethylene by adsorption and persulfate oxidation in batch studies. Industrial Engineering and Chemical Research, 2009, no. 48 (18), pp. 8373–8380.

- Sires I., Brillas E. Remediation of water pollution caused by pharmaceutical residues based on electrochemical separation and degradation technologies: A review. Environment International, 2012, no. 40, pp. 212–229.

- Fernandes A., Pacheco M. J., Ciriaco L., Lopes A. Anodic oxidation of a biological treated leachate on a boron-doped diamond. Journal of Hazardous Materials, 2012, no. 199–200, pp. 82–87.

- Comninellis C., Kapalka A., Malato S., Parsons S. A., Poulios I., Mantzavinos D. Advanced oxidation processes for water treatment: advances and trends for R&D. Journal of Chemical Technology and Biotechnology, 2008, no. 83, pp. 769–776.

- Harif T., Khai M., Adin A. Electrocoagulation versus chemical coagulation: Coagulation/ flocculation mechanisms and resulting floc characteristics. Water Research, 2012, no. 46 (10), pp. 3177–3188.

- Wang K., Liu S., Zhang Q., He Y. Pharmaceutical wastewater treatment by internal microelectrolysis-coagulation, biological treatment and activated carbon sorption. Environmental Technology, 2009, no. 30, pp. 1469–1474.

- Jeong J., Lee J. Electrochemical oxidation of industrial wastewater with the tube type electrolysis module system. Separation and Purification Technology, 2012, no. 84, pp. 35–40.

- Perez G., Saiz J., Ibanez R., Urtiaga A. M., Ortiz I. Assessment of the formation of inorganic oxidation by-products during the electrocatalytic treatment of ammonium from landfill leachates. Water Research, 2012, no. 46 (8), pp. 2579–2590.

- Mook W. T., Chakrabarti M. H., Aroua M. K., Khan G. M. A., Ali B. S., Islam M. S., Abu Hassan M. A. Removal of total ammonia nitrogen (TAN), nitrate and total organic carbon (TOC) from aquaculture wastewater using electrochemical technology: A review. Desalination, 2012, no. 285, pp. 1–13.

- Pikaar I., Li E., Rozendal R. A., Yuan Z., Keller J., Rabaey K. Long-term field test of an electrochemical method for sulfide removal from sewage. Water Research, 2012, no. 46 (9), pp. 3085–3093.

- Oller I., Malato S., Sanchez-Perez J. A. Combination of advanced oxidation processes and biological treatments for wastewater decontamination: A review. Science of the Total Environment, 2011, no. 409, pp. 4141–4166.

- Kastaned F., Maleterova Y., Kastanek P. Combination of advanced oxidation and/or reductive dehalogenation and biodegradation for the decontamination of waters contaminated with chlorinated organic compounds. Separation Science and Technology, 2007, no. 42, pp. 1613–1625.

- Lapertot M., Ebrahimi S., Dazio S., Rubinelli A., Pulgarin C. Photo-Fenton and bilogical integrated process for degradation of a mixture of pesticides. Journal of Photochemistry and Photobiology, 2007, no. 186, pp. 34–40.

- Ballesteros-Martin M. M., Sanchez-Perez J. A., Garcia-Sanchez J. L., Montes de Oca L., Casas Lopez J. L., Oller I. Degradation of alachlor and pyrimethanil by combined photo-Fenton and biological oxidation. Journal of Hazardous Materials, 2008, no. 155, pp. 342–349.

- Sirtori C., Zapata A., Oller I., Gernjak W., Aguera A., Malato S. Decontamination industrial pharmaceutical wastewater by combining solar photo-Fenton and biological treatment. Water Research, 2009, no. 43, pp. 661–668.

- Gunnarson L., Adolfsson-Erici B., Rutgersson C., Forlin L., Larsson D. G. J. Comparison of six different sewage treatment processes-reduction of estrogenic substances and effects on gene expression in exposed male fish. Science of the Total Environment, 2009, no. 407, pp. 5235–5242.

- Carballa M., Manterola G., Larrea L., Ternes T., Omil F., Lema J. M. Influence of ozone pretreatment on sludge anaerobic digestion: removal of pharmaceutical and personal care products. Chemosphere, 2007, no. 67, pp. 1444–1452.

- Prieto-Rodriguez L., Miralles-Cuevas S., Oller I., Aguera A., Li Puma G., Malato S. Treatment of emerging contaminants in wastewater treatment plants (WWTP) effluents by solar photocatalysis using low TiO2 concentrations. Journal of Hazardous Materials, 2012, no. 211–212, pp. 131–137.

- De la Cruz N., Gimenez J., Espugas S., Grandjean D., de Alencastro L. F., Pulgarin C. Degradation of 32 emergent contaminants by UV and neutral photo-Fenton in domestic wastewater effluent previously treated by active sludge. Water Research, 2012, no. 46 (6), pp. 1947–1957.

- Garsia-Montano J., Torrades F., Garcia-Hortal J. A., Domenech X., Peral J. Degradation of Procion Red H-E7B reactive dye by coupling a photo-Fenton system with a sequencing batch reactor. Journal of Hazardous Materials, 2006, no. 134, pp. 220–229.

- Harrelkas F., Paulo A., Alves M. M., El Khadir L., Zaharaa O., Pons M. N. Photocatalytic and combined anaerobic-photocatalytic treatment of textile dyes. Chemosphere, 2008, no. 72, pp. 1816–1822.

- Hai F. L., Yamamoto K., Fukushi K. Development of a submerged membrane fungi reactor for textile wastewater treatment. Desalination, 2006, no. 192, pp. 315–322.

- Soloman P. A., Basha C. A., Velan M., Balasubramanian N., Marimuthu P. Augmentation of biodegradability of pulp and paper industry wastewater by electrochemical pretreatment and optimization by RSM. Separation and Purification Technology, 2009, no. 69, pp. 109–117.

- Balcioglu I. A., Sarac C., Kivilcimdan C., Tarlan E. Application of ozonation and biotreatment for forest industry wastewater. Ozone Science & Engineering, 2006, no. 28, pp. 431–436.

- Dogruel S., Genceli E. A., Babuna F. G., Orhon D. An investigation on the optimal location of ozonation within biological treatment for a tannery wastewater. Journal of Chemical Technology and Biotechnology, 2006, no. 81, pp. 1877–1885.

- Vidal G., Nietro J., Mansilla H. D., Bornhardt C. Combined oxidative and biological treatment of separated streams of tannery wastewater. Water Science and Technology, 2004, no. 49, pp. 287–292.

- Bressan M., Liberatore L., D'Alessandro N., Tonucci L., Belli C., Ranalli G. Improved combined chemical and biological treatments of olive mill wastewater. Journal of Agricultural and Food Chemistry, 2004, no. 5, pp. 1228–1233.

- Khoufi S., Aloui F., Sayadi S. Pilot scale hybrid process for olive mill wastewater treatment and reuse. Chemical Engineering Processes, 2009, no. 48, pp. 643–650.

- Sangave P. C., Gogate P. R., Pandit A. B. Combination of ozonation with conventional aerobic oxidation for distillery wastewater treatment. Chemosphere, 2007, no. 68, pp. 32–41.

- Neczai E., Kacprzak M., Lach J., Okoniewaska E. Effect of sonication on combined treatment of landfill leachate and domestic sewage in SBR reactor. Desalination, 2007, no. 204, pp. 227–233.

- Fernandes A., Pacheco M. J., Ciriaco L., Lopes A. Anodic oxidation of a biologically treated leachate on a boron-doped diamond anode. Journal of Hazardous Materials, 2012, no. 199–200, pp. 82–87.

- Oller I., Malato S., Sanchez-Perez J. A., Maldonado M. I., Gernjak W., Perez-Estrada L. A. Pre-industrial-scale combined solar photo-Fenton and immobilized biomass activated-sludge biotreatment. Industrial and Engineering Chemistry Research, 2007, no. 46, pp. 7467–7475.

- Zapata A., Malato S., Sanchez-Perez J. A., Oller I., Maldonado M. I. Scale-up strategy for a combined solar photo-Fenton/biological system for remediation of pesticide-contaminated water. Catalysis Today, 2010, no. 151, pp. 100–106.

- Di Laconi C., Ramadori R., Lopez A. The effect of ozone on tannery wastewater biological treatment at demonstrative scale. Bioresource Technology, 2009, no. 100, pp. 6121–6124.