ПИТЬЕВОЕ ВОДОСНАБЖЕНИЕ

УДК 628.11(470.6)

Михеева И. В. Водные объекты и состояние водоснабжения на территории Южного федерального округа 4

И. В. МИХЕЕВА*

* Михеева Ирина Владимировна, главный специалист-эксперт Управления Федеральной службы по надзору в сфере защиты прав потребителей и благополучия человека по Ростовской области

344019, Россия, г. Ростов-на-Дону, ул. 18 Линия, 17, тел.: (863) 251-05-92, e-mail: mikheeva@rpndon.ru

Питьевая вода является существенным фактором формирования здоровья населения и уровня жизни. Проблема сохранения качества воды как фактора, существенно влияющего не только на инфекционную, но и на общую заболеваемость населения, является одной из приоритетных задач в деятельности государственной санитарно-эпидемиологической службы. Приведены данные по водоисточникам и состоянию водоснабжения на территории Южного федерального округа. Практически все источники, как поверхностные, так и подземные, во всех субъектах Южного федерального округа подвержены антропогенному и техногенному воздействию с различной степенью интенсивности. Рассмотрены причины ухудшения качества воды поверхностных водных объектов. Основной причиной загрязнения водных объектов является сброс большого объема неочищенных и недостаточно очищенных сточных вод, ливневых вод, дренажных вод с оросительных систем в водоемы. Неудовлетворительная работа очистных сооружений канализации, наряду с отставанием развития канализационных сетей, отсутствием обеззараживания сточных вод, приводит к загрязнению подземных вод и рек, а также к подтоплению населенных пунктов грунтовыми водами. Даны предложения по проведению ряда мероприятий для улучшения состояния водоснабжения и водоотведения населенных пунктов Южного федерального округа и обеспечения эпидемиологического благополучия населения.

Ключевые слова: водные ресурсы, антропогенное загрязнение, водоснабжение, водоотведение, питьевая вода, контроль качества воды.

DRINKING WATER SUPPLY

Mikheeva I. V. Water bodies and state of the water supply within the territory of the Southern Federal District 4

I. V. MIKHEEVA*

* Mikheeva Irina Vladimirovna, Chief Specialist-Expert, Rostov Area Department, Federal Supervision Agency for Customer Protection and Human Welfare

17, 18th Line St., 344019, Rostov-on-Don, Russian Federation, tel.: +7 (863) 251-05-92, e-mail: mikheeva@rpndon.ru

Drinking water has been an essential factor of the formation of public health and life quality. The problem of maintaining water quality as a factor producing a significant effect not only on the infectious case rate but also on the sick rate of the population is one of the priority tasks of the activities of the state sanitary and epidemiological service. The data on the water supply sources and state of the public water supply within the territory of the Southern Federal District is presented. Virtually all the water sources, both surface and underground, in all the substituent territories of the Southern Federal District are subject to the anthropogenic and technogenic impact of different intensity. The causes of the water quality deterioration in the surface water bodies are considered. The main cause of the pollution is discharging big amounts of raw or partially treated wastewater, storm water, drainage water from the irrigation systems into the water bodies. Unsatisfactory performance of the wastewater treatment facilities alongside with the delayed construction of the wastewater collection systems, lack of the effluent disinfection result in the pollution of the underground waters and rivers as well as in flooding the communities with underground water. The suggestions on certain measures to be taken to improve the state of the water supply and wastewater disposal of the communities of the Southern Federal District and provide for the epidemiological welfare of the population are presented.

Key words: water resources, anthropogenic pollution, water supply, wastewater disposal, drinking water, water quality control.

УДК 628.16.094.3-926.214

Алексеева Л. П., Алексеев С. Е., Корса-Вавилова Е. В., Шмелев А. Я. Использование озона при очистке поверхностных вод для водоснабжения небольших населенных пунктов 10

Л. П. АЛЕКСЕЕВА1, С. Е. АЛЕКСЕЕВ2, Е. В. КОРСА-ВАВИЛОВА3, А. Я. ШМЕЛЕВ4

1 Алексеева Любовь Павловна, кандидат технических наук, заведующая лабораторией, ОАО «НИИ КВОВ»

125371, Россия, Москва, Волоколамское шоссе, 87, тел.: (495) 491-20-41, e-mail: vodcom@mail.ru

2 Алексеев Станислав Евгеньевич, кандидат технических наук, доцент кафедры «Водоснабжение и водоотведение», Национальный исследовательский Московский государственный строительный университет

129337, Россия, Москва, Ярославское шоссе, 26, тел.: (499) 183-27-56, e-mail: alekseevse@mgsu.ru

3 Корса-Вавилова Елена Викторовна, генеральный директор ЗАО «Московские озонаторы»

127273, Россия, Москва, Березовая аллея, 10, тел.: (499) 372-12-18, e-mail: info@mozon.ru

4 Шмелев Алексей Ярославович, главный конструктор, ЗАО «Московские озонаторы»

127273, Россия, Москва, Березовая аллея, 10, тел.: (499) 372-12-18, e-mail: shmelev.ay@mozon.ru

Рассмотрены варианты использования озона для очистки воды из поверхностных источников водоснабжения с повышенным содержанием органических веществ на блочно-модульных установках небольшой производительности. Показано влияние озонирования на изменение основных показателей качества воды. Применение озона для очистки воды поверхностных водоисточников расширяет возможности регулирования технологического процесса при частых сезонных изменениях состава исходной воды, повышает качество очищенной воды, что особенно важно при эксплуатации установок небольшой производительности. Кроме того, озонирование позволяет существенно уменьшить дозы реагентов (коагулянта и флокулянта), а в ряде случаев (в отдельные периоды года) полностью отказаться от их использования. Технологические исследования, проведенные на блочно-модульной установке фирмы «Московские озонаторы», показали, что очистку воды р. Волги для питьевого водоснабжения методом озонирования можно осуществлять по двум технологическим схемам: озонирование и сорбция на угольных фильтрах; коагулирование, озонирование и сорбция. Определены параметры и условия применения методов озонирования, коагулирования и сорбции, установлена их эффективность при минимально необходимых дозах реагентов. Установлено, что применение озона на блочно-модульных станциях очистки воды поверхностных водоисточников позволяет существенно улучшить качество питьевой воды независимо от сезонных изменений состава исходной воды, повысить надежность системы водоснабжения в целом по всем показателям, включая микробиологические. Отмечено влияние некоторых конструктивных и технологических решений в сооружениях на эффективность очистки воды.

Ключевые слова: поверхностный водоисточник, органические загрязняющие вещества, питьевая вода, озонирование, коагулирование, блочно-модульная установка.

СПИСОК ЛИТЕРАТУРЫ

- Драгинский В. Л., Алексеева Л. П., Самойлович В. Г. Озонирование в процессах очистки воды. – М.: ДеЛи Принт, 2007. 400 с.

- Алексеев С. Е., Алексеева Л. П., Курова Л. В. Оценка эффективности различных реагентов при очистке подземных вод от железа и органических веществ // Водоснабжение и санитарная техника. 2016. № 1. С. 5–8.

- Алексеева Л. П., Драгинский В. Л. Применение озона для очистки воды различных водоисточников // Химия в интересах устойчивого развития. 1995. Т. 5. С. 26–41.

Alekseeva L. P., Alekseev S. E., Korsa-Vavilova E. V., Shmelev A. Ia. The use of ozone in surface water purification for public water supply of small communities 10

L. P. ALEKSEEVA1, S. E. ALEKSEEV2, E. V. KORSA-VAVILOVA3, A. Ia. SHMELEV4

1 Alekseeva Liubov' Pavlovna, Ph. D. (Engineering), Laboratory Chief, «NII KVOV» OJSC

87 Volokolamskoe Hwy., 125371, Moscow, Russian Federation, tel.: +7 (495) 491-20-41, e-mail: vodcom@mail.ru

2 Alekseev Stanislav Evgen'evich, Ph. D. (Engineering), Assistant Professor, «Water Supply and Wastewater Disposal» Department, National Research Moscow State University of Civil Engineering

26 Iaroslavskoe Hwy., 129337, Moscow, Russian Federation, tel.: +7 (499) 183-27-56, e-mail: alekseevse@mgsu.ru

3 Korsa-Vavilova Elena Viktorovna, General Director, «Moskovskie Ozonatory» CJSC

10 Berezovaia Lane, 127273, Moscow, Russian Federation, tel.: +7 (499) 372-12-18, e-mail: info@mozon.ru

4 Shmelev Aleksei Iaroslavovich, Chief Designer, «Moskovskie Ozonatory» CJSC

10 Berezovaia Lane, 127273, Moscow, Russian Federation, tel.: +7 (499) 372-12-18, e-mail: shmelev.ay@mozon.ru

Two options of using ozone in purification of water from surface water supply sources with excess concentrations of organics in block-modular plants of small capacity are considered. Ozone effecting the changes in the basic water quality parameters is shown. The use of ozone in surface water purification extends the potential of the technologic process control under frequent seasonal fluctuations of the raw water quality, improves the quality of water after purification which is most important in the operation of small capacity plants. Besides, ozonation provides for reducing significantly the dosages of chemicals (coagulant and flocculant), and in some cases (during certain seasons) eliminating completely their use. The process studies carried out in a block-modular plant manufactured by «Moskosvskie Ozonatory» Company showed that the ozonation of the Volga River water for public water supply can be carried out using two process schemes: ozonation and sorption in carbon filters; or coagulation, ozonation and sorption. The parameters and conditions of using the ozonation, coagulation and sorption methods were specified; their efficiency at the minimum required chemical dosages was determined. It was stated that the use of ozone in block-modular plants for surface water purification provided for improving significantly the quality of drinking water regardless of the seasonal fluctuations in raw water quality; improving the integrity of the water supply system by all the parameters including microbiological ones. The effect of certain structural and technological solutions at the facilities on the water purification efficiency was noted.

Key words: surface water supply source, organic pollutants, drinking water, ozonation, coagulation, block-modular plant.

REFERENCES

- Draginskii V. L., Alekseeva L. P., Samoilovich V. G. Ozonirovanie v protsessakh ochistki vody [Ozonation in water purification processes. Moscow, DeLi Print Publ., 2007, 400 p.].

- Alekseev S. E., Alekseeva L. P., Kurova L. V. [Evaluation of the efficiency of different chemicals in removing iron and organics from underground water]. Vodosnabzhenie i Sanitarnaia Tekhnika, 2016, no. 1, pp. 5–8. (In Russian).

- Alekseeva L. P., Draginskii V. L. [The use of ozone in the purification of water from different water sources]. Khimiia v Interesakh Ustoichivogo Razvitiia, 1995, v. 5, pp. 26–41. (In Russian).

- УДК 628.164/.167

Фесенко Л. Н., Игнатенко С. И., Пчельников И. В., Федотов Р. В., Черкесов А. Ю. Технологические и проектные решения реконструкции сооружений очистки воды р. Дон (на примере ВОС хутора Хапры Мясниковского района Ростовской области) 19

Л. Н. ФЕСЕНКО1, С. И. ИГНАТЕНКО2, И. В. ПЧЕЛЬНИКОВ3, Р. В. ФЕДОТОВ4, А. Ю. ЧЕРКЕСОВ5

1 Фесенко Лев Николаевич, доктор технических наук, профессор кафедры «Водное хозяйство, инженерные сети и защита окружающей среды», Южно-Российский государственный политехнический университет (НПИ) имени М. И. Платова

346428, Россия, Ростовская область, г. Новочеркасск, ул. Просвещения, 132, тел.: (8635) 25-53-34, e-mail: 65613@mail.ru

2 Игнатенко Сергей Иванович, кандидат технических наук, заместитель директора по научной работе, ООО НПП «ЭКОФЕС»

346400, Россия, Ростовская область, г. Новочеркасск, Баклановский проспект, 200В, тел.: (8635) 26-56-13, e-mail: 65613@mail.ru

3 Пчельников Игорь Викторович, кандидат технических наук, старший преподаватель кафедры «Водное хозяйство, инженерные сети и защита окружающей среды», Южно-Российский государственный политехнический университет (НПИ) имени М. И. Платова

346428, Россия, Ростовская область, г. Новочеркасск, ул. Просвещения, 132, тел.: (8635) 25-53-34, e-mail: pchelnikov.igor@mail.ru

4 Федотов Роман Валерьевич, кандидат технических наук, доцент кафедры «Водное хозяйство, инженерные сети и защита окружающей среды», Южно-Российский государственный политехнический университет (НПИ) имени М. И. Платова

346428, Россия, Ростовская область, г. Новочеркасск, ул. Просвещения, 132, тел.: (8635) 25-53-34, e-mail: fedotov86@mail.ru

5 Черкесов Аркадий Юльевич, кандидат технических наук, главный инженер, ООО НПП «ЭКОФЕС»

346400, Россия, Ростовская область, г. Новочеркасск, Баклановский проспект, 200В, тел.: (8635) 26-56-13, e-mail: 65613@mail.ru

Вода р. Дон используется в качестве источника питьевого водоснабжения Ростова-на-Дону, Таганрога, Новочеркасска и других городов. С января по апрель (во время зимнего ледостава) речная вода характеризуется повышенной общей жесткостью (до 10–12 Ж), а также наличием сине-зеленых и диатомовых водорослей, содержание которых в летний период может достигать 100·106 кл/л воды. Перспективным методом удаления микроводорослей является механическая ситовая фильтрация. Фильтры устанавливаются, как правило, в самом начале технологического процесса и предназначены для удаления крупных взвешенных частиц, фито- и зоопланктона. Современные микрофильтры характеризуются простотой конструкции, высокой производительностью, компактными размерами и низким расходом промывной воды, их работа автоматизирована. Ввиду высокой стоимости ионообменных и мембранных технологий, а также сопутствующего образования значительного количества вторичных, трудно утилизируемых сбросных высокоминерализованных растворов была предложена технология снижения жесткости методом едконатрового умягчения. Экспериментально подтверждена целесообразность и эффективность использования этого метода. На примере водоочистных сооружений хутора Хапры Ростовской области с водозабором из нижнего течения р. Дон обоснована и разработана технологическая схема реконструкции водоочистной станции с доведением качества воды до нормативов СанПиН 2.1.4.1074-01. Применение данной технологии исключает образование жидких отходов, подлежащих сбросу в канализацию, естественные водоемы или на рельеф.

Ключевые слова: водоподготовка, фитопланктон, едконатровое умягчение, реагент, коагулирование, микрофильтр.

Fesenko L. N., Ignatenko S. I., Pchel'nikov I. V., Fedotov R. V., Cherkesov A. Iu. Technological and design solutions of upgrading the Don River water treatment facilities (the case of water treatment facilities of Khapry farm, Miasnikovskii district, Rostov Region) 19

L. N. FESENKO1, S. I. IGNATENKO2, I. V. PCHEL'NIKOV3, R. V. FEDOTOV4, A. Iu. CHERKESOV5

1 Fesenko Lev Nikolaevich, Doctor of Engineering, Professor, «Water Industry, Utility Networks and Environmental Protection» Department, Platov South-Russian State Polytechnic University (NPI)

132 Prosveshcheniia St., 346428, Novocherkassk, Rostov Region, Russian Federation, tel.: +7 (8635) 25-53-34, e-mail: 65613@mail.ru

2 Ignatenko Sergei Ivanovich, Ph. D. (Engineering), Deputy Director for Science, «ECOFES» Research and Production Enterprise, LLC

200B Baklanovskii Ave., 346400, Novocherkassk, Rostov Region, Russian Federation, tel.: +7 (8635) 26-56-13, e-mail: 65613@mail.ru

3 Pchel'nikov Igor' Viktorovich, Ph. D. (Engineering), Senior Lecturer, «Water Industry, Utility Networks and Environmental Protection» Department, Platov South-Russian State Polytechnic University (NPI)

132 Prosveshcheniia St., 346428, Novocherkassk, Rostov Region, Russian Federation, tel.: +7 (8635) 25-53-34, e-mail: pchelnikov.igor@mail.ru

4 Fedotov Roman Valer'evich, Ph. D. (Engineering), Assistant Professor, «Water Industry, Utility Networks and Environmental Protection» Department, Platov South-Russian State Polytechnic University (NPI)

132 Prosveshcheniia St., 346428, Novocherkassk, Rostov Region, Russian Federation, tel.: +7 (8635) 25-53-34, e-mail: fedotov86@mail.ru

5 Cherkesov Arkadii Iul'evich, Ph. D. (Engineering), Chief Engineer, «ECOFES» Research and Production Enterprise, LLC

200B Baklanovskii Ave., 346400, Novocherkassk, Rostov Region, Russian Federation, tel.: +7 (8635) 26-56-13, e-mail: 65613@mail.ru

The Don River water is used as a source of the public water supply of Rostov-on-Don, Taganrog, Novocherkassk and other cities. From January till April (during the winter freeze-up) the river water is characterized by increased total hardness (up to 10–12) as well as by the presence of blue-green and diatom algae in concentrations reaching 100·106 c/l water in summer periods. Mechanical sieve filtration is an advanced technique of microalgae removal. Filters are installed as a rule at the very beginning of the technological process and are designed for removing coarse suspended particles, phyto- and zooplankton. Advanced microfilters are characterized by simple design, high output, compact size and low consumption of wash water; their operation is automated. In view of the high cost of ion exchange and membrane technologies and accompanying generation of considerable amounts of secondary difficult to utilize highly mineralized waste solutions the technology of hardness reduction by caustic soda softening was suggested. The expediency and efficiency of applying this method were experimentally confirmed. In case of the water treatment facilities of Khapry farm in the Rostov Region with the water intake in the lower reach of the Don River the process flow scheme of upgrading the water treatment plant providing for the water quality meeting the Sanitary Rules and Regulations 2.1.4.1074-01 was substantiated and developed. The use of this technology eliminates generation of liquid wastes subject to the discharge into the public sewer, natural water bodies or land disposal.

Key words: water treatment, phytoplankton, caustic soda softening, chemical, coagulation, microfilter.

ОХРАНА ОКРУЖАЮЩЕЙ СРЕДЫ

УДК 550.424:574

Болдырев К. А., Уткин С. С., Казаков С. В. Особенности прогнозирования распределения металлов между водной фазой и донными отложениями (на примере радионуклидов стронция и цезия) 27

К. А. БОЛДЫРЕВ1, С. С. УТКИН2, С. В. КАЗАКОВ3

1 Болдырев Кирилл Александрович, кандидат технических наук, старший научный сотрудник, Институт проблем безопасного развития атомной энергетики Российской академии наук

115191, Россия, Москва, Большая Тульская ул., 52, тел.: (495) 955-23-35, e-mail: kaboldyrev@ibrae.ac.ru

2 Уткин Сергей Сергеевич, кандидат технических наук, заведующий отделением, Институт проблем безопасного развития атомной энергетики Российской академии наук

115191, Россия, Москва, Большая Тульская ул., 52, тел.: (495) 955-23-77, e-mail: uss@ibrae.ac.ru

3 Казаков Сергей Викторович, кандидат технических наук, старший научный сотрудник, Институт глобального климата и экологии Росгидромета и Российской академии наук

107258, Россия, Москва, Глебовская ул., 20Б, тел.: (499) 169-01-41, e-mail: S_kazakov@inbox.ru

Приведены основные положения разработанного методологического подхода к прогнозированию коэффициента распределения радионуклидов Kd на материале донных отложений и взвешенных веществ водных объектов. В качестве инструмента для прогнозирования коэффициента Kd металлов в условиях изменяющегося химического состава воды предложено использовать геохимическое моделирование. Изложены основные подходы, модели и результаты прогнозирования значений коэффициента Kd металлов в референтном водном объекте (поверхностный водоем – хранилище жидких радиоактивных отходов), для которого характерно значительное загрязнение радионуклидами стронция 90Sr и цезия 137Cs, а также интенсивное снижение минерализации воды. В процессе работы были детально исследованы особенности гидрогеохимического и гидродинамического поведения металлов в системе водных объектов, а также методы геохимического моделирования. Разработана геохимическая термодинамическая модель сорбции металлов (на примере 90Sr и 137Сs) взвешенным веществом и слоем донных отложений модельного водоема, позволяющая рассматривать ее как базис для моделирования эволюции реальных загрязненных водоемов по различным сценариям. Рассчитаны значения коэффициента Kd 90Sr и 137Сs в условиях снижения минерализации воды. Выяснено, что при снижении минерализации значения Kd увеличиваются. Модель может применяться для прогнозирования темпов самоочищения загрязненных водоемов от металлов.

Ключевые слова: поверхностный водоем, донные отложения, сорбция тяжелых металлов, коэффициент распределения радионуклидов, геохимическое моделирование, самоочищение водоема.

СПИСОК ЛИТЕРАТУРЫ

- Mueller B., Duffek A. Similar adsorption parameters for trace metals with different aquatic particles // Aquatic Geochemistry. 2001. V. 7. P. 107–126.

- Пивоваров С. А. Физико-химическое моделирование поведения тяжелых металлов (Си, Zn, Cd) в природных водах: комплексы в растворе, адсорбция, ионный обмен, транспортные явления: Дисс. … канд. хим. наук. – М., МГУ, 2003. 137 с.

- Липатникова О. А. Экспериментальное исследование и термодинамическое моделирование форм нахождения микроэлементов в донных отложениях Иваньковского водохранилища: Дисс. … канд. геолого-минералог. наук. – М., 2011. 130 с.

- Липатникова О. А. Термодинамическая модель поведения тяжелых металлов в эвтрофицированных осадках: Материалы Международной конференции «Современное состояние наук о земле», посвященной памяти Виктора Ефимовича Хаина. Москва, 1–4 февраля 2011 г.

- Соколова О. В., Шестакова Т. В., Гричук Д. В., Шваров Ю. В. Термодинамическое моделирование форм нахождения тяжелых металлов в системе «вода – донные отложения» при автотранспортном загрязнении // Вестник Московского университета. Сер. 4. Геология. 2006. № 3. С. 36–45.

- Соколова О. В. Экспериментальное исследование и термодинамическое моделирование миграции тяжелых металлов в системе «вода – донные отложения» в зоне антропогенного воздействия: Дисс. … канд. техн. наук. – М., 2008. 188 с.

- Беззапонная О. В. Прогноз содержания соединений тяжелых металлов в поверхностных водных объектах: Дисс. … канд. техн. наук. – Екатеринбург, 2004. 162 c.

- Создание методологии экспериментального и натурного изучения процессов аккумуляции и выноса тяжелых металлов в донных отложениях водохранилищ и озер: Отчет о НИОКР. – М., ЗАО «ДАР/ВОДГЕО», 2014.

- Скотникова О. Г., Фесенко С. В. Математическая модель миграции радионуклидов в непроточном водоеме / Радиационная безопасность и защита АЭС. – М.: Энергоиздат, 1986. Вып. 10. С. 117–122.

- Hakanson L., Brittain J. E., Monte L, Heling R., Bergstrom U. Modelling of radiocaesium in lakes – the VAMP model // Environmental Radioactivity. 1996. V. 33. № 3. P. 255–308.

- Heling R. LAKECO, the ecological consequences of an accidental release of radionuclides on a lake ecosystem and it integration into the real-time-on-line decision support system for the off-site Emergency Management following a Nuclear Accident (RODOS): KEMA report Nr. 40352-NUC 93-5852. 1994.

- Kroot M. P. J. M. Behavior of radionuclides. Part of the issue «Description of the modifications of the model IMPAQT». T740.01. Delft Hydraulics Delft, 1992.

- Monte L., Fratarcangeli F., Pompei S., Quaggia S., Andres G. A predictive model for the behaviour of dissolved radioactive substances in stratified lakes // Environmental Radioactivity. 1991. № 13. P. 297–308.

- Казаков С. В., Уткин С. С. Подходы и принципы радиационной защиты водных объектов. – М.: Наука, 2008. 318 с.

- Tessier A., Campbell H. G., Bisson M. Sequential extraction procedure for the speciationof particulate trace metals // Analalytical Chemistry. 1979. V. 51. № 7. P. 844–851.

- Syrovetnik K. Long-term metal retention processes in a peat bog: field studies, data and modeling: Doctoral thesis. – Stockholm, Sweden, Department of Chemical Engineering and Technology Royal Institute of Technology, 2005. 86 p.

- Bradbury M. H., Baeyens B. A generalised sorption model for the concentration dependent uptake of caesium by argillaceous rocks // Journal of Contaminant Hydrology. 2000. № 42. P. 141–163.

- Brouwer E., Baeyens B., Maes A., Cremers A. Cesium and rubidium ion equilibria in illite clay // Journal of Physical Chemistry. 1983. № 87. P. 1213–1219.

- Generic models for use in assessing the impact of discharges of radioactive substances to the environment: Safety Report Series № 19. – Vienna, IAEA, 2001.

- Методика разработки нормативов допустимых сбросов радиоактивных веществ в водные объекты (ДС-2010). – М., Федеральная служба по экологическому, технологическому и атомному надзору, 2010.

ENVIRONMENTAL PROTECTION

Boldyrev K. A., Utkin S. S., Kazakov S. V. Specificities of predicting metal distribution between aqueous phase and bottom sediments (by the example of strontium and cesium radionuclides) 27

K. A. BOLDYREV1, S. S. UTKIN2, S. V. KAZAKOV3

1 Boldyrev Kirill Aleksandrovich, Ph. D. (Engineering), Research Worker, Nuclear Safety Institute, Russian Academy of Sciences

52 Bol’shaia Tul’skaia St., 115191, Moscow, Russian Federation, tel.: +7 (495) 955-23-35, e-mail: kaboldyrev@ibrae.ac.ru

2 Utkin Sergei Sergeevich, Ph. D. (Engineering), Department Chief, Nuclear Safety Institute, Russian Academy of Sciences

52 Bol’shaia Tul’skaia St., 115191, Moscow, Russian Federation, tel.: +7 (495) 955-23-77, e-mail: uss@ibrae.ac.ru

3 Kazakov Sergei Viktorovich, Ph. D. (Engineering), Senior Research Worker, Global Climate and Ecology Institute, RosGidroMet and Russian Academy of Sciences

20B Glebovskaia St., 107258, Moscow, Russian Federation, tel.: +7 (499) 169-01-41, e-mail: S_kazakov@inbox.ru

The basic provisions of the developed methodological approach to predicting the coefficient of Kd radionuclide distribution on the material of bottom sediments and suspended substances in water bodies are presented. As a tool for predicting Kd coefficient of metals under the conditions of the changing chemical composition of water the use of geochemical modelling was suggested. The basic approaches, models and results of predicting the values of Kd coefficient of metals in a reference water body (a surface water reservoir – radioactive waste storage) characterized by high pollution with 90Sr strontium and 137Cs radionuclides as well as by the intensive decrease of water mineralization are laid out. During the research the specific features of hydrochemical and hydrodynamic behavior of metals in the water body system as well as the methods of geochemical modeling were studied in detail. The geochemical thermodynamic model of metal sorption (by the example of 90Sr and 137Сs) by the suspended matter and bottom sediment layer in the model water reservoir was developed that could be considered as a basis for modeling different scenarios of the evolution of real polluted water reservoirs. The values of Kd coefficient for 90Sr and 137Сs at decreasing water mineralization were calculated. It was determined that Kd value is increasing while water mineralization is decreasing. The model can be used for predicting the rate of metal elimination from polluted water bodies in the process of self-purification.

Key words: surface water reservoir, bottom sediments, sorption of heavy metals, radionuclide distribution coefficient, geochemical modeling, self-purification of a water reservoir.

REFERENCES

- Mueller B., Duffek A. Similar adsorption parameters for trace metals with different aquatic particles. Aquatic Geochemistry, 2001, v. 7, pp. 107–126.

- Pivovarov S. A. Fiziko-khimicheskoe modelirovanie povedeniia tiazhelykh metallov (Si, Zn, Cd) v prirodnykh vodakh: kompleksy v rastvore, adsorbtsiia, ionnyi obmen, transportnye iavleniia [Physical and chemical modeling of heavy metals behavior (Cu, Zn, Cd) in natural water: complexes in solutions, adsorption, ion exchange, transport phenomena. Ph. D. thesis in Chemistry. Moscow, MGU Publ., 2003, 137 p.].

- Lipatnikova O. A. Eksperimental'noe issledovanie i termodinamicheskoe modelirovanie form nakhozhdeniia mikroelementov v donnykh otlozheniiakh Ivan'kovskogo vodokhranilishcha [Experimental study and thermodynamic modeling of microelement forms present in bottom sediments of the Ivan’kovskoe water reservoir. Ph. D. thesis in Geology and Mineralogy. Moscow, 2011, 130 p.].

- Lipatnikova O. A. [Thermodynamic model of heavy metal behavior in eutrophicated deposits]. Proceedings of «Current status of geosciences» International Conference dedicated to the memory of Viktor Efimovich Khain. Moscow, February 1–4, 2011. (In Russian).

- Sokolova O. V., Shestakova T. V., Grichuk D. V., Shvarov Iu. V. [Thermodynamic modeling of heavy metal forms in the «water-bottom sediments» system at auto pollution]. Vestnik Moskovskogo Universiteta. Ser. 4. Geologiia, 2006, no. 3, pp. 36–45. (In Russian).

- Sokolova O. V. Eksperimental'noe issledovanie i termodinamicheskoe modelirovanie migratsii tiazhelykh metallov v sisteme «voda – donnye otlozheniia» v zone antropogennogo vozdeistviia [Experimental study and thermodynamic modeling of heavy metal migration in the «water-bottom sediments» system in the anthropogenic impact zone. Ph. D. thesis in Engineering science. Moscow, 2008, 188 p.].

- Bezzaponnaia O. V. Prognoz soderzhaniia soedinenii tiazhelykh metallov v poverkhnostnykh vodnykh ob"ektakh [Prediction of the concentration of heavy metal compounds in surface water bodies. Ph. D. thesis in Engineering science. Ekaterinburg, 2004, 162 p.].

- Sozdanie metodologii eksperimental'nogo i naturnogo izucheniia protsessov akkumuliatsii i vynosa tiazhelykh metallov v donnykh otlozheniiakh vodokhranilishch i ozer [Development of the methodology of experimental and full-scale studies of the processes of heavy metal accumulation and release in bottom sediments of water reservoirs and lakes. R&D report. Moscow, «DAR/VODGEO» CJSC Publ., 2014].

- Skotnikova O. G., Fesenko S. V. [Mathematical model of radionuclides migration in a standing water body]. Radiatsionnaia Bezopasnost' i Zashchita AES, Moscow, Energoizdat Publ., 1986, Issue. 10, pp. 117–122. (In Russian).

- Hakanson L., Brittain J. E., Monte L, Heling R., Bergstrom U. Modelling of radiocaesium in lakes – the VAMP model. Environmental Radioactivity, 1996, v. 33, no. 3, pp. 255–308.

- Heling R. LAKECO, the ecological consequences of an accidental release of radionuclides on a lake ecosystem and it integration into the real-time-on-line decision support system for the off-site Emergency Management following a Nuclear Accident (RODOS). KEMA report Nr. 40352-NUC 93-5852, 1994.

- Kroot M. P. J. M. Behavior of radionuclides. Part of the issue «Description of the modifications of the model IMPAQT». T740.01. Delft Hydraulics Delft, 1992.

- Monte L., Fratarcangeli F., Pompei S., Quaggia S. Andres G. A predictive model for the behaviour of dissolved radioactive substances in stratified lakes. Environmental Radioactivity, 1991, no. 13, pp. 297–308.

- Kazakov S. V., Utkin S. S. Podkhody i printsipy radiatsionnoi zashchity vodnykh ob"ektov [Approaches and principles of radiological protection of water bodies. Moscow, Nauka Publ., 2008, 318 p.].

- Tessier A., Campbell H. G., Bisson M. Sequential extraction procedure for the speciationof particulate trace metals. Analytical Chemistry, 1979, v. 51, no. 7, pp. 844–851.

- Syrovetnik K. Long-term metal retention processes in a peat bog: field studies, data and modeling. Doctoral thesis. Stockholm, Sweden, Department of Chemical Engineering and Technology Royal Institute of Technology, 2005, 86 p.

- Bradbury M. H., Baeyens B. A generalised sorption model for the concentration dependent uptake of caesium by argillaceous rocks. Journal of Contaminant Hydrology, 2000, no. 42, pp. 141–163.

- Brouwer E., Baeyens B., Maes A., Cremers A. Cesium and rubidium ion equilibria in illite clay. Journal of Physical Chemistry, 1983, no. 87, pp. 1213–1219.

- Generic models for use in assessing the impact of discharges of radioactive substances to the environment. Safety Report Series no. 19. Vienna, IAEA, 2001.

- Metodika razrabotki normativov dopustimykh sbrosov radioaktivnykh veshchestv v vodnye ob"ekty (DS-2010) [Methods of developing maximum permissible limits of radioactive substances discharge into water bodies (DS-2010). Moscow, Federal'naia Sluzhba po Ekologicheskomu, Tekhnologicheskomu i Atomnomu Nadzoru Publ., 2010].

УДК 628.35:316.12

Кофман В. Я. Извлечение азота и фосфора в виде струвита из сточных вод с высоким содержанием биогенов 34

В. Я. КОФМАН*

* Кофман Владимир Яковлевич, старший научный сотрудник, Всероссийский институт научной и технической информации Российской академии наук

125190, Россия, Москва, ул. Усиевича, 20, тел.: (499) 155-43-08, e-mail: kofman16@mail.ru

Мировые запасы фосфатных руд насчитывают порядка 7000 млн тонн, из числа которых 40 млн тонн ежегодно используют для производства удобрений. По оценкам экспертов, этот ресурс будет исчерпан к 2090 г. В этой связи поиск альтернативных возобновляемых источников фосфора является весьма актуальной задачей, поэтому уделяется большое внимание разработке экономичных и технически доступных способов извлечения фосфора, а также азота из разного вида сточных вод. Одним из способов является осаждение данных биогенов в виде струвита (MgNH4PO46H2O). Исследования по получению струвита из сточных вод и его применению в качестве удобрения ведутся во многих странах мира. Разработаны технологические схемы извлечения струвита из муниципальных сточных вод, сталеплавильных и кожевенных заводов, процессов коксования, фильтрата полигонов хранения твердых бытовых отходов, животноводческих и молочных ферм, а также черных сточных вод при их децентрализованной обработке. Изучение использования струвита в качестве удобрения продолжается уже на протяжении почти 20 лет. В целом не оспаривается перспективность инвестиций в производство этого продукта для агросектора. Практика производства и поставки струвита в качестве удобрения уже достаточно развита в Японии. Применение струвита не приводит к увеличению содержания тяжелых металлов в сельскохозяйственных культурах. Быстрорастворимые удобрения неэффективны на пастбищах или лесных угодьях, где внесение удобрений практикуется один раз в несколько лет. В таких условиях более целесообразно применение медленно растворяющихся удобрений, к которым относится и струвит.

Ключевые слова: сточные воды, азот, фосфор, струвит, фильтрат полигонов твердых бытовых отходов, фосфорные удобрения.

Kofman V. Ia. Extraction of nitrogen and phosphorus in the form of struvite from wastewater with high nutrients concentration 34

V. Ia. KOFMAN*

* Kofman Vladimir Iakovlevich, Senior Research Fellow, All-Russian Institute of Scientific and Technical Information, Russian Academy of Sciences

20 Usuevicha St., 125190, Moscow, Russian Federation, tel.: +7 (499) 155-43-08, e-mail: kofman16@mail.ru

The world reserves of phosphate ore total about 7000 mln tons among them 40 mln tons are used annually in fertilizer production. It is estimated that this resource will be depleted by 2090. In this connection exploring alternative renewable phosphorus sources is a critical task; therefore much attention is paid to developing economical and technically available methods of phosphorus extraction together with nitrogen from different wastewater types; one of the methods being nutrients precipitation  in the form of struvite (MgNH4PO46H2O). Studies of struvite production from wastewater and its use as a fertilizer have been carried out in many countries of the world. Process schemes of struvite extraction from municipal wastewater, steel making and tanning wastes, coking process effluents, solid waste landfill filtrate, livestock and dairy farm wastewater as well as from black wastewater in the process of its decentralized processing have been developed. Studying the use of struvite as a fertilizer has been carried out almost 20 years already. Generally the prospects of investing in the production of this product for the agricultural sector are not disputed. The practice of production and supply of struvite as a fertilizer has been fairly developed in Japan. The application of struvite does not result in the increase of heavy metal concentration in farm crops. Fast dissolving fertilizers are not efficient when used on pastures or in forests where fertilization is practiced every few years. In this context the use of slowly dissolving fertilizers including struvite is more reasonable.

Key wards: wastewater, nitrogen, phosphorus, struvite, filtrate of solid waste landfills, phosphatic fertilizers.

REFERENCES

- Taylor A., Frazier A., Gurney E. Solubility products of magnesium ammonium phosphate. Transactions of Faraday Society, 1963, no. 59, pp. 1580–1584.

- Kumar R., Pal P. Assessing the feasibility of N and P recjvery by struvite precipitation from nutrient-rich wastewater: A review. Environmental Science and Pollution Research, 2015, v. 22, pp. 17453–17464.

- Kumar R., Pal P. Response surface-optimized Fenton's pretreatment for chemical precipitation of struvite and recycling of water through downstream nanofiltration. Chemical Engineering Journal, 2012, no. 210, pp. 33–44.

- Liu Y. H., Kwag J. H., Kim J. H., Ra C. S. Recovery of nitrogen and phosphorus by struvite crystalization from swine wastewater. Desalination, 2011, v. 277, no. 1–3, pp. 364–369.

- Suzuki K., Tanaka Y., Kuroda K., et al. Removal and recovery of phosphorus from swine wastewater by demonstration crystallization reactor and struvite accumulation device. Bioresource Technology, 2007, v. 98, pp. 1573–1578.

- Yilmazel Y. D., Demirer G. N. Removal and recovery of nutrients as struvite from anaerobic digestion residues from poultry manure. Environmental Technology, 2011, v. 32, pp. 783–794.

- Larsen T. A., Peters I., Alder A., et al. Reengineering the toilet for sustainable wastewater management. Environmental Science and Technology, 2001, v. 35, pp. 192A–197A.

- Kaikake K., Sekito T., Dote Y. Phosphate recovery from phosphorus-reach solution obtained from chicken manure inciniration ash. Waste Management, 2009, no. 29, pp. 1084–1088.

- Dichtl N., Rogge S., Bauerfeld K. Novel strategies in sewage sludge treatment. Clean, 2007, v. 35, pp. 473–479.

- Kabdasli I., Safak A., Tunay O. Bench-scale evaluation of treatment schemes incorporating struvite precipitation for young landfill leachate. Waste Management, 2008, v. 28, pp. 2386–2392.

- Di Laconi C., Rossetti S., Lopez A., Ried A. Effective treatment of stabilized municipal landfill leachate. Chemical Engineering Journal, 2011, v. 168, pp. 1085–1092.

- Kumar R., Pal P. A membrane-integrated advanced scheme for treatment of industrial wastewater: dynamic modelling towards scale up. Chemosphere, 2013, v. 92, pp. 1375–1382.

- Lahav O., Telzhensky M., Zewuhn A., et al. Struvite recovery from municipal-wastewater sludge centrifuge supernatant using seawater NF concentrate as a cheap Mg (II) source. Separation and Purification Technology, 2013, v. 108, pp. 103–110.

- Ueno Y., Fujii M. Three years' experience of operating and selling recovered struvite from full-scale plant. Environmental Technology, 2001, v. 22, pp. 1373–1381.

- El Diwani G., El Rafie S., El Ibiari N. N., El-Alia H. I. Recovery of ammonia nitrogen from industrial wastewater as struvite slow releasing fertilizer. Desalination, 2007, v. 214, pp. 200–214.

- Ryu H. D., Lim C. S., Kang M. K., et al. Evaluation of struvite obtained from semiconductor wastewater as a fertilizer in cultivating Chinese cabbage. Journal of Hazardous Materials, 2012, v. 221, pp. 248–255.

- Uysal A., Yilmazel Y. D., Demirer G. N. The determination of fertilizer quality of the formed struvite from effluent of a sewage sludge anaerobic digester. Journal of Hazardous Materials, 2010, v. 181, pp. 248–254.

- Gaterell M. R., Gay R., Wilson R., et al. An economic and environmental evaluation of the opportunities for substituting phosphorus recovered from wastewater treatment works in existing UK fertilizer markets. Environmental Technology, 2000, v. 21, pp. 1067–1084.

- Shu L., Schneider P., Jegatheesan V., Johnson J. An economic evaluation of phosphorus recovery as struvite from digester supernatant. Bioresource Technology, 2006, no. 97, pp. 2211–2216.

- Munch E. V., Barr K. Controlled struvite crystallisation for removing phosphorus from anaerobic digester side streams. Water Research, 2001, v. 35, pp. 151–159.

- НАИЛУЧШИЕ ДОСТУПНЫЕ ТЕХНОЛОГИИ

- УДК 628.3:504.06

Данилович Д. А., Скобелев Д. О., Щелчков К. А. Определение технологических показателей качества очистки городских сточных вод, соответствующих применению наилучших доступных технологий 40

Д. А. ДАНИЛОВИЧ1, Д. О. СКОБЕЛЕВ2, К. А. ЩЕЛЧКОВ3

1 Данилович Дмитрий Александрович, кандидат технических наук, руководитель Центра технической политики и модернизации, Ассоциация ЖКХ «Развитие»; эксперт-директор журнала «НДТ», координатор технической рабочей группы ТРГ10 Бюро НДТ «Очистка сточных вод с использованием централизованных систем водоотведения поселений, городских округов»

119435, Россия, Москва, Малая Пироговская ул., 13, стр. 1, БЦ «П13», тел.: (499) 558-38-32, e-mail: da_danilovich@mail.ru

2 Скобелев Дмитрий Олегович, руководитель Бюро НДТ

117418, Россия, Москва, Нахимовский проспект, 31, корп. 2, тел.:  (495) 543-72-62, e-mail: buro.ndt@vniismt.ru

3 Щелчков Кирилл Александрович, руководитель секретариата Бюро НДТ

117418, Россия, Москва, Нахимовский проспект, 31, корп. 2, тел.:  (495) 543-72-62, e-mail: buro.ndt@vniismt.ru

В 2015 г. был разработан информационно-технический справочник по наилучшим доступным технологиям «Очистка сточных вод с использованием централизованных систем водоотведения поселений, городских округов» (ИТС 10-2015). Документ создан впервые в России и не имеет аналогов в Европейском союзе. Одной из основных сложностей при создании ИТС 10-2015 была разработка методологии определения технологических показателей наилучших доступных технологий. Проведен анализ этой методологии в сравнении с действующими в ЕС перспективными подходами к установлению технологических показателей НДТ, включавший в себя: отбор и разделение объектов на группы; сбор данных об эмиссии в окружающую среду; выбор загрязняющих веществ; анализ доступной информации о сбросах; дифференцированное определение уровня эмиссии, связанного с использованием наилучших доступных технологий. Технологические показатели для загрязняющих веществ, удаляемых при традиционной биологической очистке (взвешенные вещества, БПК5, ХПК), определены в результате анализа данных, полученных при анкетировании объектов отрасли. Установление технологических показателей по соединениям азота и фосфора в значительной степени основано на экспертных оценках в связи с малым количеством объектов, работающих по НДТ, и отличиями используемых на них технологий. В целом порядок определения технологических показателей НДТ в полной мере соответствовал практикам, в настоящее время применяемым для этой цели в ЕС. В дальнейшем при подготовке к пересмотру ИТС 10-2015 рекомендуется провести мониторинг и технологический аудит референц-объектов НДТ, а также перспективных технологий.

Ключевые слова: очистка сточных вод, наилучшие доступные технологии, технологические показатели, уровень эмиссии, референц-объект.

СПИСОК ЛИТЕРАТУРЫ

- Данилович Д. А., Эпов А. Н., Канунникова М. А. Применение основных технологий на сооружениях очистки сточных вод: анализ данных в целях технологического нормирования // Наилучшие доступные технологии водоснабжения и водоотведения. 2015. № 3–4. С. 18–28.

- Данилович Д. А. НДТ очистки сточных вод поселений: концепция информационно-технического справочника // Наилучшие доступные технологии водоснабжения и водоотведения. 2015. № 3–4. С. 13–17.

- Данилович Д. А. Подготовлен проект Справочника НДТ по очистке сточных вод поселений // Наилучшие доступные технологии водоснабжения и водоотведения. 2015. № 5. С. 22–32.

- Данилович Д. А. Справочник по наилучшим доступным технологиям по очистке сточных вод поселений утвержден. Что дальше? // Наилучшие доступные технологии водоснабжения и водоотведения. 2016. № 1. С. 16–21.

- Polders C., Van den Abeele L., Derden A., Huybrechts D. Methodology for determining emission levels associated with the best available techniques for industrial waste water // Journal of Cleaner Production. 2012. № 29–30. P. 113–121.

- Cunningham D. IPPC, BAT, and voluntary agreements // Journal of Hazardous Materials. 2000. № 78. P. 105–121.

- Dijkmans R. Methodology for selection of best available techniques (BAT) at the sector level // Journal of Cleaner Production. 2000. № 8. P. 11–21.

- Geldermann J., Rentz O. The reference installation approach for the technoeconomic assessment of emission abatement options and the determination of BAT according to the IPPC-directive // Journal of Cleaner Production. 2004. № 12. P. 389–402.

- Silvo K., Melane M., Honkasalo A., Ruonala S., Lindström M. Integrated pollution prevention and control: the Finnish approach // Resources, Conservation and Recycling. 2002. № 35. P. 45–60.

- Barros M. C., Magán A., Valiño S., Bello B. M., Casares J. J., Blanco J. M. Identification of best available techniques in the seafood industry: a case study // Journal of Cleaner Production. 2009. № 17. P. 391–399.

- Bréchet T., Tulkens H. Beyond BAT: selecting optimal combinations of available techniques, with an example from the limestone industry // Journal of Environmental Management. 2009. № 90. P. 1790–1801.

- Kocabas A. M., Yukseler H., Dilek F. B., Yetis U. Adoption of European Union’s IPPC Directive to a textile mill: analysis of water and energy consumption // Journal of Environmental Management. 2009. № 91. P. 102–113.

- Schollenberger H., Treitz M., Geldermann J. Adapting the European approach of best available techniques: case studies from Chile and China // Journal of Cleaner Production. 2008. № 16. P. 1856–1864.

BEST AVAILABLE TECHNIQUES

Danilovich D. A., Skobelev D. O., Shchelchkov K. A. Determination of technological indicators of municipal wastewater treatment quality corresponding to the use of the best available techniques 40

D. A. DANILOVICH1, D. O. SKOBELEV2, K. A. SHCHELCHKOV3

1 Danilovich Dmitrii Aleksandrovich, Ph. D. (Engineering), Head of Technical Policy and Upgrade Centre, «Razvitie» Public Utilities Association; Expert-Director of «NDT» Journal, Coordinator of the Technical Working Group TWG 10 «Wastewater Treatment in Public Wastewater Disposal Systems of Communities, Urban Districts», NDT Bureau

Build. 1, 13 «P13» Business Centre, Malaia Pirogovskaia St., 119435, Moscow, Russian Federation, tel.: +7 (499) 558-38-32, e-mail: da_danilovich@mail.ru

2 Skobelev Dmitrii Olegovich, Head of NDT Bureau

Build. 2, 31 Nakhimovskii Ave., 117418, Moscow, Russian Federation, tel.: +7 (495) 543-72-62, e-mail: buro.ndt@vniismt.ru

3 Shchelchkov Kirill Aleksandrovich, Secretariat Head, NDT Bureau

Build. 2, 31 Nakhimovskii Ave., 117418, Moscow, Russian Federation, tel.: +7 (495) 543-72-62, e-mail: buro.ndt@vniismt.ru

In 2015 «Wastewater treatment in public wastewater disposal systems of communities, urban districts» information and technical guidance was compiled (ITG 10-2105). The document has been developed for the first time in Russia and has no parallels in European Union. One of the main problems in compiling ITG 10-2105 was developing the methodology of determining the technological indicators of the best available techniques. The analysis of this methodology was carried out in comparison with the existing in EC perspective approaches to the determination of the technological indicators including: projects selection and split up into groups; acquisition of data on the emissions into the atmosphere; selection of pollutants; analysis of the available information on discharges; differentiated determination of the emission level in relation to the best available techniques. The technological indicators of pollutants removed in the traditional biological treatment processes (suspended solids, BOD5, COD) were determined in the process of analyzing the data obtained by questioning the branch projects. Determination of the technological indicators for nitrogen and phosphorus compounds is based greatly on the expert assessment due to the small number of projects using BAT and due to the differences in the technologies applied. Generally the method for determining the technological indicators of BAT to the full extent corresponded to the practices currently used for this purpose in EC. In future during the preparation for ITG 10-2015 revision it is recommended to carry out monitoring and technological audit of BAT reference-projects as well as of the perspective technologies.

Key words: wastewater treatment, best available techniques, technological indicators, emission level, reference-project.

REFERENCES

- Danilovich D. A., Epov A. N., Kanunnikova M. A. [The use of basic technologies at the wastewater treatment facilities: data analysis for the technological regulations]. Nailuchshie Dostupnye Tekhnologii Vodosnabzheniia i Vodootvedeniia, 2015, no. 3–4, pp. 18–28. (In Russian).

- Danilovich D. A. [BAT of community wastewater treatment: the concept of the information technical guidance]. Nailuchshie Dostupnye Tekhnologii Vodosnabzheniia i Vodootvedeniia, 2015, no. 3–4, pp. 13–17. (In Russian).

- Danilovich D. A. [The draft BAT Guidance on community wastewater treatment has been designed]. Nailuchshie Dostupnye Tekhnologii Vodosnabzheniia i Vodootvedeniia, 2015, no. 5, pp. 22–32. (In Russian).

- Danilovich D. A. [The guidance on the best available techniques of community wastewater treatment approved. What comes next?]. Nailuchshie Dostupnye Tekhnologii Vodosnabzheniia i Vodootvedeniia, 2016, no. 1, pp. 16–21. (In Russian).

- Polders C., Van den Abeele L., Derden A., Huybrechts D. Methodology for determining emission levels associated with the best available techniques for industrial waste water. Journal of Cleaner Production, 2012, no. 29–30, pp. 113–121.

- Cunningham D. IPPC, BAT, and voluntary agreements. Journal of Hazardous Materials, 2000, no. 78, pp. 105–121.

- Dijkmans R. Methodology for selection of best available techniques (BAT) at the sector level. Journal of Cleaner Production, 2000, no. 8, pp. 11–21.

- Geldermann J., Rentz O. The reference installation approach for the technoeconomic assessment of emission abatement options and the determination of BAT according to the IPPC-directive. Journal of Cleaner Production, 2004, no. 12, pp. 389–402.

- Silvo K., Melane M., Honkasalo A., Ruonala S., Lindström M. Integrated pollution prevention and control: the Finnish approach. Resources, Conservation and Recycling, 2002, no. 35, pp. 45–60.

- Barros M. C., Magán A., Valiño S., Bello B. M., Casares J. J., Blanco J. M. Identification of best available techniques in the seafood industry: a case study. Journal of Cleaner Production, 2009, no. 17, pp. 391–399.

- Bréchet T., Tulkens H. Beyond BAT: selecting optimal combinations of available techniques, with an example from the limestone industry. Journal of Environmental Management, 2009, no. 90, pp. 1790–1801.

- Kocabas A. M., Yukseler H., Dilek F. B., Yetis U. Adoption of European Union’s IPPC Directive to a textile mill: analysis of water and energy consumption. Journal of Environmental Management, 2009, no. 91, pp. 102–113.

- Schollenberger H., Treitz M., Geldermann J. Adapting the European approach of best available techniques: case studies from Chile and China. Journal of Cleaner Production, 2008, no. 16, pp. 1856–1864.

НОВЫЕ ТЕХНОЛОГИИ И ОБОРУДОВАНИЕ

УДК 543.6:628.316

Белов С. Г., Наумчик Г. О. Разработка инструментального метода определения интенсивности окраски сточных вод текстильных предприятий 53

С. Г. БЕЛОВ1, Г. О. НАУМЧИК2

1 Белов Сергей Григорьевич, кандидат технических наук, доцент кафедры «Водоснабжение, водоотведение и охрана водных ресурсов», Брестский государственный технический университет

224017, Республика Беларусь, г. Брест, Московская ул., 267, тел.: +375 33 672-20-40, e-mail: vvit@bstu.by

2 Наумчик Григорий Остапович, ассистент кафедры «Водоснабжение, водоотведение и охрана водных ресурсов», Брестский государственный технический университет

224017, Республика Беларусь, г. Брест, Московская ул., 267, тел.: +375 29 221-96-07, e-mail: vvit@bstu.by

Разработан новый инструментальный метод определения интенсивности окраски сточных вод с использованием сканирующего спектрофотометра, который учитывает особенности человеческого зрения. Это позволяет обеспечить соответствие с традиционным методом определения интенсивности окраски по степени разбавления (результаты определений различаются незначительно). При этом для выполнения анализа достаточно снять спектр окрашенного образца и обработать его с помощью специально разработанной компьютерной программы «Окраска-мониторинг». Дана подробная методика выполнения анализа. Разработанный метод в отличие от традиционного позволяет значительно сократить время анализа, для его проведения не требуется яркое освещение, а также специальные цилиндры из бесцветного стекла, многократно сокращается расход дистиллированной воды. Главным его преимуществом является полное исключение субъективных факторов при определении данного показателя. Метод отличается высокой воспроизводимостью, что позволит значительно повысить достоверность определения интенсивности окраски. Метод определения интенсивности окраски на основе параметров спектров поглощения может применяться на предприятиях, осуществляющих контроль данного показателя (водоканалы, комитеты природных ресурсов и охраны окружающей среды, промышленные предприятия, экологические организации и т. д.).

Ключевые слова: сточные воды текстильных предприятий, интенсивность окраски стоков, сканирующий спектрофотометр, степень разбавления раствора красителя, спектр поглощения, критерий неравномерности поглощения.

СПИСОК ЛИТЕРАТУРЫ

- Лурье Ю. Ю. Аналитическая химия промышленных сточных вод. – М.: Химия, 1984. 447 с.

- Житенев Б. Н., Белов С. Г., Наумчик Г. О. Применение озона для снижения окраски сточных вод предприятий легкой промышленности на примере ОАО «Брестский чулочный комбинат» / Проблемы водоснабжения, водоотведения и энергосбережения в западном регионе Республики Беларусь: Материалы Международной научно-технической конференции, посвященной 65-летию победы в Великой Отечественной войне, Брест, 22–23 апреля 2010 г. – Брест, БрГТУ, 2010. С. 284–289.

- Берштейн И. Я., Каминский Ю. Л. Спектрометрический анализ в органической химии. – Л.: Химия, 1986. 199 с.

- Степанов Б. И. Введение в химию и технологию органических красителей. – М.: Химия, 1977. 488 с.

- Самойлов В. Ф., Хромой Б. П. Основы цветного телевидения. – М.: Радио и связь, 1982. 160 с.

ADVANCED TECHNOLOGIES AND EQUIPMENT

Belov S. G., Naumchik G. O. Development of an instrumental method of determining the color intensity of textile wastewater 53

S. G. BELOV1, G. O. NAUMCHIK2

1 Belov Sergei Grigor'evich, Ph. D. (Engineering), Assistant Professor, «Water supply, Wastewater Disposal and Water Resources Protection» Department, Brest State Technical University

267 Moskovskaia St., 224017, Brest, Republic of Belarus, tel.: +375 33 672-20-40, e-mail: vvit@bstu.by

2 Naumchik Grigorii Ostapovich, Teaching Assistant, «Water Supply, Wastewater Disposal and Water Resources Protection» Department, Brest State Technical University

267 Moskovskaia St., 224017, Brest, Republic of Belarus, tel.: +375 29 221-96-07, e-mail: vvit@bstu.by

A new instrumental method of determining the color intensity of textile wastewater with the use of a scanning spectrophotometer that accounts for the specificity of human vision has been developed. This ensures the conformability with the traditional method of determining color intensity by dilution rate (the determinations do not differ much). However, to carry out the analysis collecting spectral data on the colored sample and processing it with the help of specially developed «Coloration-monitoring» software will be enough. The detailed procedure of the analysis is presented. Compared to the traditional method the developed method provides for reducing significantly the analysis time; no bright lighting or special cylinders made of flint glass is required; the consumption of distilled water is also reduced. The key advantage of the method is the complete elimination of judgmental factor in determining this parameter. The method is characterized by high reproducibility that will improve the validity of the color intensity determinations. The method of determining color intensity based on the absorbance spectrum parameters can be used by the enterprises that monitor the given indicator (water and wastewater services providers, committees for natural resources and environmental protection, industrial enterprises, environmental organizations etc.).

Key words: textile wastewater, wastewater color intensity, scanning spectrophotometer, dye bath dilution rate, absorbance spectrum, absorption irregularity criterion.

REFERENCES

- Lur'e Iu. Iu. Analiticheskaia khimiia promyshlennykh stochnykh vod [Analytical chemistry of industrial wastewater. Moscow, Khimiia Publ., 1984, 447 p.].

- Zhitenev B. N., Belov S. G., Naumchik G. O. [The use of ozone for decoloring wastewater of consumer goods industries by the example of «Brest Hosiery Factory» OJSC]. Problems of water supply, wastewater disposal and energy saving in the West Region of the Republic of Belarus: Proceedings of International Scientific and Technical Conference dedicated to the 65th Anniversary of the Victory in the Great Patriotic War, Brest, April 22–23, 2010. Brest, BrGTU Publ., 2010, pp. 284–289. (In Russian).

- Bershtein I. Ia., Kaminskii Iu. L. Spektrometricheskii analiz v organicheskoi khimii [Spectrometric analysis in organic chemistry. Leningrad, Khimiia Publ., 1986, 199 p.].

- Stepanov B. I. Vvedenie v khimiiu i tekhnologiiu organicheskikh krasitelei [Introduction to the chemistry and technology of organic pigments. Moscow, Khimiia Publ., 1977, 488 p.].

- Samoilov V. F., Khromoi B. P. Osnovy tsvetnogo televideniia [Basics of the color television. Moscow, Radio i Sviaz’ Publ., 1982, 160 p.].

# ИНВЕСТИЦИОННЫЕ ПРОЕКТЫ

# УДК 628.1/.3:338.2

Нефедова Е. Д., Хямяляйнен М. М., Ковжаровская И. Б. Опыт оценки эффективности мероприятий инвестиционной программы предприятия водопроводно-канализационного хозяйства 66

Е. Д. НЕФЕДОВА1, М. М. ХЯМЯЛЯЙНЕН2, И. Б. КОВЖАРОВСКАЯ3

1 Нефедова Елена Дмитриевна, кандидат технических наук, руководитель проекта по межотраслевому балансовому регулированию, ГУП «Водоканал Санкт-Петербурга»

191015, Россия, Санкт-Петербург, Кавалергардская ул., 42, тел.: (812) 326-53-89, e-mail: Nefedova_ED@vodokanal.spb.ru

2 Хямяляйнен Михаил Михайлович, кандидат технических наук, директор департамента схем водоснабжения и водоотведения и гидравлического моделирования, ГУП «Водоканал Санкт-Петербурга»

191015, Россия, Санкт-Петербург, Кавалергардская ул., 42, тел.: (812) 326-53-37, e-mail: Hamalainen_MM@vodokanal.spb.ru

3 Ковжаровская Ирина Борисовна, руководитель технического обследования и оценки эффективности, ГУП «Водоканал Санкт-Петербурга»

191015, Россия, Санкт-Петербург, Кавалергардская ул., 42, тел.: (812) 326-53-37, e-mail: Kovzharovskaya_IB@vodokanal.spb.ru

Коммунальная инфраструктура водопроводно-канализационного хозяйства России требует обновления. Необходимость реконструкции и модернизации объектов централизованных систем водоснабжения и водоотведения с каждым годом значительно возрастает. На этом фоне каждое предприятие при формировании адресных перечней мероприятий программ развития сталкивается с проблемами обоснования и оценки эффективности мероприятий, расстановки приоритетов их реализации. Определенные в отраслевых нормативных правовых актах подходы не позволяют в достаточной степени провести оценку эффективности мероприятий по строительству, реконструкции и модернизации объектов централизованных систем водоснабжения и водоотведения и обосновать их необходимость. Учитывая специфику предприятия и имеющийся опыт в этой области, в ГУП «Водоканал Санкт-Петербурга» разработан дополнительный инструмент для оценки эффективности и обоснования мероприятий, подготовлены и введены в действие методические документы. Предложенный инструмент включает оценку экономической эффективности мероприятия, его вклада в достижение плановых показателей надежности, качества и энергетической эффективности, а также риска невыполнения мероприятия. Инструмент был апробирован при подготовке инвестиционной программы ГУП «Водоканал Санкт-Петербурга».

Ключевые слова: водоснабжение, водоотведение, инвестиционная программа, оценка эффективности, технологический аудит.

СПИСОК ЛИТЕРАТУРЫ

- gov.cap.ru/UserFiles/orgs/GrvId_21/statistika_po_otrasli.doc (дата обращения 25.01.2017).

- Гогина Е. С., Гуринович А. Д. Применение методики LCC для оценки эффективности инвестиционных проектов сооружений очистки сточных вод // Водоснабжение и санитарная техника. 2016. № 9. С. 36–41.

- Храменков С. В. Стратегия модернизации водопроводной сети. – М.: Стройиздат, 2005. 400 с.

# INVESTMENT PROJECTS

Nefedova E. D., Hiamialiainen M. M., Kovzharovskaia I. B. The experience of evaluating the efficiency of the actions of the investment program of a water supply and wastewater disposal enterprise 66

E. D. NEFEDOVA1, M. M. HIAMIALIAINEN2, I. B. KOVZHAROVSKAIA3

1 Nefedova Elena Dmitrievna, Ph. D. (Engineering), Manager of the Project on Intersectoral Balance Regulation, SUE «Vodokanal of St. Petersburg»

42 Kavalergardskaia St., 191015, St. Petersburg, Russian Federation, tel.: +7 (812) 326-53-89, e-mail: Nefedova_ED@vodokanal.spb.ru

2 Hiamialiainen Mikhail Mikhailovich, Ph. D. (Engineering), Director, Department for Water Supply and Wastewater Disposal Schemes and Hydraulic Simulation, SUE «Vodokanal of St. Petersburg»

42 Kavalergardskaia St., 191015, St. Petersburg, Russian Federation, tel.: +7 (812) 326-53-37, e-mail: Hamalainen_MM@vodokanal.spb.ru

3 Kovzharovskaia Irina Borisovna, Manager of Engineering Survey and Efficiency Evaluation, SUE «Vodokanal of St. Petersburg»

42 Kavalergardskaia St., 191015, St. Petersburg, Russian Federation, tel.: +7 (812) 326-53-37, e-mail: Kovzharovskaya_IB@vodokanal.spb.ru

The communal infrastructure of the water supply and wastewater disposal in Russia needs renovation. The need in reconstruction and upgrade of the public water supply and wastewater disposal systems increases year by year. In this context any enterprise in compiling targeted lists of development program actions faces the problems of substantiating and evaluating the efficiency of the actions, prioritizing their implementation. The approaches determined in the branch regulatory legal acts do not provide for evaluating adequately the efficiency of construction, renovation and upgrade of the public water and wastewater facilities and substantiating the need in them. Taking into account the specificity of the enterprise and the experience gained SUE «Vodokanal of St. Petersburg» developed an additional tool for evaluating the efficiency and substantiating the actions; prepared and put in force the guidance documents. The suggested tool includes the evaluation of the economic efficiency of an action, its contribution to the achievement of the planned reliability, quality and energy efficiency as well as of the implementation risk. The tool has been approbated in the process of developing the investment program of SUE «Vodokanal of St. Petersburg».

Key words: water supply, wastewater disposal, investment program, efficiency evaluation, technology audit.

REFERENCES

- gov.cap.ru/UserFiles/orgs/GrvId_21/statistika_po_otrasli.doc (accessed 25.01.2017).

- Gogina E. S., Gurinovich A. D. [The use of LLC method in evaluating the efficiency of investment projects of wastewater treatment facilities]. Vodosnabzhenie i Sanitarnaia Tekhnika, 2016, no. 9, pp. 36–41. (In Russian).

- Khramenkov S. V. Strategiia modernizatsii vodoprovodnoi seti [The strategy of water distribution network upgrade. Moscow, Stroiizdat Publ., 2005, 400 p.].