ОХРАНА ОКРУЖАЮЩЕЙ СРЕДЫ

УДК 504.06

Скрябин А. Ю., Поповьян Г. В., Тронь И. А. Изучение факторов, влияющих на интенсивное развитие микроводорослей в реке Дон 5

А. Ю. Скрябин1, Г. В. Поповьян2, И. А. ТРОНЬ3

1 Скрябин Александр Юрьевич, кандидат технических наук, первый заместитель главы администрации г. Ростова-на-Дону

344050, Россия, г. Ростов-на-Дону, ул. Б. Садовая, 47, тел.: (863) 244-12-22, e-mail: skryabin@rostov-gorod.ru

2 Поповьян Гаяне Владимировна, инженер-технолог, АО «Ростовводоканал»

344022, Россия, г. Ростов-на-Дону, ул. М. Горького, 293, тел.: (863) 283-02-96, e-mail: ogt@rvdk.ru

3 Тронь Игорь Анатольевич, директор технологического департамента, АО «Ростовводоканал»

344022, Россия, г. Ростов-на-Дону, ул. М. Горького, 293, тел.: (863) 283-02-96, e-mail: vdp1@rvdk.ru

Изучены факторы, способствующие росту и изменению видового состава микроводорослей в реке Дон. Доказано влияние фосфора на интенсивность развития фитопланктона. Проанализированы источники поступления биогенных элементов в водоемы. Приведены факторы антропогенного эвтрофирования водоема: строительство плотин, способствующее снижению скорости течения рек; разложение растительности на затопленных при строительстве водохранилищ и каналов землях; интенсивное развитие промышленного производства, сельского хозяйства, животноводства; увеличение объемов использования синтетических моющих средств в быту и промышленности. Предложена стратегия по борьбе с цветением водоисточника: подавление цветения в период его развития непосредственно в водоеме, применение биологических методов очистки (восстановление естественного биоценоза с помощью заселения в них определенных видов микроводорослей – Chlorella различных штаммов, а также некоторых сортов растительноядных рыб, например белого толстолобика; отказ от использования фосфорсодержащих моющих средств в быту и промышленности и замена их средствами, не содержащими фосфатов; усиление контроля за использованием азотных и фосфорных удобрений в сельском хозяйстве; строительство и поэтапная реконструкция или модернизация очистных сооружений, обеспечивающих глубокое извлечение соединений азота и фосфора.

Ключевые слова: водоснабжение, поверхностный водоисточник, микроводоросли, фитопланктон, азот, фосфор, геосмин, запах, одоранты.

СПИСОК ЛИТЕРАТУРЫ

- Лысов В. А., Скрябин А. Ю., Поповьян Г. В. Микрофиты в системе подготовки донской воды: Материалы юбилейной Международной научно-практической конференции «Строительство–2009». – Ростов-на-Дону, Ростовский государственный строительный университет, 2009. 334 с.

- Скрябин А. Ю., Поповьян Г. В. Изменение качества реки Дон: микроводоросли как фактор, влияющий на органолептические свойства воды / Технологии очистки воды «ТЕХНОВОД–2014»: Материалы VIII Международной научно-практической конференции. – Новочеркасск: Лик, 2014. 285 с.

- Скрябин А. Ю., Поповьян Г. В., Тронь И. А. Микроводоросли как фактор, влияющий на органолептические свойства реки Дон // Водоснабжение и санитарная техника. 2015. № 8. С. 38–42.

- Страхова Н. М., Кудрякова З. Н., Пирогов Н. О., Куцева Н. К. Определение органических соединений, придающих запах воде, методом газовой хроматографии с масс-селективным детектированием // Заводская лаборатория. Диагностика материалов. 2006. № 7. С. 3–7.

- Кульский Л. А., Сиренко Л. А., Шкавро З. Н. Фитопланктон и вода. – Киев: Наукова думка, 1986. 135 с.

- Жмур Н. С. Практика глубокого извлечения соединений азота и фосфора в процессе биологической очистки сточных вод в странах Европейского союза (по материалам отчета Европейской комиссии и докладов на Евро-Русском Форуме в Берлине, 2013 г.) // Водоснабжение и канализация. 2014. № 9–10. С. 4–28.

- Скрябин А. Ю., Поповьян Г. В., Тронь И. А. Изменение качества воды реки Дон. Изучение факторов, влияющих на интенсивное развитие микроводорослей / Технологии очистки воды «ТЕХНОВОД–2016»: Материалы IX Международной научно-практической конференции. – Новочеркасск: Лик, 2016. 300 с.

- Богданов Н. И. Биологическая реабилитация водоемов. – Пенза, РИО ПГСХА, 2008. 152 с.

ENVIRONMENTAL PROTECTION

Skriabin A. Iu., Popov'ian G. V., Tron' I. A. Study of the factors effecting the intensive growth of microalgae in the Don River 5

A. Iu. SKRIABIN1, G. V. POPOV'IAN2, I. A. TRON'3

1 Skriabin Aleksandr Iur'evich, Ph. D. (Engineering), First Deputy of Head of Rostov-on-Don Administration

47 B. Sadovaia St., 344050, Rostov-on-Don, Russian Federation, tel.: +7 (863) 244-12-22, e-mail: skryabin@rostov-gorod.ru

2 Popov'ian Gaiane Vladimirovna, Process Engineer, «Rostovvodokanal» JSC

293 M.Gor’kogo St., Rostov-on-Don, 344022, Russian Federation, tel.: +7 (863) 283-02-96, e-mail: ogt@rvdk.ru

3 Tron' Igor' Anatol'evich, Director of the Technological Department, «Rostovvodokanal» JSC

293 M. Gor’kogo St., Rostov-on-Don, 344022, Russian Federation, tel.: +7 (863) 283-02-96, e-mail: vdp1@rvdk.ru

The factors enhancing the growth and evolution of the microalgae species composition in the Don River were studied. The effect of phosphorus on the phytoplankton growth rate was proved. The sources of nutrients incoming to the water bodies were analyzed. The factors of anthropogenic eutrophication of water bodies are presented: dyke construction facilitating the reduction of the river flow rate; decay of the vegetation on the lands immersed during the construction of water reservoirs and canals; intensive development of industrial production, agricultural activities, cattle breeding; increase of household and industrial use of synthetic detergents. The strategy of controlling water source bloom is proposed: bloom suppression during its development directly in the water body; the use of biological treatment methods (rehabilitation of the natural biocenosis by seeding certain microalgae species – different Chlorella strains and some species of herbivorous fish, e.g. silver carp; elimination of household and industrial use of phosphorus-containing detergents and their replacement with phosphate free detergents; strengthening of control of nitrogen and phosphorus fertilizer use in agriculture; construction and stepwise reconstruction or upgrade of wastewater treatment facilities that provide for enhanced removal of nitrogen and phosphorous compounds.

Key words: water supply, surface water source, microalgae, phytoplankton, nitrogen, phosphorus, geosmin, odor, odorants.

REFERENCES

- Lysov V. A., Skriabin A. Iu., Popov'ian G. V. [Microphytes in the Don water purification system]. Proceedings of the Jubilee International Scientific-Practical Conference «Construction–2009». Rostov-on-Don, Rostovskii Gosudarstvennyi Stroitel'nyi Universitet Publ., 2009, 334 p. (In Russian).

- Skriabin A. Iu., Popov'ian G. V. [Changes in the Don River water quality: microalgae as a factor effecting the organoleptic water properties. «TEKHNOVOD–2014» water treatment technologies]. Proceedings of VIII International Scientific-Practical Conference. Novocherkassk, Lik Publ., 2014, 285 p. (In Russian).

- Skriabin A. Iu., Popov'ian G. V., Tron' I. A. [Microalgae as a factor effecting the organoleptic properties of the Don River water]. Vodosnabzhenie i Sanitarnaia Tekhnika, 2015, no. 8, pp. 38–42. (In Russian).

- Strakhova N. M., Kudriakova Z. N., Pirogov N. O., Kutseva N. K. [Determination of organic water flavoring compounds by gas chromatography in selected ion mode]. Zavodskaia Laboratoriia. Diagnostika Materialov, 2006, no. 7, pp. 3–7. (In Russian).

- Kul'skii L. A., Sirenko L. A., Shkavro Z. N. Fitoplankton i voda [Phytoplankton and water. Kiev, Naukova Dumka Publ., 1986, 135 p.].

- Zhmur N. S. [The practice of enhanced elimination of nitrogen and phosphorous compounds in biological wastewater treatment in EU countries (adapted from the report of the European Commission and presentations at the European-Russian Forum in Berlin, 2013)]. Vodosnabzhenie i Kanalizatsiia, 2014, no. 9–10, pp. 4–28. (In Russian).

- Skriabin A. Iu., Popov'ian G. V., Tron' I. A. [Changes in the Don River water quality. Study of the factors effecting the intensive growth of microalgae. «TEKHNOVOD–2014» water treatment technologies]. Proceedings of IX International Scientific-Practical Conference. Novocherkassk, Lik Publ., 2016, 300 p. (In Russian).

- Bogdanov N. I. Biologicheskaia reabilitatsiia vodoemov [Biological rehabilitation of water bodies. Penza, RIO PGSKHA Publ., 2008, 152 p.].

МЕЖДУНАРОДНЫЕ ВЫСТАВКИ И КОНФЕРЕНЦИИ

XII Международная научно-техническая конференция, посвященная памяти академика РАН Сергея Васильевича Яковлева 9

INTERNATIONAL EXHIBITIONS AND CONFERENCES

XII International Scientific and Practical Conference dedicated to the memory of Sergei Vasil’evich Iakovlev, Academician of RAS 9

ПИТЬЕВОЕ ВОДОСНАБЖЕНИЕ

УДК 628.16:62-278

Фесенко Л. Н., Вергунов А. И., Швецов В. Н., Морозова К. М., Костюков В. П. Биосорбционно-мембранный реактор с плоскими фильтрующими элементами: методика расчета 12

Л. Н. ФЕСЕНКО1, А. И. ВЕРГУНОВ2, В. Н. ШВЕЦОВ3, К. М. МОРОЗОВА4, В. П. КОСТЮКОВ5

## 1 Фесенко Лев Николаевич, доктор технических наук, профессор кафедры «Водное хозяйство, инженерные сети и защита окружающей среды», Южно-Российский государственный политехнический университет (НПИ) имени М. И. Платова

346428, Россия, Ростовская область, г. Новочеркасск, ул. Просвещения, 132, тел.: (8635) 26-05-46, e-mail: 65613@mail.ru

## 2 Вергунов Алексей Игоревич, инженер-технолог, ООО НПП «ЭКОФЕС»

## 346400, Россия, г. Новочеркасск Ростовской области, Баклановский проспект, 200В, тел.: (8635) 26-05-46, e-mail: vergunov_al@mail.ru

## 3 Швецов Валерий Николаевич, доктор технических наук, профессор, заведующий Экспертно-консультационным центром, АО «НИИ ВОДГЕО»

## 119435, Россия, Москва, Б. Саввинский пер., 9, стр. 1, тел.: (499) 245-96-33, e-mail: post@vstmag.ru

## 4 Морозова Ксения Михайловна, кандидат технических наук, старший научный сотрудник

## E-mail: morozova06@inbox.ru

## 5 Костюков Владимир Павлович, кандидат технических наук, заведующий кафедрой «Водное хозяйство, инженерные сети и защита окружающей среды», Южно-Российский государственный политехнический университет (НПИ) имени М. И. Платова

346428, Россия, Ростовская область, г. Новочеркасск, ул. Просвещения, 132, тел.: (8635) 25-53-33, e-mail: kostyukov_w@bk.ru

Приведены результаты исследований по очистке донской воды с применением биосорбционно-мембранного метода в биосорбционно-мембранном реакторе с плоскорамными фильтрующими элементами. Для этого на Новочеркасских водопроводных очистных сооружениях была смонтирована установка производительностью до 2,7 м3/сут с применением порошкообразного активированного угля. Удельный поток через мембрану составил до 17,6 л/(м2·ч). Результаты экспериментальных исследований показали высокую эффективность очистки природной воды в биосорбционно-мембранном реакторе до требований СанПиН. Эффективность снижения ХПК составляла в среднем 41,2%, цветности – 57,3%, перманганатной окисляемости – 33,3%. На протяжении всего эксперимента в пермеате отсутствовали взвешенные вещества. Полученные данные послужили основой для расчета удельной скорости окисления органических веществ, оцениваемых по ХПК и перманганатной окисляемости. Определены значения максимальной скорости окисления Vmax и константы Михаэлиса Km графическим методом двойных обратных величин. Получена зависимость удельной скорости окисления органических загрязнений по перманганатной окисляемости и ХПК от температуры. Определена температурная константа χ уравнения Вант-Гоффа по ХПК и перманганатной окисляемости для расчета удельной скорости окисления органических загрязнений в зимний (10–11 °С) и летний (20–22 °С) периоды года. Приведена методика расчета биосорбционно-мембранного реактора.

Ключевые слова: биосорбционно-мембранный реактор, очистка природной воды, порошкообразный активированный уголь, биосорбция, мембранное фильтрование, удельная скорость окисления.

СПИСОК ЛИТЕРАТУРЫ

- Швецов В. Н., Морозова К. М., Фесенко Л. Н., Скрябин А. Ю., Теремязева О. В. Биосорбционно-мембранная технология для предотвращения образования хлор- и броморганических соединений в воде р. Дон // Водоснабжение и санитарная техника. 2012. № 2. С. 7–13.

- Швецов В. Н., Морозова К. М., Фесенко Л. Н., Скрябин А. Ю., Вергунов А. И. Хлор- и броморганические соединения в питьевой воде: методы их удаления // Водоснабжение и санитарная техника. 2014. № 2. С. 30–35.

- Вергунов А. И. Очистка донской воды в биосорбционно-мембранном реакторе / «Яковлевские чтения»: Сборник докладов XI научно-технической конференции, посвященной памяти академика РАН Сергея Васильевича Яковлева, Москва, 16 марта 2016 г. – М.: Издательство АСВ, 2016. С. 36–38.

- Вергунов А. И., Швецов В. Н., Морозова К. М., Фесенко Л. Н. Биосорбционно-мембранный реактор с плоскорамными мембранами для очистки донской воды / Технология очистки воды «ТЕХНОВОД–2016»: Материалы IX Международной научно-практической конференции, г. Ростов-на-Дону, 5–7 октября 2016 г. – Новочеркасск: Лик, 2016. С. 146–151.

- Вергунов А. И. Биосорбционно-мембранная технология очистки донской воды // Вестник СГАСУ. Градостроительство и архитектура. 2016. № 2 (23). С. 23–26.

- Хенце М. Очистка сточных вод. Биологические и химические процессы. – М.: Мир, 2004. 480 с.

- Диксон М., Уэбб Э. Ферменты. Т. 1. – М.: Мир, 1982. 392 с.

- Смирнова И. И. Исследование процесса очистки природных вод биосорбционно-мембранным методом: Дисс. ... канд. техн. наук. – М., 2009. 113 с.

DRINKING WATER SUPPLY

Fesenko L. N., Vergunov A. I., Shvetsov V. N., Morozova K. M., Kostiukov V. P. Biosorption-membrane reactor with flat filtering elements: the calculation method 12

L. N. FESENKO1, A. I. VERGUNOV2, V. N. SHVETSOV3, K. M. MOROZOVA4, V. P. KOSTIUKOV5

## 1 Fesenko Lev Nikolaevich, Doctor of Engineering, Professor, «Water Industry, Utilities and Environmental Protection» Department, Platov South-Russian State Polytechnic University (NPI)

132 Prosveshcheniia St., 346428, Novocherkassk, Rostov Area, Russian Federation, tel.: +7 (8635) 26-05-46, e-mail: 65613@mail.ru

## 2 Vergunov Aleksei Igorevich, Process Engineer, «ECOFES» Research and Production Enterprise, LLC

## 200B Baklanovskii Ave., 346400, Novocherkassk, Rostov Area, Russian Federation, tel.: +7 (8635) 26-05-46, e-mail: vergunov_al@mail.ru

## 3 Shvetsov Valerii Nikolaevich, Doctor of Engineering, Professor, Head of Competency Centre, «NII VODGEO» JSC

## Build. 1, 9 B. Savvinskii Lane, 119435, Moscow, Russian Federation, tel.: +7 (499) 245-96-33, e-mail: post@vstmag.ru

4 Morozova Kseniia Mikhailovna, Ph. D. (Engineering), Senior Research Worker

E-mail: morozova06@inbox.ru

## 5 Kostiukov Vladimir Pavlovich, Ph. D. (Engineering), Head of «Water Industry, Utilities and Environmental Protection» Department, Platov South-Russian State Polytechnic University (NPI)

## 132 Prosveshcheniia St., 346428, Novocherkassk, Rostov Area, Russian Federation, tel.: +7 (8635) 25-53-33, e-mail: kostyukov_w@bk.ru

The results of studying the Don River water purification with the use of biosorption-membrane method in a biosorption-membrane reactor with flat-frame filtering elements are presented. An experimental plant with a capacity of less than 2.7 m3/day with addition of powdered activated carbon was constructed at the Novocherkassk water treatment facilities. The specific flow through the membrane was less than 17.6 l/(m2·h). The experimental results showed the high efficiency of natural water purification in a biosorption-membrane reactor that provided for meeting the requirements of Sanitary Rules and Regulations (SanPiN). The efficiency of COD reduction averaged to 41.2%, color – 57.3%, permanganate value – 33.3%. During the entire experiment no suspended solids were found in permeate. The obtained data were used as a basis for calculating the specific rate of organics oxidation estimated by COD and permanganate value. The maximum values of Vmax oxidation rate and Km Michaelis constant were determined by graphical method of double reciprocal. The permanganate value dependence of the specific organics oxidation rate and COD from temperature was obtained. Temperature constant χ of Van Hoff’s equation was determined by COD and permanganate value to calculate the specific rate of organic pollutants oxidation during winter (10–11 °С) and summer seasons (20–22 °С). The method of calculating a biosorption-membrane reactor is presented.

Key words: biosorption-membrane reactor, natural water purification, powdered activated carbon, biosorption, membrane filtration, specific oxidation rate.

REFERENCES

- Shvetsov V. N., Morozova K. M., Fesenko L. N., Skriabin A. Iu., Teremiazeva O. V. [Biosorption membrane technology for prevention of chlororganic and bromorganic compound formation in the Don River water]. Vodosnabzhenie i Sanitarnaia Tekhnika, 2012, no. 2, pp. 7–13. (In Russian).

- Shvetsov V. N., Morozova K. M., Fesenko L. N., Skriabin A. Iu., Vergunov A. I. [Chlororganic and bromorganic compounds in drinking water: methods of their removal]. Vodosnabzhenie i Sanitarnaia Tekhnika, 2014, no. 2, pp. 30–35. (In Russian).

- Vergunov A. I. [The Don River water purification in a biosorption membrane reactor. «Readings from Iakovlev»]. Book of reports of XI Scientific and Technical Conference dedicated to the memory of Sergei Vasil’evich Iakovlev, academician of RAS, Moscow, March 16, 2016. Moscow, ASV Publ., 2016, pp. 36–38. (In Russian).

- Vergunov A. I., Shvetsov V. N., Morozova K. M., Fesenko L. N. [Biosorption reactor with flat-frame membranes for the Don River water purification]. «TEKHNOVOD–2016» water treatment technologies. Proceedings of IX International Scientific-Practical Conference, Rostov-on-Don, October 5–7, 2016. Novocherkassk, Lik Publ., 2016, pp. 146–151. (In Russian).

- Vergunov A. I. [Biosorption-membrane technology of the Don River water purification]. Vestnik SGASU. Gradostroitel'stvo i Arkhitektura, 2016, no. 2 (23), pp. 23–26. (In Russian).

- Henze M. Ochistka stochnykh vod. Biologicheskie i khimicheskie protsessy [Wastewater treatment. Biological and chemical processes. Moscow, Mir Publ., 2004, 480 p.].

- Dixon M., Webb E. Fermenty [Enzymes. Moscow, Mir Publ., 1982, v. 1, 392 p.].

- Smirnova I. I. Issledovanie protsessa ochistki prirodnykh vod biosorbtsionno-membrannym metodom [Study of natural water purification by biosorption-membrane method. Ph. D. thesis in Engineering Science. Moscow, 2009, 113 p.].

ОЧИСТКА СТОЧНЫХ ВОД

УДК 628.31:661.5

Степанов С. В., Солкина О. С., Морозова К. М., Степанов А. С., Жукова М. А. Влияние химических промывок мембран на процессы биологической очистки 19

С. В. Степанов1, О. С. Солкина2, К. М. Морозова3, А. С. Степанов4, М. А. ЖУКОВА5

1 Степанов Сергей Валериевич, доктор технических наук, технический директор ООО ГК «Эколос»; профессор кафедры «Водоснабжение и водоотведение», Архитектурно-строительный институт Самарского государственного технического университета

443001, Россия, г. Самара, Молодогвардейская ул., 194, тел.: (846) 242-37-63, e-mail: stepanov@ecolos.ru

2 Солкина Ольга Сергеевна, руководитель инженерного отдела, ООО ГК «Эколос»; аспирант, Архитектурно-строительный институт Самарского государственного технического университета

443001, Россия, г. Самара, Молодогвардейская ул., 194, тел.: (846) 242-37-63, e-mail: solkina@ecolos.ru

3 Морозова Ксения Михайловна, кандидат технических наук, старший научный сотрудник

E-mail: morozova06@inbox.ru

4 Степанов Александр Сергеевич, кандидат технических наук, генеральный директор ООО ГК «Эколос»

443036, Россия, г. Самара, Набережная реки Самары, 1, тел.: (846) 205-99-55, e-mail: alstepanov@ecolos.ru

5 Жукова Мария Александровна, инженер, ООО ГК «Эколос»; магистрант, Архитектурно-строительный институт Самарского государственного технического университета

443036, Россия, г. Самара, Набережная реки Самары, 1, тел.: (846) 205-99-55, e-mail: engineer2@ecolos.ru

Представлены результаты научных исследований в области биомембранной очистки сточных вод предприятий молочной промышленности. Экспериментальные исследования проводили в течение четырех месяцев. В качестве модельного стока использовали раствор молока с добавлением биогенных элементов в виде минеральных солей – однозамещенного фосфорнокислого калия, хлористого аммония и нитрата натрия в пропорциях, соответствующих концентрациям этих веществ в натурных сточных водах, прошедших физико-химическую обработку. Целью эксперимента являлось получение технологических и кинетических характеристик биологических процессов в мембранном биореакторе. Исследования, проведенные с модельным стоком на пилотном биореакторе, показали, что при поддержании дозы ила 7–9 г/л и при удельном потоке 0,36 м3/(м2сут), соответствующем обычному режиму эксплуатации плоскорамных мембран, работа установки без химических промывок возможна в течение около двух месяцев. Установлено, что промывка мембран лимонной кислотой с концентрацией раствора 1000 мг/л (концентрация кислоты в мембранном резервуаре 5–20 мг/л) снижает скорость биохимических процессов, причем исключительно за счет подкисления иловой смеси. Токсичность лимонной кислоты не проявляется в диапазоне рН иловой смеси 6,83–7,17. Изучение влияния промывки мембран раствором гипохлорита натрия с концентрацией по NaOCl 2000 мг/л показало наличие конкурентного торможения скорости окисления органических веществ с практически полным подавлением биохимических процессов при концентрации активного хлора в реакторе выше 26,1 мг/л. Продолжительность восстановления первоначальной скорости окисления органических веществ при концентрации активного хлора в реакторе 6,6 мг/л составила 220 минут после включения подачи сточных вод и отвода пермеата.

Ключевые слова: сточные воды молокозавода, биологическая очистка, мембранный биореактор, химическая промывка мембран, удельная скорость окисления, кинетическая константа.

СПИСОК ЛИТЕРАТУРЫ

- Rosenberger S., Kruger U., Witzig R., Manz W., Szewzyk U., Kraume M. Performance of a bioreactor with submerged membranes for aerobic treatment of municipal wastewater // Water Research. 2002. V. 36 (15). Р. 413–420.

- Rosenberger S., Laabs C., Lesjean B., Gnirss R., Amy G., Jekel M., Schrotter J.-C. Impact of colloidal and soluble organic material on membrane performance in membrane bioreactors for municipal wastewater treatment // Water Research. 2006. V. 40 (4). P. 710–720.

- Швецов В. Н., Морозова К. М., Нечаев И. А., Киристаев А. В. Теоретические и технологические аспекты применения биомембранной технологии глубокой очистки сточных вод (часть 2) // Водоснабжение и санитарная техника. 2007. № 1. С. 10–13.

- Степанов С. В., Солкина О. С., Морозова К. М., Степанов А. С., Соколова Т. В. Исследование биологической очистки сточных вод молокозаводов на модельном стоке / Технологии очистки воды «ТЕХНОВОД–2016»: Материалы IX Международной научно-практической конференции, г. Ростов-на-Дону, 5–7 октября 2016 г. – Новочеркасск: Лик, 2016. С. 210–215.

- Степанов С. В., Солкина О. С., Морозова К. М., Степанов А. С., Соколова Т. В., Жукова М. А. Биологическая очистка сточных вод предприятий молочной промышленности в мембранном биореакторе (часть 1) // Водоснабжение и санитарная техника. 2016. № 12. С. 28–34.

- Степанов С. В., Солкина О. С., Морозова К. М., Степанов А. С., Соколова Т. В., Жукова М. А. Биологическая очистка сточных вод предприятий молочной промышленности в мембранном биореакторе (часть 2) // Водоснабжение и санитарная техника. 2017. № 2. С. 60–65.

- Роговская Ц. И. Биохимический метод очистки производственных сточных вод. – М.: Стройиздат, 1967. 140 с.

WASTEWATER TREATMENT

Stepanov S. V., Solkina O. S., Morozova K. M., Stepanov A. S., Zhukova M. A. The effect of membrane chemical washing on biological treatment processes 19

S. V. STEPANOV1, O. S. SOLKINA2, K. M. MOROZOVA3, A. S. STEPANOV4, M. A. ZhUKOVA5

1 Stepanov Sergei Valerievich, Doctor of Engineering, Technical Director, «Ecolos» Group, LLC; Professor, «Water Supply and Wastewater Disposal» Department, Architecture and Civil Engineering Institute, Samara State Technical University

194 Molodogvardeiskaia St., 443001, Samara, Russian Federation, tel.: +7 (846) 242-37-63, e-mail: stepanov@ecolos.ru

2 Solkina Ol'ga Sergeevna, Chief of Engineering Section, Architecture and Civil Engineering Institute, Samara State Technical University

194 Molodogvardeiskaia St., 443001, Samara, Russian Federation, tel.: +7 (846) 242-37-63, e-mail: solkina@ecolos.ru

3 Morozova Kseniia Mikhailovna, Ph. D. (Engineering), Senior Research Worker

E-mail: morozova06@inbox.ru

4 Stepanov Aleksandr Sergeevich, Ph. D. (Engineering), General Director, «Ecolos» Group, LLC

1 Samara River Emb., 443036, Samara, Russian Federation, tel.: +7 (846) 205-99-55, e-mail: alstepanov@ecolos.ru

5 Zhukova Mariia Aleksandrovna, Engineer, «Ecolos» Group, LLC; Graduate Student, Architecture and Civil Engineering Institute, Samara State Technical University

1 Samara River Emb., 443036, Samara, Russian Federation, tel.: +7 (846) 205-99-55, e-mail: engineer2@ecolos.ru

The results of research studies in the field of biomembrane treatment of dairy wastewater are presented. The experimental studies were carried out during four months. As a model effluent milk solution was used with addition of nutrients in the form of mineral salts – potassium monophosphate, ammonium chloride and sodium nitrate in ratios corresponding to the concentrations of these substances in original wastewater after physical and chemical treatment. The experiment was aiming at determining the process and kinetic characteristics of biological processes in a membrane bioreactor. The studies of the model wastewater in a pilot bioreactor showed that at 7–9 g/l sludge dosage and 0.36 m3/(m2×day) specific flow rate corresponding to the regular operation mode of flat-frame membranes the operation of the plant without chemical washing was possible for about two months. It was stated that membrane washing with citric acid at 1000 mg/l solution concentration (5–20 mg/l acid concentration in the membrane tank) provided for reducing the rate of biochemical processes with only mixed liquor acidification. The citric acid toxicity did not appear at mixed liquor pH in the range of 6.83–7.17. The study of membrane washing with sodium hypochlorite solution in concentration of 2000 mg/l as NaOCl showed the competitive inhibition of the organics oxidation rate with practically complete inhibition of biochemical processes at the active chlorine concentration in the reactor more than 26.1 mg/l. The time of restoring the original rate of organics oxidation at 6.6 mg/l active chlorine concentration in the reactor was 220 minutes after wastewater supply had been started and permeate had been removed.

Key words: dairy wastewater, biological treatment, membrane bioreactor, membrane chemical washing, specific oxidation rate, kinetic constant.

REFERENCES

- Rosenberger S., Kruger U., Witzig R., Manz W., Szewzyk U., Kraume M. Performance of a bioreactor with submerged membranes for aerobic treatment of municipal wastewater. Water Research, 2002, v. 36 (15), pp. 413–420.

- Rosenberger S., Laabs C., Lesjean B., Gnirss R., Amy G., Jekel M., Schrotter J.-C. Impact of colloidal and soluble organic material on membrane performance in membrane bioreactors for municipal wastewater treatment. Water Research, 2006, v. 40 (4), pp. 710–720.

- Shvetsov V. N., Morozova K. M., Nechaev I. A., Kiristaev A. V. [Theoretical and technological aspects of the use of biomembrane technologies for advanced wastewater treatment (part 2)]. Vodosnabzhenie i Sanitarnaia Tekhnika, 2007, no. 1, pp. 10–13. (In Russian).

- Stepanov S. V., Solkina O. S., Morozova K. M., Stepanov A. S., Sokolova T. V. [Study of dairy wastewater biological treatment with model effluent]. «TEKHNOVOD–2016» water treatment technologies. Proceedings of IX International Scientific-Practical Conference, Rostov-on-Don, October 5–7, 2016. Novocherkassk, Lik Publ., 2016, pp. 210–215. (In Russian).

- Stepanov S. V., Solkina O. S., Morozova K. M., Stepanov A. S., Sokolova T. V., Zhukova M. A. [Biological treatment of dairy industry wastewater in a membrane bioreactor (part 1)]. Vodosnabzhenie i Sanitarnaia Tekhnika, 2016, no. 12, pp. 28–34. (In Russian).

- Stepanov S. V., Solkina O. S., Morozova K. M., Stepanov A. S., Sokolova T. V., Zhukova M. A. [Biological treatment of dairy industry wastewater in a membrane bioreactor (part 2)]. Vodosnabzhenie i Sanitarnaia Tekhnika, 2017, no. 2, pp. 60–65. (In Russian).

- Rogovskaia Ts. I. Biokhimicheskii metod ochistki proizvodstvennykh stochnykh vod [Biochemical method of industrial wastewater treatment. Moscow, Stroiizdat Publ., 1967, 140 p.].

УДК 628.166.085

Смирнов А. Д., Стрелков А. К., Ткачев А. А. К вопросу выбора технологии обеззараживания сточных вод 25

А. Д. СМИРНОВ1, А. К. СТРЕЛКОВ2, А. А. ТКАЧЕВ3

1 Смирнов Александр Дмитриевич, доктор технических наук, главный научный сотрудник, АО «НИИ ВОДГЕО»

119435, Россия, Москва, Б. Саввинский пер., 9, стр. 1, тел.: (499) 272-47-58, e-mail: nii-vodgeo@mail.ru

2 Стрелков Александр Кузьмич, доктор технических наук, профессор, заведующий кафедрой «Водоснабжение и водоотведение», Архитектурно-строительный институт Самарского государственного технического университета

443001, Россия, г. Самара, Молодогвардейская ул., 194, тел.: (846) 333-56-76, е-mail: a19400209@yandex.ru

3 Ткачев Андрей Анатольевич, заместитель генерального директора по технологии, НПО «ЛИТ»

107076, Россия, Москва, Краснобогатырская ул., 44, тел.: (495) 733-95-26, e-mail: tkachev@npo.lit.ru

Ужесточение природоохранного законодательства в части сброса хлора и хлорорганических соединений побуждает к использованию современных безопасных и эффективных бесхлорных методов обеззараживания сточных вод. Использование ультрафиолетового излучения позволяет соблюдать все требования, предъявляемые к технологии обеззараживания сточных вод, не нанося никакого ущерба окружающей среде. Применение такой технологии исключает возможность начисления штрафов за загрязнение окружающей среды хлором и хлорорганическими соединениями. Экономическое сравнение двух методов обеззараживания (применение гипохлорита натрия и УФ-облучение) также показывает значительную эксплуатационную выгоду при внедрении УФ-обеззараживания. Годовая экономия в эксплуатационных расходах составляет 50% и более при сопоставимых капитальных затратах. Достоинства УФ-обеззараживания: эффективность в отношении всех типов индикаторных и патогенных микроорганизмов, в то время как хлорсодержащие реагенты зачастую не обеспечивают достаточной эффективности по вирусам и простейшим; не образуются побочные продукты, отсутствует негативное воздействие на водные объекты (а значит, нет и штрафов); нет опасности передозировки; легкий и простой контроль над процессом обеззараживания, нет необходимости в системе дозирования и дополнительных системах контроля; независимость эффекта обеззараживания при колебаниях качества сточных вод в пределах проектных параметров; минимальная занимаемая площадь, не нужны дополнительные сооружения (например, контактный резервуар для дехлорирования); возможность «вписаться» в уже существующие здания или сооружения, например, при модернизации очистных сооружений канализации и замене хлорирования; низкие эксплуатационные расходы (замена ламп один раз в полтора-два года и электроэнергия). В таких условиях переход на использование бесхлорных технологий обеззараживания, не приводящих к образованию хлорорганических загрязнителей, является верным решением.

Ключевые слова: сточные воды, обеззараживание, хлорирование, дехлорирование, ультрафиолетовое излучение, плата за сбросы.

## СПИСОК ЛИТЕРАТУРЫ

- Загорский В. А., Козлов М. Н., Данилович Д. А. Методы обеззараживания сточных вод // Водоснабжение и санитарная техника. 1998. № 2. С. 2–5.

- Информационно-технический справочник по наилучшим доступным технологиям ИТС 10-2015 «Очистка сточных вод с использованием централизованных систем водоотведения поселений, городских округов». – М., Бюро НДТ, 2015. 378 с.

- Бреслов Б. Е., Бивалькевич А. И., Смирнов А. Д., Стрелков А. К. Эффективность и экономическая целесообразность промышленных методов обеззараживания сточных вод // Водоснабжение и санитарная техника. 2012. № 1. С. 34–41.

- Василяк Л. М., Смирнов А. Д. Современная проблематика обеззараживания сточных вод // ЖКХ для руководителя. 2015. № 5. С. 45–48.

Smirnov A. D., Strelkov A. K., Tkachev A. A. Revisiting the selection of wastewater disinfection technology 25

A. D. SMIRNOV1, A. K. STRELKOV2, A. A. TKACHEV3

1 Smirnov Aleksandr Dmitrievich, Doctor of Engineering, Chief Research Worker, «NII VODGEO» JSC

Build. 1, 9 B. Savvinskii Lane, 119435, Moscow, Russian Federation, tel.: +7 (499) 272-47-58, e-mail: nii-vodgeo@mail.ru

2 Strelkov Aleksandr Kuz'mich, Doctor of Engineering, Professor, Chairman of «Water Supply and Wastewater Disposal» Department, Institute of Architecture and Civil Engineering, Samara State Technical University

194 Molodogvardeiskaia St., 443001, Samara, Russian Federation, tel.: +7 (846) 333-56-76, е-mail: a19400209@yandex.ru

3 Tkachev Andrei Anatol'evich, Deputy General Director for Technology, «LIT» Scientific-Production Association

44 Krasnobogatyrskaia St., 107076, Moscow, Russian Federation, tel.: +7 (495) 733-95-26, e-mail: tkachev@npo.lit.ru

Environmental law enforcement in relation to discharging chlorine and organochlorine contaminants induces the use of advanced safe and efficient chlorine free methods of wastewater disinfection. The use of ultraviolet irradiation provides for meeting all the requirements imposed on the technology of wastewater disinfection eliminating any adverse impact on the environment. The use of this technology eliminates possible penalty charges for polluting the environment with chlorine and organochlorine contaminants. The economic comparison of two disinfection methods (the application of sodium hypochlorite and UV-irradiation) reveals significant operational benefit when introducing UV-disinfection. Annual savings of operational expenditures is 50% and more at the comparable capital expenditures. The advantages of UV-disinfection are as follows: efficiency in relation to all types of indicator and pathogenic microorganisms, whereas chlorine-containing chemicals do not often ensure fair efficiency in viruses and protozoa elimination; no by-products are generated; no adverse impact on the water bodies is produced (consequently, no penalties); no threat of overdosing; easy and simple control over the disinfection process; no need in dosing system and additional control systems; disinfection effect independence from fluctuations of effluents quality within the limits of the design parameters; minimum footprint; no additional facilities are needed (e. g., contact tank for dechlorination); possible retrofitting into the existing buildings or facilities, for example, in the process of wastewater treatment facilities upgrade and chlorination replacement; low operational expenditures (lamp replacing once in a year and a half – two years and energy saving). In this context switching to chlorine free disinfection technologies that eliminate the formation of organochlorine contaminants is a correct solution.

Key words: wastewater, disinfection, chlorination, dechlorination, ultraviolet irradiation, effluents charge.

## REFERENCES

- Zagorskii V. A., Kozlov M. N., Danilovich D. A. [Methods of wastewater disinfection]. Vodosnabzhenie i Sanitarnaia Tekhnika, 1998, no. 2, pp. 2–5. (In Russian).

- Informatsionno-tekhnicheskii spravochnik po nailuchshim dostupnym tekhnologiiam ITS 10-2015 «Ochistka stochnykh vod s ispol'zovaniem tsentralizovannykh sistem vodootvedeniia poselenii, gorodskikh okrugov» [Information and Technical Guide for the Best Available Techniques ITS 10-2015 «Wastewater Treatment with the Use of Public Wastewater Disposal Systems of Communities, Urban Districts». Moscow, Biuro NDT Publ., 2015, 378 p.].

- Breslov B. E., Bival'kevich A. I., Smirnov A. D., Strelkov A. K. [Efficiency and economic expediency of industrial methods of wastewater disinfection]. Vodosnabzhenie i Sanitarnaia Tekhnika, 2012, no. 1, pp. 34–41. (In Russian).

- Vasiliak L. M., Smirnov A. D. [Contemporary problematics of wastewater disinfection]. ZHKKH dlia Rukovoditelia, 2015, no. 5, pp. 45–48. (In Russian).

УДК 628.31:628.38

Кофман В. Я. Повышение эффективности использования водных ресурсов: децентрализованная обработка серых сточных вод 30

В. Я. КОФМАН*

* Кофман Владимир Яковлевич, старший научный сотрудник, Всероссийский институт научной и технической информации Российской академии наук

125190, Россия, Москва, ул. Усиевича, 20, тел.: (495) 155-43-08, e-mail: kofman16@rambler.ru

Сложившаяся на протяжении многих десятилетий стратегия обработки городских сточных вод требует внесения корректив, поскольку она не ориентирована на повторное использование воды (вместо сброса в водотоки-приемники), не предназначена для удаления биогенов и требует значительных затрат на очистку от ксенобиотиков. При централизованной обработке сточных вод невозможно качественно решить данную проблему. За рубежом активно разрабатывается концепция раздельного отвода и децентрализованной обработки концентрированных черных сточных вод, содержащих мочу и фекалии, а также серых сточных вод, содержащих стоки ванных комнат, душевых, раковин для умывания, стиральных и посудомоечных машин, кухонь. Серые сточные воды содержат значительно меньше (в сравнении с черными сточными водами и общим потоком городских стоков) ксенобиотиков и патогенов и могут служить источником воды различной степени чистоты. Данный подход создает предпосылки для более устойчивого городского водоснабжения, в частности, за счет отказа от обеспечения всех потребностей из одного источника питьевой воды, тем более в случаях, когда нет необходимости потребления воды высокого качества. В США, Японии, Австралии, Израиле, ряде стран Евросоюза ведутся различного масштаба исследования, на практике уже реализованы схемы разных вариантов непитьевого использования серых сточных вод, прошедших необходимую обработку.

Ключевые слова: серые сточные воды, децентрализованная обработка, технологические схемы очистки, ксенобиотики, повторное использование воды.

Kofman V. Ia. Improving the efficiency of water resources management: decentralized grey water treatment 30

V. Ia. KOFMAN*

* Kofman Vladimir Iakovlevich, Senior Research Fellow, All-Russian Institute of Scientific and Technical Information, Russian Academy of Sciences

20 Usuevicha St., 125190, Moscow, Russian Federation, tel.: +7 (495) 155-43-08, e-mail: kofman16@rambler.ru

The established over the course of decades strategy of municipal wastewater treatment needs to be revised since it is not oriented toward water reuse (instead of effluent discharge into the receiving waters); it is not designed for nutrients removal and demands significant expenditures for the removal of xenobiotics. The centralized wastewater treatment does not provide for the qualitative solution of the given problem. Overseas the concept of separate collection and decentralized treatment of concentrated black water containing urea and faeces and grey water containing bath, shower and sink water, water from washing machines and dishwashers, kitchen water has been rapidly developed. Grey water contains much less (compared to black water and municipal wastewater) xenobiotics and pathogens and can be a source of water of different purity levels. This approach predetermines more sustainable municipal water supply, in particular, by avoiding meeting all the demands from a single drinking water source, all the more in cases when there is no need in high-quality water. In USA, Japan, Australia, Israel, some EU countries studies of different scales are carried out; in practice different schemes of nonpotable use of grey water after the required treatment have been implemented.

Key words: grey water, decentralized treatment, treatment process flow schemes, xenobiotics, water reuse.

REFERENCES

- Zeeman G., Kujawa K., de Mes T., et al. Anaerobic treatment as a core technology for energy, nutrients and water recovery from source-separated domestic waste(water). Water Science & Technology, 2008, v. 57, pp. 1207–1212.

- Zeeman G., Katarzyna K., Brendo M., Flip K. Full scale demonstration of vacuum collection, transport & treatment of black water. Advanced Sanitation Conference. Aachen, Germany, 2007.

- Ahmed M., Arora M. Suitability of grey water recycling as decentralized alternative water supply option for integrated urban water management. IOSR Journal of Engineering, 2012, v. 2, pp. 31–35.

- McCarty P. L., Bae J., Kim J. Domestic wastewater treatment as a net energy producer – Can this be achieved? Environmental Science & Technology, 2011, no. 45, pp. 7100–7106.

- Weirich S. R., Silverstein J., Rajagopalan B. Effect of everage flow and capacity utilization on effluent water quality from US municipal wastewater treatment facilities. Water Research, 2011, no. 45 (14), pp. 4279–4286.

- Barsan N., Nedeff V., Lazar G. Current stage of domestic wastewater treatment in small plants. Journal of Engineering Studies and Research, 2011, no. 7 (4), pp. 13–23.

- Li F., Behrendt J., Wichmann K., Otterpohl R. Resources and nutrients oriented greywater treatment for non-potable reuses. Water Science and Technology, 2008, v. 57, no. 12, pp. 1901–1907.

- Li F., Wichmann K., Otterpohl R. Review of the technological approaches for grey water treatment and reuses. Science of the Total Environment, 2009, v. 407, pp. 3439–3449.

- Ernst M., Sperlich A., Zheng X., et al. An integrated wastewater treatment and reuse concept for the Olympic Park 2008, Beijing. Desalination, 2006, v. 202, no. 1–2, pp. 293–301.

- Maeda M., Nakada K., Kawamoto K., Ikeda M. Area-wide use of reclaimed water in Tokyo, Japan. Water Science and Technology, 1996, v. 33, no. 10–11, pp. 51–57.

- Ramon G., Green M., Semiat R., et al. Low strength greywater characterization and treatment by direct membrane filtration. Desalination, 2004, v. 170, no. 3, pp. 241–250.

- Pidou M., Avery I., Stephenson T., et al. Chemical solutions for greywater recycling. Chemosphere, 2008, v. 71, no. 1, pp. 147–155.

- Nolde E. Greywaterreuse systems for toilet flushing in multi-story buildings – over ten yeares experience in Berlin. Urban Water, 1999, no. 1, pp. 275–284.

- Gross A., Shmueli O., Ronen Z., Raveh E. Recycled vertical flow constructed wetland (RVFCW) – a novel method of recycling greywater for irrigation in small communities. Chemosphere, 2007, v. 66, no. 5, pp. 916–923.

- Hernandez L. L., Temmink H., Zeeman G., Buisman C. J. N. Comparison of three systems for biological greywater treatment. Water, 2010, v. 2, no. 2, pp. 155–169.

- Hernandez L. L., Temmink H., Zeeman G., Buisman C. J. N. Bioflocculation of grеy water for improved energy recovery within decentralized sanitation concepts. Bioresourсe Technology, 2010, v. 101, pp. 9065–9070.

- Al-Jayyousi O. R. Greywater reuse: towards sustainable water management. Desalination, 2003, v. 156, pp. 181–192.

- Merz C., Scheumann R., El Hamouri B., et al. Membrane bioreactor technology for the treatment of greywater from a sports and leisure club. Desalination, 2007, v. 215, pp. 37–43.

- Pangarkar B. L., Parjane S. B., Sane M. G. Design and economical performance of gray water treatment plant in rural region. International Journal of Environmental, Chemical, Ecological and Geopsysical Engineering, 2010, v. 4, no. 1, pp. 6–10.

- March J. G., Gual M., Orozco F. Experience on greywater re-use for toilet flushing in a hotel (Mallorca Island, Spain). Desalination, 2004, v. 164, pp. 241–247.

- Friedler E., Hadari M. Economic feasibility of on-site greywater reuse in multi-storey buildings. Desalination, 2006, v. 190, pp. 221–234.

- Hernandez L. L., Vieno N., Temmink H., et al. Occurence of xenobiotics in gray water and removal in three bilogical treatment systems. Environmental Science and Technology, 2010, v. 44, no. 17, pp. 6835–6842.

- Zeng T., Mitch W. A. Contribution of N-nitrosamines and their precorsors to domestic sewage by greywaters and blackwaters. Environmental Science & Technology, 2015, v. 49, pp. 13158–13167.

- Butkovskyi A., Jeremiasse A. W., Hernandes L. L., et al. Electrochemical conversion of micropollutants in gray water. Environmental Science & Technology, 2014, v. 48, pp. 1893–1901.

- Ramprasad C., Philip L. Surfactants and personal care products removal in pilot scale horizontal and vertical flow constructed wetlands while treating greywater. Chemical Engineering Journal, 2016, v. 284, pp. 458–468.

ГОРЯЧЕЕ ВОДОСНАБЖЕНИЕ

УДК 697.34

Рафальская Т. А., Мансуров Р. Ш. Оценка влияния температуры воды в системе горячего водоснабжения на температурный режим помещений 42

Т. А. РАФАЛЬСКАЯ1, Р. Ш. МАНСУРОВ2

1 Рафальская Татьяна Анатольевна, кандидат технических наук, доцент, Новосибирский государственный архитектурно-строительный университет (Сибстрин)

630008, Россия, г. Новосибирск, Ленинградская ул., 113, тел.: (913) 982-55-76, e-mail: rafalskaya.ta@yandex.ru

2 Мансуров Рустам Шамильевич, кандидат технических наук, доцент, заведующий кафедрой теплогазоснабжения и вентиляции, Новосибирский государственный архитектурно-строительный университет (Сибстрин)

630008, Россия, г. Новосибирск, Ленинградская ул., 113, тел.: (909) 534-71-43, e-mail: rmansurov@inbox.ru

В настоящее время в системах централизованного теплоснабжения применяются две срезки температурного графика центрального регулирования. Согласно действующим нормативным документам по обеспечению качества горячей воды, с целью улучшения ее микробиологических характеристик необходимо увеличить температуру воды на нужды горячего водоснабжения. Для этого была поднята нижняя срезка температурного графика центрального регулирования. В то же время в связи с невозможностью поддерживать высокотемпературные графики источников теплоты используется верхняя срезка температурного графика. Однако в современных схемах тепловых пунктов применяется связанная подача теплоты в системы горячего водоснабжения и отопления. Исследовано влияние температурных срезок на тепловой режим помещений. Проведен сравнительный поверочный расчет тепловых режимов теплового пункта в 2011–2012 годах, когда применялись две срезки температурного графика, и в 2014–2015 годах, когда верхняя срезка была поднята (также была поднята и нижняя срезка). Температура внутреннего воздуха помещений определялась с учетом различных коэффициентов тепловой аккумуляции зданий, при различных температурах наружного воздуха и переменном водопотреблении в системе горячего водоснабжения. Расчеты показали, что поднятие нижней срезки оказало более существенное влияние на тепловой режим помещений, чем поднятие верхней срезки. Увеличение температуры воды в системе горячего водоснабжения может вызвать неблагоприятный тепловой режим в отапливаемых помещениях, поэтому поднятие нижней срезки нельзя считать энергоэффективным решением проблемы. Устранение недостатков эксплуатации систем горячего водоснабжения, а также периодическое проведение термической дезинфекции воды позволит обеспечить соответствие современным нормативным документам без ухудшения качества теплоснабжения.

Ключевые слова: система теплоснабжения, система отопления, система горячего водоснабжения, тепловой режим помещений, температурный график центрального регулирования, тепловой пункт, коэффициент тепловой аккумуляции зданий.

СПИСОК ЛИТЕРАТУРЫ

- Научно-методическое обоснование микробиологической безопасности снижения температуры горячей воды в системах водоснабжения закрытого типа. – М., ФГБУ ФНИЦЭМ, 2015. 48 с.

- Петрущенков В. А. Обоснование пониженного температурного графика регулирования централизованных систем теплоснабжения // Новости теплоснабжения. 2015. № 8. C. 30–37.

- Гершкович В. Ф. Сто пятьдесят… Норма или перебор? (Размышления о параметрах теплоносителя) // Энергосбережение. 2004. № 5. C. 14–19.

- Рафальская Т. А. Тепловые и гидравлические режимы систем централизованного теплоснабжения / Актуальные вопросы технических наук: теоретический и практический аспекты. – Уфа: Аэтерна, 2016. Вып. 5. С. 116–171.

- Малявина Е. Г., Цыганков А. В. Влияние различных факторов на темп остывания помещения после отключения теплоснабжения // Известия вузов. Строительство. 2015. № 2. С. 53–59.

- Перехоженцев А. Г. Нормирование сопротивления теплопередаче наружных ограждений зданий по условию теплового комфорта в помещении // Вестник МГСУ. 2016. № 2. С. 173–185.

- Бодров В. И., Корягин М. В. Методика расчета теплового режима зданий массовой застройки в период «температурных срезов» // Известия вузов. Строительство. 2007. № 2. С. 42–46.

- Кононович Ю. В. Тепловой режим зданий массовой застройки. – М.: Стройиздат, 1986. 157 с.

- Попырин Л. С. Природно-техногенные аварии в системах теплоснабжения // Вестник РАН. 2000. Т. 70. № 7. С. 604–610.

HOT WATER SUPPLY

Rafal'skaia T. A., Mansurov R. Sh. Evaluation of the water temperature in the hot water system effect on the indoor temperature conditions 42

T. A. RAFAL'SKAIA1, R. Sh. MANSUROV2

1 Rafal'skaia Tat'iana Anatol'evna, Ph. D. (Engineering), Associate Professor, Novosibirsk State University of Architecture and Civil Engineering (Sibstrin)

113 Leningradskaia St., 630008, Novosibirsk, Russian Federation, tel.: +7 (913) 982-55-76, e-mail: rafalskaya.ta@yandex.ru

2 Mansurov Rustam Shamil'evich, Ph. D. (Engineering) Associate Professor, Head of Heat and Gas Supply and Ventilation Department, Novosibirsk State University of Architecture and Civil Engineering (Sibstrin)

113 Leningradskaia St., 630008, Novosibirsk, Russian Federation, tel.: +7 (909) 534-71-43, e-mail: rmansurov@inbox.ru

In the modern centralized heat supply systems two breakpoints of the central control heating curve are used. According to the effective regulative documents on ensuring the quality of hot water the temperature of supplied hot water should be raised in order to improve its microbiological characteristics. For this purpose the lower breakpoint of the central control heating curve was raised. At the same time in view of the impossibility of maintaining high temperature heating curves of the heat sources the upper breakpoint of the heating curve is used. However in the present day schemes of heat supply stations coordinated heat supply to the systems of hot water and heat supply is used. The effect of temperature breakpoints on the indoor conditions was studied. A comparative checking calculation of the heating curves of a heat supply station in 2011–2012 was carried out when two breakpoints of the heating curve were used and in 2014–2015 when the upper breakpoint was raised (the lower breakpoint was raised too). The indoor temperature was determined with account of various building heat accumulation coefficients at different outdoor temperatures and variable water consumption in the hot water system. The calculations showed that raising the lower breakpoint produced more significant effect on the indoor heat conditions compared to the effect of raising the upper breakpoint. Raising the temperature in the hot water system can cause unfavorable heat conditions in the heated space; therefore raising the lower breakpoint cannot be considered as an energy efficient solution of the problem. Eliminating the drawbacks in the operation of hot water systems as well as carrying out regular thermal water disinfection can ensure meeting the requirements of the effective regulative documents eliminating deterioration of the heat supply quality.

Key words: heat supply system, heating system, hot water system, indoor heat conditions, heating curve of the central control, heat supply station, building heat accumulation coefficient.

REFERENCES

- Nauchno-metodicheskoe obosnovanie mikrobiologicheskoi bezopasnosti snizheniia temperatury goriachei vody v sistemakh vodosnabzheniia zakrytogo tipa [Scientific-methodological substantiation of microbiological safety of lowering hot water temperature in closed-type water supply systems. Moscow, FGBU FNIITSEM Publ., 2015, 48 p.].

- Petrushchenkov V. A. [Substantiation of the lowered heating curves in centralized heat supply system control]. Novosti Teplosnabzheniia, 2015, no. 8, pp. 30–37. (In Russian).

- Gershkovich V. F. [One hundred fifty… Norm or overkill? (Reflections on the heat conductor parameters)]. Energosberezhenie, 2004, no. 5, pp. 14–19. (In Russian).

- Rafal'skaia T. A. [Heat and hydraulic conditions of the centralized heat supply systems. Topical issues of engineering sciences: theoretical and practical aspects]. Ufa, Aeterna Publ., 2016, issue 5, pp. 116–171. (In Russian).

- Maliavina E. G., Tsygankov A. V. [Effect of different factors on the rate of indoor cooling after heat supply cutoff]. Izvestiia Vuzov. Stroitel'stvo, 2015, no. 2, pp. 53–59. (In Russian).

- Perekhozhentsev A. G. [Regulation of heat transmission resistance of cladding by indoor thermal comfort]. Vestnik MGSU, 2016, no. 2, pp. 173–185. (In Russian).

- Bodrov V. I., Koriagin M. V. [Method of calculating heat conditions in low-cost housing during the period of «temperature breakpoints»]. Izvestiia Vuzov. Stroitel'stvo, 2007, no. 2, pp. 42–46. (In Russian).

- Kononovich Iu. V. Teplovoi rezhim zdanii massovoi zastroiki [Heat conditions in low-cost housing. Moscow, Stroiizdat Publ., 1986, 157 p.].

- Popyrin L. S. [Natural and man-made failures in heat supply systems]. Vestnik RAN, 2000, v. 70, no. 7, pp. 604–610. (In Russian).

НАДЕЖНОСТЬ ОБЪЕКТОВ ВКХ

УДК 628.1/.2:62-192

Алексеев М. И., Баранов Л. А., Ермолин Ю. А. Оценка времени жизни объектов ВКХ под воздействием периодически изменяющегося потока отказов 50

(в порядке обсуждения)

М. И. АЛЕКСЕЕВ1, Л. А. БАРАНОВ2, Ю. А. ЕРМОЛИН3

1 Алексеев Михаил Иванович, доктор технических наук, профессор, академик РААСН, Санкт-Петербургский государственный архитектурно-строительный университет

190005, Россия, Санкт-Петербург, 2-я Красноармейская ул., 4, тел.: (812) 316-72-97, e-mail: m.i.alexeev@mail.ru

2 Баранов Леонид Аврамович, доктор технических наук, профессор, Московский государственный университет путей сообщения Императора Николая II (МИИТ)

127994, Россия, Москва, ул. Образцова, 9, стр. 9, тел.: (495) 684-21-68, e-mail: baranov.miit@gmail.ru

3 Ермолин Юрий Александрович, доктор технических наук, профессор, Московский государственный университет путей сообщения Императора Николая II (МИИТ)

127994, Россия, Москва, ул. Образцова, 9, стр. 9, тел.: (495) 684-21-68, e-mail: ermolin.y@yandex.ru

Надежность объектов водопроводно-канализационного хозяйства принято оценивать рядом показателей, важнейшими из которых являются интенсивность отказов, функция надежности – вероятность безотказной работы объекта к моменту времени, среднее время между смежными отказами (время жизни) объекта. Приводится приближенное аналитическое решение задачи оценки надежности объекта для случая, когда интенсивность потока отказов, воздействующего на объект, представляет собой периодическую, кусочно-постоянную функцию времени. Предложено заменять такой нестационарный процесс фиктивным стационарным. Получена формула, выражающая постоянную интенсивность отказов эквивалентного фиктивного потока через параметры исходного реального процесса. Это дает возможность приближенного решения надежностных задач в аналитическом виде, удобном для практического применения и последующего анализа.

Ключевые слова: водопроводно-канализационное хозяйство, надежность, стационарность, интенсивность потока отказов, время жизни объекта, эквивалентная замена.

СПИСОК ЛИТЕРАТУРЫ

- Вентцель Е. С. Теория вероятностей. – М.: Наука, 1969. 576 с.

- Алексеев М. И., Ермолин Ю. А. Надежность сетей и сооружений систем водоотведения. – М.: Издательство АСВ, 2015. 200 с.

- Mays L. W. (Editor). Reliability analysis of water distribution systems. – New York: ASCE, 1989. 532 p.

- Hayworth G. I. (Editor). Reliability engineering advances. – New York: Nova Science Publishers, Inc., 2009. 441 p.

- Хай Л. Исследование надежности водораспределительных сетей Вьетнама и пути ее повышения: Дисс. … канд. техн. наук. – М., МГСУ, 2002. 217 с.

- Ermolin Yu. A. Reliability calculation under seasonally varying failure rate // ISA Transactions. 2007. V. 46. P. 127–130.

- Ermolin Yu. A. Stationarization of the seasonally changing failure flow (with reference to reliability problems) // Applied Mathematical Modelling. 2008. V. 32. № 10. P. 2034–2040.

- Баранов Л. А., Ермолин Ю. А. Оценка показателей надежности «линейно-стареющего» объекта // Надежность. 2015. № 4 (55). P. 57–60.

- Гнеденко Б. В., Беляев Ю. К., Соловьев А. Д. Математические методы в теории надежности. – М., 1965. 524 с.

- Храменков С. В. Стратегия модернизации водопроводной сети. – М.: Стройиздат, 2005. 400 с.

RELIABILITY OF WATER AND WASTEWATER FACILITIES

Alekseev M. I., Baranov L. A., Ermolin Iu. A. Evaluation of water and wastewater facilities life time under the conditions of periodic failure flow 50

(to be discussed)

M. I. ALEKSEEV1, L. A. BARANOV2, Iu. A. ERMOLIN3

1 Alekseev Mikhail Ivanovich, Doctor of Engineering, Professor, Academician, Russian Academy of Architecture and Constructional Sciences, Saint-Petersburg State University of Architecture and Civil Engineering

4, 2nd Krasnoarmeiskaia St., 190005, St. Petersburg, Russian Federation, tel.: +7 (812) 316-72-97, e-mail: m.i.alexeev@mail.ru

2 Baranov Leonid Avramovich, Doctor of Engineering, Professor, Emperor Nikolas II Moscow State University of Railway Engineering (MIIT)

Build. 9, 9 Obraztsova St., 127994, Moscow, Russian Federation, tel.: +7 (495) 684-21-68, e-mail: baranov.miit@gmail.ru

3 Ermolin Iurii Aleksandrovich, Doctor of Engineering, Professor, Emperor Nikolas II Moscow State University of Railway Engineering (MIIT)

Build. 9, 9 Obraztsova St., 127994, Moscow, Russian Federation, tel.: +7 (495) 684-21-68, e-mail: ermolin.y@yandex.ru

It is customary to evaluate the reliability of water and wastewater facilities by a number of indicators; the most important among them being the failure rate, reliability function – probability of failure free operation of a project by the time, average time between neighboring failures (life time) of the project. An approximate analytical solution of the task of the project reliability evaluation is presented for the case when the failure flow rate influencing the project is a periodic piecewise constant time function. It was suggested to replace this nonstationary process with a fictitious stationary one. The formula was derived stating the constant failure rate of the equivalent fictitious flow through the parameters of the initial real process. This provides for the possible approximate solution of reliability tasks in analytic form that is suitable for practical use and further analysis.

Key words: water and wastewater services, reliability, stationarity, failure flow intensity, project life time, equivalent replacement.

REFERENCES

- Venttsel' E. S. Teoriia veroiatnostei [Calculus of probabilities. Moscow, Nauka Publ., 1969, 576 p.].

- Alekseev M. I., Ermolin Iu. A. Nadezhnost' setei i sooruzhenii sistem vodootvedeniia [Reliability of wastewater networks and facilities. Moscow, Izdatel'stvo ASV Publ., 2015. 200 p.].

- Mays L. W. (Editor). Reliability analysis of water distribution systems. New York, ASCE, 1989, 532 p.

- Hayworth G. I. (Editor). Reliability engineering advances. New York, Nova Science Publishers, Inc., 2009, 441 p.

- Khai L. Issledovanie nadezhnosti vodoraspredelitel'nykh setei V'etnama i puti ee povysheniia [Study of water distribution network reliability in Vietnam and ways of its improvement: Ph. D. thesis in Engineering science. Moscow, MGSU Publ., 2002, 217 p.].

- Ermolin Yu. A. Reliability calculation under seasonally varying failure rate. ISA Transactions, 2007, v. 46, pp. 127–130.

- Ermolin Yu. A. Stationarization of the seasonally changing failure flow (with reference to reliability problems). Applied Mathematical Modelling, 2008, v. 32, no. 10, pp. 2034–2040.

- Baranov L. A., Ermolin Iu. A. [Evaluation of the indicators of the reliability of a linearly aging project]. Nadezhnost’, 2015, no. 4 (55), pp. 57–60. (In Russian).

- Gnedenko B. V., Beliaev Iu. K., Solov'ev A. D. Matematicheskie metody v teorii nadezhnosti [Mathematical methods in the reliability theory. Moscow, 1965, 524 p.].

- Khramenkov S. V. Strategiia modernizatsii vodoprovodnoi seti [Strategy of the water distribution network upgrade. Moscow, Stroiizdat Publ., 2005, 400 p.].

ВОДОПРОВОДНЫЕ СЕТИ

УДК 628.144:532.542

Баранчикова Н. И., Епифанов С. П., Зоркальцев В. И., Куртин А. В., Обуздин С. Ю. Потокораспределение в системах подачи и распределения воды с автоматическими регуляторами давления 55

Н. И. БАРАНЧИКОВА1, С. П. ЕПИФАНОВ2, В. И. ЗОРКАЛЬЦЕВ3, А. В. КУРТИН4, С. Ю. ОБУЗДИН5

1 Баранчикова Надежда Ивановна, кандидат физико-математических наук, доцент кафедры «Городское строительство и хозяйство», Иркутский национальный исследовательский технический университет

664074, Россия, г. Иркутск, ул. Лермонтова, 83, тел.: (964) 212-36-37, e-mail: nadin-0105@mail.ru

2 Епифанов Сергей Петрович, кандидат физико-математических наук, доцент кафедры «Городское строительство и хозяйство», Иркутский национальный исследовательский технический университет

664074, Россия, г. Иркутск, ул. Лермонтова, 83, тел.: (914) 946-86-12, e-mail: epifanov.1954@mail.ru

3 Зоркальцев Валерий Иванович, доктор технических наук, главный научный сотрудник, Институт систем энергетики имени Л. А. Мелентьева Сибирского отделения Российской академии наук

664033, Россия, г. Иркутск, ул. Лермонтова, 130, а/я 309, тел.: (3952) 42-88-27, e-mail: zork@isem.sei.irk.ru

4 Куртин Андрей Владимирович, заместитель главного инженера по эксплуатации, МУП «Водоканал» города Иркутска

664081, Россия, г. Иркутск, ул. Станиславского, 2, тел.: (3952) 21-47-99, e-mail: kurtinav@irkvkx.ru

5 Обуздин Сергей Юрьевич, заместитель главного инженера по автоматизации и режимам, МУП «Водоканал» города Иркутска

664081, Россия, г. Иркутск, ул. Станиславского, 2, тел.: (3952) 21-47-99, e-mail: obuzdinsy@irkvkx.ru

Рассматривается задача потокораспределения в произвольных трубопроводных системах, в том числе в системах подачи и распределения воды с автоматическими регуляторами давления. Для повышения надежности функционирования систем часто устанавливают регуляторы давления «после себя» (редукционные клапаны) и «до себя» (резервуарные, перепускные клапаны). На водопроводных сетях г. Иркутска в последние годы были установлены регуляторы давления производства фирмы Bermad, что позволило снизить и стабилизировать давление в некоторых зонах. Но есть зоны, которые имеют питание от нескольких водоводов с различным давлением в местах подключения, поэтому иногда происходит полное закрытие некоторых регуляторов (чаще меньших диаметров), либо возможен переворот потока, и вода движется через регулятор в обратном направлении. При этом регулятор не выполняет свою функцию. Математическое моделирование таких крайних ситуаций очень важно, так как именно они существенно усложняют эксплуатацию систем подачи и распределения воды. Предложены математические модели автоматических регуляторов давления «до себя», «после себя» и перепада давления с учетом их исполнения: нормально открыты или нормально закрыты. Приведена математическая постановка задачи потокораспределения в системах подачи и распределения воды с регуляторами давления. Решена актуальная для практики задача потокораспределения в трубопроводных системах с автоматическими регуляторами в постановке, описывающей различные ситуации работы автоматических регуляторов. Предложен простой и эффективный метод решения поставленной задачи потокораспределения, который не требует вычисления производных и имеет достаточно быструю сходимость. На основании численных экспериментов можно предположить, что число итераций незначительно зависит от размерности задачи (числа узлов и дуг), но существенно зависит от разброса значений гидравлических сопротивлений и в меньшей степени – узловых отборов.

Ключевые слова: системы подачи и распределения воды, трубопроводная система, потокораспределение, давление, пьезометрический напор, математическая модель.

СПИСОК ЛИТЕРАТУРЫ

- Меренков А. П., Хасилев В. Я. Теория гидравлических цепей. – М.: Наука, 1985. 294 с.

- Новицкий Н. Н., Токарев В. В. Релейная методика расчета потокораспределения в гидравлических цепях с регулируемыми параметрами // Известия РАН. Энергетика. 2001. № 2. С. 88–98.

- Епифанов С. П., Зоркальцев В. И. Применение теории двойственности при моделировании гидравлических систем с регуляторами расхода // Известия вузов. Математика. 2010. № 9. С. 76–81.

- Епифанов С. П., Зоркальцев В. И., Медвежонков Д. С. Модель гидравлической сети с регуляторами расхода // Управление большими системами. 2010. № 30.1. С. 286–299.

- Емеличев В. А., Мельников О. И., Сарванов В. И., Тышкевич Р. И. Лекции по теории графов. – М.: Наука, 1990. 384 с.

WATER DISTRIBUTION NETWORKS

Baranchikova N. I., Epifanov S. P., Zorkal'tsev V. I., Kurtin A. V., Obuzdin S. Iu. Flow distribution in water supply and distribution systems with automatic pressure regulators 55

N. I. BARANCHIKOVA1, S. P. EPIFANOV2, V. I. ZORKAL'TSEV3, A. V. KURTIN4, S. Iu. OBUZDIN5

1 Baranchikova Nadezhda Ivanovna, Ph. D. (Engineering), Associate Professor, «Urban Development and Municipal Services» Department, Irkutsk National Research Technical University

83 Lermontova St., 664074, Irkutsk, Russian Federation, tel.: +7 (964) 212-36-37, e-mail: nadin-0105@mail.ru

2 Epifanov Sergei Petrovich, Ph. D. (Physics and Mathematics), Associate Professor, «Urban Development and Municipal Services» Department, Irkutsk National Research Technical University

83 Lermontova St., 664074, Irkutsk, Russian Federation, tel.: +7 (914) 946-86-12, e-mail: epifanov.1954@mail.ru

3 Zorkal'tsev Valerii Ivanovich, Doctor of Engineering, Chief Research Scientist, Melentiev Energy Systems Institute, Siberian Branch of the Russian Academy of Sciences

P.O.B. 309, 130, Lermontova St., 664033, Russian Federation, tel.: +7 (3952) 42-88-27, e-mail: zork@isem.sei.irk.ru

4 Kurtin Andrei Vladimirovich, Deputy Chief Engineer for Operations, «Vodokanal» of Irkutsk, MUE

2 Stanislavskogo St., 664081, Irkutsk, Russian Federation, tel.: +7 (3952) 21-47-99, e-mail: kurtinav@irkvkx.ru

5 Obuzdin Sergei Iur'evich, Deputy Chief Engineer for Automation and Modes, «Vodokanal» of Irkutsk, MUE

2 Stanislavskogo St., 664081, Irkutsk, Russian Federation, tel.: +7 (3952) 21-47-99, e-mail: obuzdinsy@irkvkx.ru

The task of flow distribution in general pipeline systems including the systems of water supply and distribution with automatic pressure regulators is considered. To improve the reliability of the system operation pressure regulators are often installed «downstream» (reducing valves) and «upstream» (tank release valves). Pressure regulators manufactured by Bermad have been installed in the water distribution networks of Irkutsk; that provided for reducing and stabilizing the pressure in some zones. However zones exist that are supplied from several water pipelines with different pressure at the connection points, therefore sometimes complete closing of some regulators occurs (more often with small diameters) or flow reversal is possible, and the water flows through the regulator in the opposite direction. In this case the regulator fails to fulfill its function. Mathematical modeling of such situations is very important because it is them which significantly complicate the operation of the water supply and distribution systems. Mathematical models of automatic «upstream» and «downstream» pressure and pressure drop regulators are suggested with account of their version: normally open or normally closed. The mathematical formulation of the problem of flow distribution in the systems of water supply and distribution with pressure regulators is presented. The critical for the practice task of flow distribution in the pipelines with automatic regulators has been solved for the setting that describes different scenarios of the automatic regulators operation. A simple and efficient method of solving the set task of flow distribution that does not require derivation and has rapid enough convergence is suggested. On the basis of the numerical experiments it is fair to assume that the number of iterations moderately depends on the dimension of the problem (number of nodes and semicircular arcs), considerably depends on spread of hydraulic resistance values and to a lesser extent – node selection.

Key words: systems of water supply and distribution, pipeline system, flow distribution, pressure, piezometer head, mathematical model.

REFERENCES

- Merenkov A. P., Khasilev V. Ia. Teoriia gidravlicheskikh tsepei [Theory of hydraulic circuits. Moscow, Nauka Publ., 1985, 294 p.].

- Novitskii N. N., Tokarev V. V. [Relay method of calculating flow distribution in hydraulic circuits with regulated parameters]. Izvestiia RAN. Energetika, 2001, no. 2, pp. 88–98. (In Russian).

- Epifanov S. P., Zorkal'tsev V. I. [The use of duality theory in simulation of hydraulic systems with flow regulators]. Izvestiia Vuzov. Matematika, 2010, no. 9, pp. 76–81. (In Russian).

- Epifanov S. P., Zorkal'tsev V. I., Medvezhonkov D. S. [Model of a hydraulic network with flow regulators]. Upravlenie Bol'shimi Sistemami, 2010, no. 30.1, pp. 286–299. (In Russian).

- Emelichev V. A., Mel'nikov O. I., Sarvanov V. I., Tyshkevich R. I. Lektsii po teorii grafov [Lectures on theory of graphs. Moscow, Nauka Publ., 1990, 384 p.].