КОНТРОЛЬ КАЧЕСТВА ВОДЫ

УДК 628.1.033:543.3

Нефедова Е. Д., Хямяляйнен М. М., Ковжаровская И. Б., Шевчик Г. В. Риск-ориентированный подход к организации контроля качества питьевой воды 5

Е. Д. НЕФЕДОВА1, М. М. ХЯМЯЛЯЙНЕН2, И. Б. КОВЖАРОВСКАЯ3, Г. В. ШЕВЧИК4

1 Нефедова Елена Дмитриевна, кандидат технических наук, руководитель проекта по межотраслевому балансовому регулированию, ГУП «Водоканал Санкт-Петербурга»

191015, Россия, Санкт-Петербург, Кавалергардская ул., 42, тел.: (812) 372-59-47, e-mail: Nefedova_ED@vodokanal.spb.ru

2 Хямяляйнен Михаил Михайлович, кандидат технических наук, директор департамента схем водоснабжения и водоотведения и гидравлического моделирования, ГУП «Водоканал Санкт-Петербурга»

191015, Россия, Санкт-Петербург, Кавалергардская ул., 42, тел.: (812) 326-53-37, e-mail: Hamalainen_MM@vodokanal.spb.ru

3 Ковжаровская Ирина Борисовна, руководитель технического обследования и оценки эффективности, департамент схем водоснабжения и водоотведения и гидравлического моделирования, ГУП «Водоканал Санкт-Петербурга»

191015, Россия, Санкт-Петербург, Кавалергардская ул., 42, тел.: (812) 372-59-19, e-mail: Kovzharovskaya_IB@vodokanal.spb.ru

4 Шевчик Галина Викторовна, ведущий инженер первой категории, департамент схем водоснабжения и водоотведения и гидравлического моделирования, ГУП «Водоканал Санкт-Петербурга»

191015, Россия, Санкт-Петербург, Кавалергардская ул., 42, тел.: (812) 372-59-19, e-mail: Shevchik_GV2@vodokanal.spb.ru

Обеспечение населения безопасной питьевой водой является одной из приоритетных задач в области водоснабжения. На протяжении многих лет подход к организации контроля качества питьевой воды, подаваемой населению с использованием централизованной системы водоснабжения, не менялся и осуществлялся в соответствии с действующими на то время нормативными правовыми актами. В 2016 г. вышло Постановление Правительства РФ от 17 августа 2016 г. № 806 «О применении риск-ориентированного подхода при организации отдельных видов государственного контроля (надзора), что позволило предприятиям, осуществляющим услуги водоснабжения, использовать риск-ориентированный подход к организации производственного контроля качества питьевой воды с учетом риска для здоровья населения. ГУП «Водоканал Санкт-Петербурга», в соответствии с действующим законодательством, разработало «Программу производственного контроля качества и безопасности питьевой воды в г. Санкт-Петербурге на период 2017–2022 гг.» с учетом риск-ориентированного подхода к организации контроля качества питьевой воды, реализуемого в настоящее время в контрольно-надзорной деятельности органами Роспотребнадзора.

Ключевые слова: водоснабжение, риск-ориентированный подход, контроль качества питьевой воды, интегральная оценка, оценка риска для здоровья.

СПИСОК ЛИТЕРАТУРЫ

- Ерастова Н. В., Мельцер А. В. Гигиеническое обоснование профилактических мер для обеспечения населения г. Санкт-Петербурга питьевой водой высокого качества // Анализ риска здоровью. 2013. № 1. С. 52–57.

- Киселев А. В., Мельцер А. В., Ерастова Н. В. Интегральная оценка питьевой воды по показателям химической безвредности на основе методологии оценки риска для здоровья населения // Профилактическая и клиническая медицина. 2011. № 3. С. 284–288.

WATER QUALITY CONTROL

Nefedova E. D., Hamalainen M. M., Kovzharovskaia I. B., Shevchik G. V. Risk-oriented approach to the arrangement of drinking water quality control 5

## E. D. NEFEDOVA1, M. M. HAMALAINEN2, I. B. KOVZHAROVSKAIA3, G. V. SHEVCHIK4

1 Nefedova Elena Dmitrievna, Ph. D. (Engineering), Senior Manager of the Intersectoral Balance Regulation Project, SUE «Vodokanal of St. Petersburg»

42 Kavalergardskaia St., 191015, St. Petersburg, Russian Federation, tel.: + 7 (812) 372-59-47, e-mail: Nefedova_ED@vodokanal.spb.ru

## 2 Hamalainen Mikhail Mikhailovich, Ph. D. (Engineering), Director, Department of Water Supply and Wastewater Disposal Schemes and Hydraulic Simulation, SUE «Vodokanal of St. Petersburg»

42 Kavalergardskaia St., 191015, St. Petersburg, Russian Federation, tel.: + 7 (812) 326-53-37, e-mail: Hamalainen_MM@vodokanal.spb.ru

3 Kovzharovskaia Irina Borisovna, Senior Manager of Technical Inspection and Efficiency Assessment, Department of Water Supply and Wastewater Disposal Schemes and Hydraulic Simulation, SUE «Vodokanal of St. Petersburg»

42 Kavalergardskaia St., 191015, St. Petersburg, Russian Federation, tel.: + 7 (812) 372-59-19, e-mail: Kovzharovskaya_IB@vodokanal.spb.ru

4 Shevchik Galina Viktorovna, First Rank Leading Engineer, Department of Water Supply and Wastewater Disposal Schemes and Hydraulic Simulation, SUE «Vodokanal of St. Petersburg»

42 Kavalergardskaia St., 191015, St. Petersburg, Russian Federation, tel.: + 7 (812) 372-59-19, e-mail: Shevchik_GV2@vodokanal.spb.ru

Supplying the population with safe drinking water has been one of the priority tasks in the water supply sector. For many years the approach to the arrangement of the quality control of drinking water supplied to the population with the use of a public water supply system has not been changed and has been carried out in full compliance with the active for that time laws and regulations. In 2016 RF Government Decree of August 17, 2016 No. 806 «On using the risk-oriented approach in the arrangement of certain types of the state control (inspection)» was adopted that allowed the providers of water supply services to use the risk-oriented approach in the arrangement of the process control of drinking water quality with account of the population health risk. In accordance with the current legislation SUE «Vodokanal of St. Petersburg» has developed the «Program of the process control of the drinking water quality and safety in the city of Saint-Petersburg for the period of 2017–2022» with account of the risk-oriented approach in the arrangement of drinking water quality control that is currently implemented in the control and supervision by Rospotrebnadzor bodies.

Key words: water supply, risk-oriented approach, drinking water quality control, integral estimation, health risk estimation.

REFERENCES

- Erastova N. V., Mel'tser A. V. [Hygienic substantiation of the preventive measures to supply the population of Saint-Petersburg with high quality drinking water]. Analiz Riska Zdorov'iu, 2013, no. 1, pp. 52–57. (In Russian).

- Kiselev A. V., Mel'tser A. V., Erastova N. V. [Integral estimation of drinking water by the indicators of the chemical safety on the basis of the methodology of the population health risk estimation]. Profilakticheskaia i Klinicheskaia Meditsina, 2011, no. 3, pp. 284–288. (In Russian).

УДК 628.1.03

Пономарев А. П., Подколзин И. В. Морфология и минеральный состав наноструктур в составе питьевой воды 10

А. П. ПОНОМАРЕВ1, И. В. ПОДКОЛЗИН2

1 Пономарев Алексей Петрович, доктор биологических наук, старший научный сотрудник, профессор кафедры биологии и экологии, Владимирский государственный университет им. А. Г. и Н. Г. Столетовых

600900, Россия, г. Владимир, ул. Горького, 87, тел.: (4922) 26-11-46, e-mail: aleksei_pp-44@mail.ru

2 Подколзин Иван Владимирович, кандидат химических наук, заведующий лабораторией химического анализа, Федеральный центр охраны здоровья животных

600901, Россия, г. Владимир, мкр. Юрьевец, тел.: (4922) 26-15-12, e-mail: podkolzin@arriah.ru

В результате исследований отработан метод выделения наноструктур из образцов питьевой воды на основе микро- и ультрафильтрации с использованием микрофильтров (размер пор 450 нм) и полупроницаемой мембраны (размер пор 15 нм). Установлено, что в процессе ультрафильтрации минеральный состав воды до и после прохождения через мембрану не изменяется. При полном переходе воды в ультрафильтрат на мембране задерживаются органоминеральные наноструктуры диаметром от 10 до 200 нм, в состав которых входят макро- и микроэлементы. Комплексный метод микро- и ультрафильтрации обеспечивает эффективное удаление наноструктуры или нанобактерии из питьевой воды при сохранении ее минерального состава, что позволяет рекомендовать данный метод для получении очищенной воды для человека и животных. Анализ минерального состава наноструктур показал, что превалирующими макроэлементами являются кальций и натрий, относительное процентное содержание которых в сумме составляет 60–70%. Эти данные подтверждают предыдущее сообщение авторов о том, что кальций является основным строительным материалом оболочек нанобактерий, выявляемых в крови человека и животных. Принимая во внимание морфологические особенности наноструктур, выделяемых из питьевой воды, их форму и размеры, следует отметить их идентичность трансформированным клеткам нанобактерий из крови человека и животных. Сопоставление результатов анализов трех независимых методов – электронной микроскопии, масс-спектрометрии и хроматографии позволяет утвердиться во мнении, что первоисточником нанобактерий является вода, вместе с которой они попадают в живой организм. При этом исследователи, исключающие существование нанобактерий как живых организмов, признают, что данные наноструктуры могут существенным образом влиять на здоровье человека.

Ключевые слова: питьевая вода, наноструктуры, электронная микроскопия, масс-спектрометрия, жидкостная хроматография, ультрафильтрация.

СПИСОК ЛИТЕРАТУРЫ

- Folk R. L. SEM imaging of bacteria and nannobacteria in carbonate sediments and rocks // Journal of Sedimentary Petrology. 1993. V. 63. P. 990–999.

- Folk R. L. Interaction between bacteria, nanobacteria, and precipitation in hot springs of central Italy // Geog. Phys. Quatern. 1994. V. 48. P. 233–246.

- Волков В. Т., Смирнов Г. В., Медведев М. А., Волкова Н. Н. Нанобактерия (перспективы исследований). – Томск: Твердыня, 2003. 447 с.

- Волков В. Т., Рихванов Л. П., Волкова Н. Н. Нанобактерия в питьевой воде – новейший фактор современной экологии // Вестник Сибирского отделения РАН ВШ, 2005. № 1 (9). С. 20–25.

- Волкова Н. Н. Исследование биоминерализационного геоэкологического фактора в подземных водах Томского района: Дисс. … канд. геол.-минералог. наук. – Томск, 2006. 152 с.

- Смирнов Д. Г. Геоэкологические особенности нанобактерий и разработка способов контроля их количества в различных природных объектах: Дисс. ... канд. техн. наук. – Томск, 2007. 132 с.

- Смирнов Д. Г., Волкова Н. Н. Нанобактерии как биоиндикатор экологического неблагополучия среды или заболевания человека // Известия Томского государственного университета. 2006. Т. 309. № 8. С. 179–182.

- Kajander E. O., Kurpnen J., Akerman K., et al. Nanobacteria from blood the smallest cubturable automously replicating agent on Earth; Science // Nature. 1997. V. 3111. P. 420–428.

- Kajander E. O. Nanobacteria – propagating calcifying nanoparticles // Lettes in Applied Microbiology. 2006. V. 42 (6). P. 549–552.

- Гарасько Е. В., Шиляев Р. Р., Пономарев А. П., Горшенин А. П. Кальцинирующие наночастицы в питьевой воде // Вестник Ивановской медицинской академии. 2011. Т. 16. № 2. С. 14–19.

- Горшенин А. П., Гарасько Е. В., Пономарев А. П. Влияние нанобактерий на качество и безопасность питьевой воды // Водоснабжение и санитарная техника. 2010. № 12. С. 20–24.

- Пономарев А. П. Электронная микроскопия нанобактерий и других представителей микро- и наномира. – Владимир: ИП Журавлева, 2011. 180 с.

- Пономарев А. П., Амелин В. Г., Подколзин И. В. Органоминеральный состав нанобактерий, выделяемых из питьевой воды, крови животных и человека // Нанотехнологии и охрана здоровья. 2012. Т. IV. № 3 (12). С. 52–59.

- Бгатов А. В. Биогенная классификация химических элементов // Философия науки. 1999. № 2 (6). http://www.water.ru/digest/biogen_cllassification.shtml (дата обращения 20.10.2017).

- Raoult D., Drancourt M., Azza S., et al. Nanobacteria are minerale fetuin complex // PloS Pathogens. 2008. V. 4 (2).

- Martel J., Young J. D.-E. Purported nanobacteria in human blood as calcium carbonate nanoparticles // PNAS. 2008. V. 105. № 14. P. 5549–5554.

- Cisar J. O., Xu D.-Q., Thompson J. An alternative explanation of nanobacteria-induced biomineralization // Proceedings of the National Academy of Sciences of the United States of America. 2000. V. 97. P. 368–372.

- Мартел Я., Янг Д. Нанобактерии: взлет и падение // В мире науки. 2010. № 3. С. 47–55.

Ponomarev A. P., Podkolzin I. V. Morphology and mineral composition of nanostructures in drinking water 10

A. P. PONOMAREV1, I. V. PODKOLZIN2

1 Ponomarev Aleksei Petrovich, Doctor of Biological Sciences, Senior Scientific Worker, Professor, Department of Biology and Ecology, Vladimir State University named after Alexander and Nikolay Stoletovs

87 Gorkogo St., 600900, Vladimir, Russian Federation, tel.: +7 (4922) 26-11-46, e-mail: aleksei_pp-44@mail.ru

2 Podkolzin Ivan Vladimirovich, Ph. D. (Chemistry), Chief of the Chemical Analysis Laboratory, Federal Centre for Animal Health

Iurevets Housing Development, 600901, Vladimir, Russian Federation, tel.: +7 (4922) 26-15-12, e-mail: podkolzin@arriah.ru

As a result of the studies the method of excreting nanostructures from drinking water samples on the basis of micro- and ultrafiltration with the use of microfilters (450 nm pore size) and semipermeable membrane (15 nm pore size) was developed. It was found that in the process of ultrafiltration the mineral composition of water before and after passing through the membrane did not change. At the complete water passing into ultrafiltrate the membrane traps organomineral nanostructures with a diameter of 10–200 nm containing macro- and microelements. The integrated method of micro- and ultrafiltration provides for the efficient removal of nanostructure or nanobacteria from drinking water while preserving its mineral composition; this allows recommending this method for obtaining purified water for humans and animals. The analysis of the nanostructure mineral composition showed that calcium and sodium were prevailing macroelements; their total respective percentage content was 60–70%. This data proves the prior information given by the authors that calcium has been the main building blocks of the nanobacteria coat identified in human and animal blood. Taking into account the morphological specific features of the nanostructures excreted from drinking water, their form and dimensions, it should be noted that they are identical to the transformed cells of nanobacteria of human and animal blood. Comparison of the results of the analyses of three independent methods – electron microscopy, mass-spectrometry and chromatography allows being reinforced in view that nanobacteria originate from water that brings them into living organisms. Herewith, research workers that deny the existence of nanobacteria as living organisms recognize the fact that the given nanostructures can produce a significant impact on the human health.

Key words: drinking water, nanostructures, electron microscopy, mass-spectrometry, liquid chromatography, ultrafiltration.

REFERENCES

- Folk R. L. SEM imaging of bacteria and nanobacteria in carbonate sediments and rocks. Journal of Sedimentary Petrology, 1993, v. 63, pp. 990–999.

- Folk R. L. Interaction between bacteria, nanobacteria, and precipitation in hot springs of central Italy. Geog. Phys. Quatern, 1994, v. 48, pp. 233–246.

- Volkov V. T., Smirnov G. V., Medvedev M. A., Volkova N. N. Nanobakteriia (perspektivy issledovanii) [Nanobacteria (research perspectives). Tomsk, Tvedynia Publ., 2003, 447 p.].

- Volkov V. T., Rikhvanov L. P., Volkova N. N. [Nanobacteria in drinking water – the newest factor in the advanced ecology]. Vestnik Sibirskogo Otdeleniia RAN VSH, 2005, no. 1 (9), pp. 20–25. (In Russian).

- Volkova N. N. Issledovanie biomineralizatsionnogo geoekologicheskogo faktora v podzemnykh vodakh Tomskogo raiona [Study of the biomineralization geoecological factor in the underground water of the Tomsk Area. Ph. D. thesis in Geology and Mineralogy. Tomsk, 2006, 152 p.].

- Smirnov D. G. Geoekologicheskie osobennosti nanobakterii i razrabotka sposobov kontrolia ikh kolichestva v razlichnykh prirodnykh ob"ektakh [Geological and ecological specific features of nanobacteria and the development of methods of their quantitative control in different natural objects. Ph. D. thesis in Engineering Science. Tomsk, 2007, 132 p.].

- Smirnov D. G., Volkova N. N. [Nanobacteria as a bioindicator of the environmental problem or human disease]. Izvestiia Tomskogo Gosudarstvennogo Universiteta, 2006, v. 309, no. 8, pp. 179–182. (In Russian).

- Kajander E. O., Kurpnen J., Akerman K., et al. Nanobacteria from blood the smallest cubturable automously replicating agent on Earth; Science. Nature, 1997, v. 3111, pp. 420–428.

- Kajander E. O. Nanobacteria – propagating calcifying nanoparticles. Letters in Applied Microbiology, 2006, v. 42 (6), pp. 549–552.

- Garas'ko E. V., Shiliaev R. R., Ponomarev A. P., Gorshenin A. P. [Calcifying nanoparticles in drinking water]. Vestnik Ivanovskoi Meditsinskoi Akademii, 2011, v. 16, no. 2, pp. 14–19. (In Russian).

- Gorshenin A. P., Garas'ko E. V., Ponomarev A. P. [Impact of nanobacteria on the quality and safety of drinking water]. Vodosnabzhenie i Sanitarnaia Tekhnika, 2010, no. 12, pp. 20–24. (In Russian).

- Ponomarev A. P. Elektronnaia mikroskopiia nanobakterii i drugikh predstavitelei mikro- i nanomira [Electron microscopy of nanobacteria and other representatives of the micro-and nanocosm. Vladimir, IP Zhuravleva Publ., 2011, 180 p.].

- Ponomarev A. P., Amelin V. G., Podkolzin I. V. [Organomineral composition of nanobacteria excreted from drinking water, animal and human blood]. Nanotekhnologii i Okhrana Zdorov'ia, 2012, v. IV, no.3 (12), pp. 52–59. (In Russian).

- Bgatov A. V. [Biogenic classification of chemical elements]. Filosofiia Nauki, 1999, no. 2 (6). http://www.water.ru/digest/biogen_classification.shtml (accessed 20.10.2017).

- Raoult D., Drancourt M., Azza S., et al. Nanobacteria are minerale fetuin complex. PloS Pathogens, 2008, v. 4 (2).

- Martel J., Young J. D.-E. Purported nanobacteria in human blood as calcium carbonate nanoparticles. PNAS. 2008, v. 105, no. 14, pp. 5549–5554.

- Cisar J. O., Xu D.-Q., Thompson J. An alternative explanation of nanobacteria-induced biomineralization. Proceedings of the National Academy of Sciences of the United States of America, 2000, v. 97, pp. 368–372.

- Martel J., Young D. [Nanobacter: ups and downs]. V Mire Nauki, 2010, no. 3, pp. 47–55. (In Russian).

ПИТЬЕВОЕ ВОДОСНАБЖЕНИЕ

УДК 628.161.2:544.723.2

Горелова Е. И., Котов В. В., Данилова Г. Н. Обезжелезивание воды сорбцией на угольно-силикатном композите 18

Е. И. ГОРЕЛОВА1, В. В. КОТОВ2, Г. Н. ДАНИЛОВА3

1 Горелова Елена Ивановна, аспирантка, Воронежский государственный аграрный университет имени императора Петра I

394087, Россия, г. Воронеж, ул. Мичурина, 1, тел.: (919) 188-62-58, e-mail: elena2280@ya.ru

2 Котов Владимир Васильевич, доктор химический наук, профессор кафедры химии, заслуженный деятель науки РФ, Воронежский государственный аграрный университет имени императора Петра I

3 Данилова Галина Николаевна, кандидат технических наук, старший преподаватель кафедры химии, Воронежский государственный аграрный университет имени императора Петра I

394087, Россия, г. Воронеж, ул. Мичурина, 1, тел.: (920) 429-33-34, e-mail: dgn.vsau@mail.ru

С целью очистки воды от примесей синтезирован композит на основе активированного угля БАУ-А, диоксида кремния и каолина с добавкой наночастиц серебра. Состав полученного композита предполагает возможность комплексной очистки воды от органических и неорганических примесей и микроорганизмов, а наличие в его составе силикат- и алюмосиликатсодержащих компонентов – проявление им катионообменных свойств. Методом растровой микроскопии установлено повышение степени дисперсности до наночастиц фрагментов композита после обработки его раствором NaOH. Исследованы процессы сорбции ионов Fe2+ из водных растворов. Установлены физико-химические показатели угольно-силикатного композита: предельная удельная сорбция, коэффициент селективности. Сорбция на 10 г сорбента осуществлялась методом ограниченного объема с циркуляцией 0,4 дм3 раствора с заданной концентрацией. Предложен механизм сорбции Fe2+, заключающийся в том, что при его малых концентрациях имеет место абсорбционный процесс с перераспределением ионов между равновесным раствором и поровым пространством, а при высоких концентрациях – ионообменная сорбция. Выявлены концентрации железа (25 мг/дм3), ниже которых происходит его полное удаление, а также верхний предел концентраций (35 мг/дм3), при котором сорбционная очистка достигает нормативных значений.

Ключевые слова: обезжелезивание, сорбент, угольно-силикатный композит, наночастицы, сорбция, коэффициент селективности.

СПИСОК ЛИТЕРАТУРЫ

- Кутлыева А. Г. Влияние минерального состава питьевой воды на состояние здоровья человека / Современная наука: Теоретический и практический взгляд: сборник статей Международной научно-практической конференции (28 ноября 2015 г., г. Челябинск). В 3 ч. Ч. 3. – Уфа, РИО МЦИИ ОМЕГА САЙНС, 2015. С. 34–37.

- Григорьев Ю. И. Оценка риска загрязнения питьевой воды для здоровья детей Тульской области // Гигиена и санитария. 2014. № 3. С. 23–26.

- Турбинский В. В., Маслюк А. И. Риск для здоровья населения химического состава питьевой воды // Гигиена и санитария. 2011. № 4. С. 23–27.

- Буравлев В. О. Применение новых сорбционных материалов и оборудования для водоподготовки в пищевой промышленности // Ползуновский вестник. 2011. № 3/2. С. 188–191.

- Лурье Ю. Ю. Справочник по аналитической химии. – М.: Химия, 1967. 390 с.

- Цветаева Н. В. Основы регуляции обмена железа // Клиническая онкогематология. 2010. Т. 3. № 3. С. 278–283.

- Heymann E., O’Donnel J. Physicochemical investigation of a cation exchange resin. II. Resin conductance // Journal of Colloid and Interface Science. 1949. V. 4. P. 405–416.

- Шилина А. С. Сорбционная очистка природных и промышленных вод от катионов тяжелых металлов и радионуклидов новым типом высокотемпературного алюмосиликатного адсорбента // Сорбционные и хроматографические процессы. 2010. Т. 10. Вып. 2. С. 237–245.

- Рыбков В. С. СВЧ-термообработка комплексных гранулированных сорбентов на основе природного глауконита // Физика и химия обработки материалов. 2012. № 6. С. 8–93.

- Гимаева А. Р. Сорбция ионов тяжелых металлов из воды активированными углеродными адсорбентами // Сорбционные и хроматографические процессы. 2011. Т. 11. Вып. 3. С. 350–356.

- Водоподготовка: Справочник / Под редакцией С. Е. Беликова. – М.: Аква-Терм, 2007. 240 с.

- Гончиков В. Ч. Фильтрующий материал для очистки воды от железа, марганца, сероводорода // Известия Томского политехнического университета. 2012. Т. 320. № 3. С. 37–40.

- Годымчук А. Ю., Ильин А. П. Исследование сорбционных процессов на природных минералах и их термомодифицированных формах // Химия и технология воды. 2003. № 6. С. 621–632.

- Поляков Е. В. Сорбционные свойства силикатных материалов на основе CaSiO4 // Радиохимия. 2011. Т. 53. № 5. С. 422–426.

DRINKING WATER SUPPLY

Gorelova E. I., Kotov V. V., Danilova G. N. Iron removal from water by sorption on carbonic-silicate composite 18

E. I. GORELOVA1, V. V. KOTOV2, G. N. DANILOVA3

1 Gorelova Elena Ivanovna, Ph. D. Candidate, Voronezh State Agricultural University named after Emperor Peter the Great

1 Michurina St., 394087, Voronezh, Russian Federation, tel.: +7 (919) 188-62-58, e-mail: elena2280@ya.ru

2 Kotov Vladimir Vasil'evich, Doctor of Chemistry, Professor, Chemistry Subdepartment, Honored Science Worker of RF, Voronezh State Agricultural University named after Emperor Peter the Great

3 Danilova Galina Nikolaevna, Ph. D. (Engineering), Senior Lecturer, Chemistry Subdepartment, Voronezh State Agricultural University named after Emperor Peter the Great

1 Michurina St., 394087, Voronezh, Russian Federation, tel.: +7 (920) 429-33-34, e-mail: dgn.vsau@mail.ru

In order to remove impurities from water a composite based on BAU-A activated carbon, silicon dioxide and kaolin with addition of silver nanoparticles was prepared. The composition of the obtained composite supposes the possibility of integrated water treatment with the removal of organic and inorganic impurities and microorganisms; while the presence of silicate- and alumosilicate-containing components in the composite supposes its cation-exchange properties. By scanning microscopy method the increase of the dispersion degree to nanoparticles of the composite fragments after processing with NaOH solution was found. The processes of Fe2+ ions sorption from aqueous solutions were studied. The physical and chemical properties of carbonic-silicate composite: saturate specific sorption and selectivity coefficient were determined. Sorption on 10 g of sorbent was carried out by the limited space method with the circulation 0.4 dm3 solution with the preset concentration. The mechanism of Fe2+ sorption was proposed that involved absorption process with redistribution of ions between equilibrium solution and pore volume at low Fe2+ concentrations; whereas at high Fe2+ concentrations - ion exchange sorption. The lowest iron concentrations were determined (25 mg/dm3) that provided for its complete removal as well as the upper limit of concentrations (35 mg/dm3) that provided for the standard values of sorption purification.

Key words: iron removal, sorbent, carbonic-silicate composite, nanoparticles, sorption, selectivity coefficient.

REFERENCES

1.	Kutlyeva A. G. [Impact of the mineral composition of drinking water on human health]. Contemporary science: Theoretical and practical opinion: collected works of International Research and Practical Conference (November 28, 2015, Chelyabinsk). In 3 parts. Part 3. Ufa, RIO MTSII OMEGA SAINS Publ., 2015, pp. 34–37. (In Russian).

2.	Grigor'ev Iu. I. [Assessment of drinking water pollution risk for children’s health in the Tula Area]. Gigiena i Sanitariia, 2014, no. 3, pp. 23–26. (In Russian).

3.	Turbinskii V. V., Masliuk A. I. [Risk of the drinking water chemical composition for the public health]. Gigiena i Sanitariia, 2011, no. 4, pp. 23–27. (In Russian).

4.	Buravlev V. O. [The use of new sorption materials and equipment for water treatment in food processing industry]. Polzunovskii Vestnik, 2011, no. 3/2, pp. 188–191. (In Russian).

5.	Lur'e Iu. Iu. Spravochnik po analiticheskoi khimii [Analytical Chemistry Reference Book. Moscow, Khmiia Publ., 1967, 390 p.].

6.	Tsvetaeva N. V. [Fundamentals of iron metabolism control]. Klinicheskaia Onkogematologiia, 2010, v. 3, no. 3, pp. 278–283. (In Russian).

7.	Heymann E., O’Donnel J. Physicochemical investigation of a cation exchange resin. II. Resin conductance. Journal of Colloid and Interface Science, 1949, v. 4, pp. 405–416.

8.	Shilina A. S. [Sorption removal of heavy metal cations and radionuclides from natural and industrial water with new type of high temperature alumosilicate adsorbent]. Sorbtsionnye i Khromatograficheskie Protsessy, 2010, v. 10, issue 2, pp. 237–245. (In Russian).

9.	Rybkov V. S. [UHF-thermal processing of complex granulated sorbents based on natural glauconite]. Fizika i Khimiia Obrabotki Materialov, 2012, no. 6. pp. 8–93. (In Russian).

10.	Gimaeva A. R. [Sorption of heavy metal ions from water with activated carbonic adsorbents]. Sorbtsionnye i Khromatograficheskie Protsessy, 2011, v. 11, issue 3, pp. 350–356. (In Russian).

11.	Vodopodgotovka [Water treatment. Reference Book. Under the editorship of S. E. Belikov. Moscow, Akva-Term Publ., 2007, 240 p.].

12.	Gonchikov V. Ch. [Filtration material for removing iron, manganese, hydrogen sulfide from water]. Izvestiia Tomskogo Politekhnicheskogo Universiteta, 2012, v. 320, no. 3, pp. 37–40. (In Russian).

13.	Godymchuk A. Iu., Il'in A. P. [Study of sorption processes on natural minerals and their thermomodified forms]. Khimiia i Tekhnologiia Vody, 2003, no. 6, pp. 621–632. (In Russian).

14.	Poliakov E. V. [Sorption properties of silicate materials based on CaSiO4]. Radiokhimiia, 2011, v. 53, no. 5, pp. 422–426. (In Russian).

## ВОДОЗАБОРЫ

## УДК 628.112.24:622.279.5.001.42

## Ивашечкин В. В., Медведева Ю. А. Совершенствование методов обследования и ремонта пескующих водозаборных скважин 24

## В. В. ИВАШЕЧКИН1, Ю. А. МЕДВЕДЕВА2

1 Ивашечкин Владимир Васильевич, доктор технических наук, профессор, декан факультета энергетического строительства, Белорусский национальный технический университет

220013, Белоруссия, г. Минск, проспект Независимости, 65, тел.: (37517) 267-98-29, e-mail: ivashechkin_vlad@mail.ru

2 Медведева Юлия Александровна, ассистент кафедры «Гидротехническое и энергетическое строительство», Белорусский национальный технический университет

220013, Белоруссия, г. Минск, проспект Независимости, 65, тел.: (37517) 268-84-50, e-mail: bobkovayuliya@mail.ru

Рассматриваются вопросы усовершенствования методов дефектации, ремонта фильтров и обсадных труб пескующих водозаборных скважин. Описывается разработанный прибор, предназначенный для обнаружения мест пескования водозаборных скважин, который состоит из пульта управления и индикатора мутности воды. В процессе обследования прибор перемещается вдоль фильтра при откачке воды из скважины. Прибор успешно прошел лабораторные и полевые испытания. Описывается специальное устройство для упрощения работ по обследованию пескующих скважин. Оно включает погружной насосный агрегат и прибор для обнаружения мест пескования. Предложена конструкция устройства для тампонирования поврежденного участка обсадной трубы или фильтра водозаборной скважины, который включает в себя контейнер для цементного раствора с принудительно открываемым клапаном и несъемную опалубку. Преимуществом данного метода является низкая стоимость и невысокая сложность выполнения ремонтных работ. К недостаткам можно отнести незначительное уменьшение пропускной способности фильтра и дебита скважины.

Ключевые слова: водоснабжение, водозабор, скважина, пескование, фильтр, индикатор мутности, фильтрация, насосный агрегат.

СПИСОК ЛИТЕРАТУРЫ

- Тугай А. М., Прокопчук И. Т. Водоснабжение из подземных источников: Справочник. – Киев: Урожай, 1990. 264 с.

- Алексеев В. С., Волоховский Г. А., Гребенников В. Т. Учебная книга мастера по ремонту скважин на воду. – М.: Колос, 1983. 255 с.

- Башкатов Д. Н. Специальные работы при бурении и оборудовании скважин на воду. – М.: Недра, 1988. 268 с.

- Басарыгин Ю. М. Теория и практика предупреждения осложнений и ремонта скважин при их строительстве и эксплуатации: Справочное пособие. В 6 т. – М.: Недра, 2003. Т. 4. 431 с.

- Ивашечкин В. В., Коледюк Д. М., Машук Ю. С., Иваньков П. П. Диагностика пескования водозаборных скважин / Наука – образованию, производству, экономике: Материалы 9-й Международной научно-практической конференции, Минск, 2011. В 2 т. – Минск, БНТУ, 2011. Т. 2. 163 с.

- Пат. 18713, Республика Беларусь. МПК С 2, Е 21 В 47/00. Устройство для обследования водозаборной скважины / Ивашечкин В. В., Автушко П. А. // Официальный бюллетень. 2014. № 5.

- Пат. 18849, Республика Беларусь. МПК С 2 E 21 B 29/00, E 21 B 33/00. Устройство для тампонирования поврежденного участка обсадной трубы или фильтра водозаборной скважины / Ивашечкин В. В., Автушко П. А. // Официальный бюллетень. 2014. № 6.

WATER INTAKES

Ivashechkin V. V., Medvedeva Iu. A. Improving the methods of inspecting and repairing sanding water abstraction wells 24

V. V. IVASHECHKIN1, Iu. A. MEDVEDEVA2

1 Ivashechkin Vladimir Vasil'evich, Doctor of Engineering, Professor, Dean of Energy Construction Faculty, Belarusian National Technical University

65 Nezavisimosti Ave., 220013, Minsk, Belarus, tel.: (37517) 267-98-29, e-mail: ivashechkin_vlad@mail.ru

2 Medvedeva Iuliia Aleksandrovna, Teaching Assistant, «Hydraulic Engineering and Energy Construction» Subdepartment, Belarusian National Technical University

65 Nezavisimosti Ave., 220013, Minsk, Belarus, tel.: (37517) 268-84-50, e-mail: bobkovayuliya@mail.ru

The issues of improving the methods of fault detection, repair of filters and casing of sanding water abstraction wells are considered. The designed instrument is described that has been developed for detecting places of water abstraction well sanding and consists of the control block and water turbidity indicator. In the process of inspection the instrument is moving along the filter during water pumping out of the well. The instrument has successfully passed laboratory and field tests. To simplify the inspection of sanding wells a special device is described that includes a submerged pumping unit and the instrument for detecting sanding places. The design of the device for plugging a damaged section of casing or filter of the water abstraction well is proposed that includes a container for cement mortar with a force opened valve and permanent shuttering. The advantage of this method is in low cost and relatively simple repair work. Some reduction of the filter capacity and well yield can be attributed to disadvantages.

Key words: water supply, water intake, well, sanding, filter, turbidity indicator, filtration, pumping unit.

REFERENCES

- Tugai A. M., Prokopchuk I. T. Vodosnabzhenie iz podzemnykh istochnikov [Water supply from underground sources. Reference book. Kiev, Urozhai Publ., 1990, 264 p.].

- Alekseev V. S., Volokhovskii G. A., Grebennikov V. T. Uchebnaia kniga mastera po remontu skvazhin na vodu [Textbook of a water well repairman. Moscow, Kolos Publ., 1983, 255 p.]

- Bashkatov D. N. Spetsial'nye raboty pri burenii i oborudovanii skvazhin na vodu [Special services during water well drilling and equipping. Moscow, Nedra Publ., 1988, 268 p.].

- Basarygin Iu. M. Teoriia i praktika preduprezhdeniia oslozhnenii i remonta skvazhin pri ikh stroitel'stve i ekspluatatsii [Theory and practice of preventing complications and well repair during their construction and operation. Reference book. In 6 v. Moscow, Nedra Publ., 2003, v. 4, 431 p.].

- Ivashechkin V. V., Kolediuk D. M., Mashuk Iu. S., Ivan'kov P. P. [Diagnostics of water abstraction well sanding]. Science – for education, production, economy. Proceedings of 9th International Scientific and Practical Conference, Minsk, 2011, in 2 v. Minsk, BNTU Publ., 2011, v. 2, 163 p. (In Russian).

- Ivashechkin V. V., Avtushko P. A. [Pat. 18713, Republic of Belarus. IPC С 2, Е 21 В 47/00. Device for water well inspection]. Ofitsial'nyi Biulleten', 2014, no. 5. (In Russian).

- Ivashechkin V. V., Avtushko P. A. [Pat. 18849, Republic of Belarus. IPC С 2 E 21 B 29/00, E 21 B 33/00. Device for plugging of a damaged water well casing section or filter]. Ofitsial'nyi Biulleten', 2014, no. 6. (In Russian).

ОЧИСТКА СТОЧНЫХ ВОД

УДК 628.166

Ткачев А. А., Баранов В. Л., Пискарева В. М. Сравнительная оценка эксплуатационных затрат на обеззараживание сточных вод при применении хлорирования и УФ-технологии 31

А. А. ТКАЧЕВ1, В. Л. БАРАНОВ2, В. М. ПИСКАРЕВА3

1 Ткачев Андрей Анатольевич, заместитель генерального директора по технологии, НПО «ЛИТ»

107076, Россия, Москва, Краснобогатырская ул., 44, тел.: (495) 733-95-26, e-mail: tkachev@npo.lit.ru

2 Баранов Виктор Львович, начальник технологического отдела, НПО «ЛИТ»

107076, Россия, Москва, Краснобогатырская ул., 44, тел.: (495) 733-95-26, e-mail: baranov@npo.lit.ru

3 Пискарева Виктория Михайловна, ведущий инженер технологического отдела, НПО «ЛИТ»

107076, Россия, Москва, Краснобогатырская ул., 44, тел.: (495) 733-95-26, e-mail: piskareva@npo.lit.ru

Охрана источников питьевой воды – важнейшая комплексная задача. Состояние поверхностных водных объектов во многом определяется количеством в них сточных вод, которые должны быть безопасны в эпидемиологическом отношении в соответствии с законодательством РФ. Наиболее экологически и экономически эффективным методом обеззараживания является использование ультрафиолетового излучения. Для оценки экономической эффективности применения технологии УФ-обеззараживания рассмотрены пять действующих сооружений очистки сточных вод различной производительности. Выполнено сравнение годовых эксплуатационных затрат на хлорирование (без учета затрат на дехлорирование и платы за сброс хлора и хлорорганических соединений) и обработку ультрафиолетовым излучением. Исследования выявили, что экономически эксплуатация станций УФ-обеззараживания более выгодна, чем станций хлорирования для очистных сооружений разной производительности.

Ключевые слова: сточные воды, обеззараживание, ультрафиолет, хлорирование, дехлорирование, эксплуатация систем обеззараживания, экономическое сравнение.

СПИСОК ЛИТЕРАТУРЫ

- Murphy K. L., Zaloum R., Fulford D. Effects of chlorination practice on soluble organics // Water Research. 1975. V. 9. Is. 4. P. 389–396.

- Бивалькевич А. И., Смирнов А. Д. Ультрафиолетовая технология – самый актуальный способ обеззараживания сточных вод // Водоснабжение и канализация. 2015. № 1–2. С. 56–60.

WASTEWATER TREATMENT

Tkachev A. A., Baranov V. L., Piskareva V. M. Comparative assessment of the operating expenditures for wastewater disinfection with the use of chlorination and UV-technology 31

A. A. TKACHEV1, V. L. BARANOV2, V. M. PISKAREVA3

1 Tkachev Andrei Anatol'evich, Deputy General Director for Technology, «LIT» SPA

44 Krasnobogatyrskaia St., 107076, Moscow, Russian Federation, tel.: +7 (495) 733-95-26, e-mail: tkachev@npo.lit.ru

2 Baranov Viktor L'vovich, Chief of the Process Engineering Department, «LIT» SPA

44 Krasnobogatyrskaia St., 107076, Moscow, Russian Federation, tel.: +7 (495) 733-95-26, e-mail: baranov@npo.lit.ru

3 Piskareva Viktoriia Mikhailovna, Leading Engineer of the Process Engineering Department, «LIT» SPA

44 Krasnobogatyrskaia St., 107076, Moscow, Russian Federation, tel.: +7 (495) 733-95-26, e-mail: piskareva@npo.lit.ru

Protection of drinking water sources has been the most important comprehensive task. The state of the surface water bodies is determined to a large extent by wastewater flow in them that should be epidemiologically safe according to the RF legislation. The use of ultraviolet irradiation has been the most environmentally and economically efficient disinfection method. To provide for the assessment of the economic efficiency of UV-disinfection technology five operating wastewater treatment facilities of different capacity were considered. The comparison of the annual operating expenditures for chlorination (exclusively of the expenditures for dechlorination and charge for chlorine and chrloorganic compounds discharge) and UV-irradiation was carried out. The studies showed that operating UV-disinfection units were more cost-effective compared to operating chlorination units at the treatment facilities of different capacity.

Key words: wastewater, disinfection, ultraviolet, chlorination, dechlorination, disinfection system operation, economic comparison.

REFERENCES

- Murphy K. L., Zaloum R., Fulford D. Effects of chlorination practice on soluble organics. Water Research, 1975, v. 9, is. 4, pp. 389–396.

- Bival'kevich A. I., Smirnov A. D. [Ultraviolet technology – a modern method of wastewater disinfection]. Vodosnabzhenie i Kanalizatsiia, 2015, no. 1–2, pp. 56–60. (In Russian).

УДК 628.316.12:628.349.087

Игнаткина Д. О., Москвичева Е. В., Доскина Э. П., Москвичева А. В., Войтюк А. А. Комбинированное устройство для очистки многокомпонентных сточных вод предприятий табачной индустрии 36

Д. О. ИГНАТКИНА1, Е. В. МОСКВИЧЕВА2, Э. П. ДОСКИНА3, А. В. МОСКВИЧЕВА4, А. А. ВОЙТЮК5

1 Игнаткина Дарья Олеговна, ассистент кафедры «Водоснабжение и водоотведение», Волгоградский государственный технический университет

400074, Россия, Волгоград, Академическая ул., 1, тел.: (8442) 96-99-13, e-mail: viv_vgasu@mail.ru

2 Москвичева Елена Викторовна, доктор технических наук, профессор, заведующая кафедрой «Водоснабжение и водоотведение», Волгоградский государственный технический университет

400074, Россия, Волгоград, Академическая ул., 1, тел.: (8442) 96-99-18, e-mail: viv_vgasu@mail.ru

3 Доскина Эльвира Павловна, кандидат технических наук, профессор кафедры «Водоснабжение и водоотведение», Волгоградский государственный технический университет

400074, Россия, Волгоград, Академическая ул., 1, тел.: (8442) 96-99-13, e-mail: viv_vgasu@mail.ru

4 Москвичева Анастасия Владимировна, кандидат технических наук, доцент кафедры «Водоснабжение и водоотведение», Волгоградский государственный технический университет

400074, Россия, Волгоград, Академическая ул., 1, тел.: (8442) 96-99-13, e-mail: viv_vgasu@mail.ru

5 Войтюк Александр Андреевич, заместитель начальника проектного отдела, ООО «СК 34»

400005, Россия, Волгоград, 7-й Гвардейской ул., 11Б, помещение 4-4, тел.: (8442) 23-52-41, e-mail: voytyuk.1987@list.ru

Рассмотрены вопросы очистки производственных сточных вод, образующихся в результате хозяйственной деятельности на табачной фабрике ООО «Империал Тобакко Волга» (Волгоград). На основании полученных данных по выявлению основных источников образования сточных вод на табачной фабрике и их химическому составу были проведены исследования на кафедре «Водоснабжение и водоотведение» Волгоградского государственного технического университета. Их цель – разработка конструктивно простого в исполнении и надежно работающего устройства, занимающего малую площадь, удобного в эксплуатации, а также определение основных технологических параметров его работы, обеспечивающих максимальную эффективность очистки сточных вод от многокомпонентных загрязнений. Представлен годовой объем водопотребления и водоотведения, выявлены основные источники образования сточных вод и определен их химический состав. Предложена технологическая схема локальной очистки производственных сточных вод (табачного и сигаретного цехов), включающая следующие этапы: предварительная механическая очистка, электрохимическая обработка, сорбционная доочистка. Разработано устройство – электролизер-адсорбер, предназначенный для очистки многокомпонентных сточных вод рассматриваемого предприятия. Использование метода электрохимического окисления в сочетании с сорбционной доочисткой на комбинированном устройстве электролизере-адсорбере при правильно подобранных условиях обработки загрязненных стоков позволяет улучшить качество очищенной воды по таким показателям, как БПКполн, ХПК, АПАВ, соединения азота, хлорид-ионы, до нормативов сброса очищенных стоков в городскую канализационную сеть.

Ключевые слова: пищевая промышленность, табачная фабрика, производственные сточные воды, электрохимическое окисление, сорбционная доочистка, электролизер-адсорбер.

СПИСОК ЛИТЕРАТУРЫ

- Маркитанова Л. И., Кисс В. В., Каверзнева Т. Т. Водоснабжение и очистка сточных вод предприятий пищевой промышленности. – СПб., НИУ ИТМО, 2000. 134 с.

- Яромский В. Н. Очистка сточных вод пищевых и перерабатывающих предприятий. – Минск: Издательский центр БГУ, 2009. 171 с.

- Воронов Ю. В., Яковлев С. В. Водоотведение и очистка сточных вод. – М.: Издательство АСВ, 2006. 704 с.

- Юсфин Ю. С., Леонтьев Л. И., Черноусов П. И. Промышленность и окружающая среда. – М.: ИКЦ «Академкнига», 2002. 459 с.

- Краснобородько И. Г., Яковлев С. В. Технология электрохимической очистки воды. − Л.: Стройиздат, 1987. 312 с.

- Климов Е. С., Бузаева М. В. Природные сорбенты и комплексоны в очистке сточных вод. – Ульяновск, УлГТУ, 2011. 201 с.

- Ильин В. И., Колесников В. А. Технология электрохимической очистки промышленных сточных вод с водооборотом // Водоснабжение и санитарная техника. 2005. № 2, ч. 1. С. 21–24.

- Москвичева Е. В., Игнаткина Д. О., Москвичева А. В., Войтюк А. А., Геращенко А. А. Технология получения гранулированного сорбционно-фильтрующего композитного материала на основе отхода производства (на примере ООО «Империал Тобакко Волга» г. Волгограда) // Вестник Волгоградского государственного архитектурно-строительного университета. Строительство и архитектура. 2017. Вып. 49 (68). С. 110–121.

Ignatkina D. O., Moskvicheva E. V., Doskina E. P., Moskvicheva A. V., Voitiuk A. A. Hybrid unit for tobacco industry multicomponent wastewater treatment 36

D. O. IGNATKINA1, E. V. MOSKVICHEVA2, E. P. DOSKINA3, A. V. MOSKVICHEVA4, A. A. VOITIUK5

1 Ignatkina Dar'ia Olegovna, Assistant, «Water Supply and Wastewater Disposal» Subdepartment, Volgograd State Technical University

1 Akademicheskaia St., 400074, Volgograd, Russian Federation, tel.: +7 (8442) 96-99-13, e-mail: viv_vgasu@mail.ru

2 Moskvicheva Elena Viktorovna, Doctor of Engineering, Professor, Head of the «Water Supply and Wastewater Disposal» Subdepartment, Volgograd State Technical University

1 Akademicheskaia St., 400074, Volgograd, Russian Federation, tel.: +7 (8442) 96-99-18, e-mail: viv_vgasu@mail.ru

3 Doskina El'vira Pavlovna, Ph. D. (Engineering), Professor, «Water Supply and Wastewater Disposal» Subdepartment, Volgograd State Technical University

1 Akademicheskaia St., 400074, Volgograd, Russian Federation, tel.: +7 (8442) 96-99-13, e-mail: viv_vgasu@mail.ru

4 Moskvicheva Anastasiia Vladimirovna, Ph. D. (Engineering), Associate Professor, «Water Supply and Wastewater Disposal» Subdepartment, Volgograd State Technical University

1 Akademicheskaia St., 400074, Volgograd, Russian Federation, tel.: +7 (8442) 96-99-13, e-mail: viv_vgasu@mail.ru

5 Voitiuk Aleksandr Andreevich, Chief of the Engineering Department, «SK 34» LLC

11B, office 4-4, 7th Gvardeiskoi St., 400005, Volgograd, Russian Federation, tel.: +7 (8442) 23-52-41, e-mail: voytyuk.1987@list.ru

The issues of purification of «Imperial Tobacco Volga» tobacco factory (Volgograd) industrial wastewater are considered. On the basis of the obtained data on the main sources and chemical composition of wastewater of the tobacco factory the experts of the «Water supply and wastewater disposal» subdepartment of the Volgograd State Technical University carried out studies. The study objective was developing a structurally simple, reliable and serviceable unit with small footprint, that will ensure high quality effluent by removing multicomponent pollutants; another objective of the studies was determining basic process parameters of the operation of the unit that will provide for the maximum treatment efficiency. The annual volume of water consumption and wastewater collection and disposal was presented; the main sources of wastewater generation and its chemical composition were determined. Process flow scheme of the local industrial wastewater treatment (from tobacco and cigarette workshops) is proposed that includes three stages: primary mechanical treatment, electrochemical treatment, sorption tertiary treatment. A unit has been designed – electrolyzer-adsorber for the purification of multicomponent wastewater of the given factory. The use of electrochemical oxidation method in combination with sorption tertiary treatment in the electrolyzer-adsorber hybrid unit under suitable conditions of wastewater treatment provides for improving the following effluent quality parameters: BODfull, COD, anionic surfactants, nitrogen compounds, chlorid-ions, to the standard requirements set to the effluent discharge into the public sewer.

Key words: food industry, tobacco factory, industrial wastewater, electrochemical oxidation, sorption tertiary treatment, electrolyzer-adsorber.

REFERENCES

- Markitanova L. I., Kiss V. V., Kaverzneva T. T. Vodosnabzhenie i ochistka stochnykh vod predpriiatii pishchevoi promyshlennosti [Water supply and purification of wastewater of food industry enterprises. St. Petersburg, NIU ITMO Publ., 2000, 134 p.].

- Iaromskii V. N. Ochistka stochnykh vod pishchevykh i pererabatyvaiushchikh predpriiatii [Purification of wastewater of food processing enterprises. Minsk, Izdatel'skii tsentr BGU Publ., 2009, 171 p.].

- Voronov Iu. V., Iakovlev S. V. Vodootvedenie i ochistka stochnykh vod [Wastewater disposal and treatment. Moscow, ASV Publ., 2006, 704 p.].

- Iusfin Iu. S., Leont'ev L. I., Chernousov P. I. Promyshlennost' i okruzhaiushchaia sreda [Industry and environment. Moscow, IKTS «Akademkniga» Publ., 2002, 459 p.].

- Krasnoborod'ko I. G., Iakovlev S. V. Tekhnologiia elektrokhimicheskoi ochistki vody [Electrochemical water treatment technology. Leningrad, Stroiizdat Publ., 1987, 312 p.].

- Klimov E. S., Buzaeva M. V. Prirodnye sorbenty i kompleksony v ochistke stochnykh vod [Natural sorbents and complexons in wastewater treatment. Ulyanovsk, UlGTU Publ., 2011, 201 p.].

- Il'in V. I., Kolesnikov V. A. [The technology of industrial wastewater electrochemical treatment with water recycling]. Vodosnabzhenie i Sanitarnaia Tekhnika, 2005, no. 2, part 1, pp. 21–24. (In Russian).

- Moskvicheva E. V., Ignatkina D. O., Moskvicheva A. V., Voitiuk A. A., Gerashchenko A. A. [Technology of producing granulated sorption-filtration composite material based on industrial wastes (through the example of «Imperial Tobacco Volga», LLC tobacco factory (Volgograd)]. Vestnik Volgogradskogo Gosudarstvennogo Arkhitekturno-Stroitel'nogo Universiteta. Stroitel'stvo i Arkhitektura, 2017, is. 49 (68), pp. 110–121. (In Russian).

УДК 628.312:54.01

Кофман В. Я. «Новые загрязняющие вещества» (emerging contaminants) в водной среде: зарубежные исследования (обзор) 46

В. Я. КОФМАН*

* Кофман Владимир Яковлевич, старший научный сотрудник, Всероссийский институт научной и технической информации Российской академии наук

125190, Россия, Москва, ул. Усиевича, 20, тел.: (499) 155-43-08, e-mail: kofman16@mail.ru

Рассмотрены научные достижения за период с конца 1990-х годов по настоящее время в области исследований, изучающих присутствие в водной среде «новых загрязняющих веществ» (еmerging contaminants). К ним относятся в основном фармацевтические препараты, запрещенные наркотические средства, ультрафиолетовые фильтры, искусственные заменители сахара, биоциды, их метаболиты и продукты трансформации, а также наноматериалы и микро(нано)пластики. Прогресс в данной сфере стал возможен благодаря внедрению в лабораторную практику аналитических методик на основе жидкостной хроматографии/масс-спектрометрии. Метод позволяет идентифицировать и определять в водной среде полярные загрязняющие вещества, их метаболиты и продукты трансформации на уровне концентраций нг/л. Используемые масс-спектрометрические детекторы характеризуются высокой чувствительностью и селективностью. Представлены усовершенствованные технологические приемы, обеспечивающие более высокую по сравнению с традиционными процессами эффективность удаления новых загрязняющих веществ из сточных вод. Обсуждаются современные подходы к оценке токсичности рассматриваемых загрязняющих веществ в ситуациях, когда низкие концентрации этих веществ в составе смеси препаратов в течение длительного времени воздействуют на нецелевые организмы.

Ключевые слова: водная среда, новые загрязняющие вещества, сточные воды, жидкостная хроматография/масс-спектрометрия, продукты трансформации загрязнений.

Kofman V. Ia. «Emerging contaminants» in water environment: international studies (review) 46

V. Ia. KOFMAN*

* Kofman Vladimir Iakovlevich, Senior Research Fellow, All-Russian Institute of Scientific and Technical Information, Russian Academy of Sciences

20 Usuevicha St., 125190, Moscow, Russian Federation, tel.: +7 (499) 155-43-08, e-mail: kofman16@mail.ru

The scientific developments over a period of the late 1990-ies up to nowadays in the field of studies of the «emerging contaminants» presence in water environment are considered. Emerging contaminants include mainly pharmaceuticals, illegal drugs, ultraviolet filters, artificial sugar substitutes, biocides, their metabolites and transformation products as well as nanomaterials and micro(nano)plastics. The progress in this field is now possible owing to the introduction of analytical methods based on liquid chromatography/mass-spectrometry into the laboratory practice. The method provides for identifying and determining polar contaminants in water environment, their metabolites and transformation products at the concentration level of ng/l. The applied mass-spectrometric detectors are characterized by the high sensitivity and selectivity. The improved techniques that provide for the higher efficiency of removing emerging contaminants from wastewater compared to the traditional processes are presented. Modern approaches to the evaluation of the toxicity of the contaminants under consideration in the situations when low concentrations of these substances in mixtures have a long-term impact on non-target organisms are discussed.

Key words: water environment, emerging contaminants, wastewater, liquid chromatography/mass-spectrometry, contaminant transformation products.

REFERENCES

- Noguera-Oviedo K., Aga D. S. Lessons learned from more than two decades of research on emerging contaminants in the environment. Journal of Hazardous Materials, 2016, v. 316, pp. 242–251.

- Kolpin D. W., Furlong E. T., Meyer M. T., et al. Pharmaceuticals hormones, and other organic wastewater contaminants in U. S. streams, 1999–2000: a national reconnaissance. Environmental Science and Technology, 2002, v. 36, pp. 1202–1211.

- Ankley G. T., Brooks B. W., Huggett D. B., Simpton J. P. Repeating history: Pharmaceuticals in the environment. Environmental Science and Technology, 2007, v. 41, pp. 8211–8217.

- Pal R., Megharaj M., Kirkbride K. P., Naidu R. Illicit drugs and the environment: A review. Science of the Total Environment, 2013, v. 463–464, pp. 1079–1092.

- Sang Z., Jiang Y., Tsoi Y.-K., Leung K. S.-Y. Evaluating the environmental impact of artificial sweeteners: A study of their distributions, photodegradation and toxicities. Water Research, 2014, v. 52, pp. 260–274.

- Diaz-Cruz M. S., Barcelo D. Chemical analysis and ecotoxicological effects of organic UV-absorbing compounds in aquatic ecosystems. Trends in Analytical Chemistry, 2009, v. 28, no. 6, pp. 708–717.

- Liu W.-R., Yang Y.-Y., Liu Y.-S., et al. Biocides in wastewater treatment plants: Mass balance analysis and pollution load estimation. Journal of Hazardous Materials, 2017, v. 329, pp. 310–320.

- Lege S., Guillet G., Merel S., et al. Denatonium – A so far unrecognized but ubiquitous water contaminant? Water Research, 2017, v. 117, pp. 254–260.

- Wiesner M. R., Lowry G. V., Alvarez P., et al. Assessing the risks of manufactured nanomaterials. Environmental Science and Technology, 2006, no. 40, pp. 4336–4345.

- Estahbanati S., Fahrenfeld N. L. Influence of wastewater treatment plant discharges on microplastic concentrations in surface water. Chemosphere, 2016, v. 162, pp. 277–284.

- Da Costa J. P., Santos P. S. M., Duarte A. C., Rocha-Santos T. (Nano)plastics in the environment – Sources, fates and effects. Science of the Total Environment, 2016, v. 566–567, pp. 15–26.

- Cabrera-Lafaurie W. A., Román F. R., Hernández-Maldonado A. J. Single and multi-component adsorption of salicylic acid clofibric acid, carbamazepineand caffeine from water onto transition metal modified and partiallycalcined inorganic–organic pillared clay fixed beds. Journal of Hazardous Materials, 2015, v. 282, pp. 174–182.

- Ong Y. T., Ahmad A. L., Zein S. H. S., Tan S. H. A review on carbon nanotubes in an environmental protection and green engineering perspective. Brazilian Journal of Chemical Engineering, 2010, v. 27, no. 2, pp. 227–242.

- Rodriguez-Mozaz S., Ricart M., Köck-Schulmeyer M., et al. Pharmaceuticals and pesticides in reclaimed water: efficiency assessment of a microfiltration–reverse osmosis (MF–RO) pilot plant. Journal of Hazardous Materials, 2015, v. 282, pp. 165–173.

- Secondes M. F. N., Naddeo V., Belgiorno V., et al. Removal of emerging contaminants by simultaneous application of membrane ultrafiltration, activated carbon adsorption, and ultrasound irradiation. Journal of Hazardous Materials, 2014, v. 264, pp. 342–349.

- Zhao H., Zhang D., Du P., et al. A combination of electro-enzymatic catalysis and electrocoagulation for the removal of endocrine disrupting chemicals from water. Journal of Hazardous Materials, 2015, v. 297, pp. 269–277.

- Keen O. S., Love N. G., Aga D. S., Linden K. G. Biodegradability of iopromide products after UV/H2O2 advanced oxidation. Chemosphere, 2016, v. 144, pp. 989–994.

- Matamoros V., Gutiérrez R., Ferrer I., et al. Capability of microalgae-based wastewater treatment systems to remove emerging organic contaminants: a pilot-scale study. Journal of Hazardous Materials, 2015, v. 288, pp. 34–42.

- Lee E., Shon H. K., Cho J. Role of wetland organic matters as photosensitizer for degradation of micropollutants and metabolites. Journal of Hazardous Materials, 2014, v. 276, pp. 1–9.

- Semblante G. U., Hai F. I., Huang X., et al. Trace organic contaminants in biosolids: impact of conventional wastewater and sludge processing technologies and emerging alternatives. Journal of Hazardous Materials, 2015, v. 300, pp. 1–17.

- Ganesh R., Smeraldi J., Hosseini T., et al. Evaluation of nanocopper removal and toxicity in municipal wastewaters. Environmental Science and Technology, 2010, v. 44, pp. 7808–7813.

- Wang Y., Westerhoff P., Hristovski K. D. Fate and biological effects of silver, titanium dioxide, and C60 (fullerene) nanomaterials during simulated wastewater treatment processes. Journal of Hazardous Materials, 2012, v. 201–202, pp. 16–22.

- Ladner D. A., Steele M., Weir A., et al. Functionalized nanoparticle interactions with polymeric membranes. Journal of Hazardous Materials, 2012, v. 211–222, pp. 288–295.

- Herbort A. F., Schuhen K. A concept for removal of microplastics from the marine environment with innovative host-guest relationships. Environmental Science and Pollution Research, 2017, v. 24, no. 12, pp. 11061–11065.

- Kumar V., Johnson A. C., Nakada N., et al. De-conjugation behavior of conjugated estrogens in the raw sewage, activated sludge and river water. Journal of Hazardous Materials, 2012, v. 227–228, pp. 49–54.

- Prasse C., Wagner M., Schulz R., Ternes T. A. Oxidation of the antiviral drug acyclovir and its biodegradation product carboxy-acyclovir with ozone: kinetics and identification of oxidation products. Environmental Science and Technology, 2012, v. 46, pp. 2169–2178.

- Olmez-Hanci T., Arslan-Alaton I., Genc B. Bisphenol A treatment by the hot persulfate process: oxidation products and acute toxicity. Journal of Hazardous Materials, 2013, v. 263, pp. 283–290.

- Salgado R., Oehmen A., Carvalho G., et al. Biodegradation of clofibric acid and identification of its metabolites. Journal of Hazardous Materials, 2012, v. 241–242, pp. 182–189.

- Heuett N. V., Batchu S. R., Gardinali P. R. Understanding the magnitude of emergent contaminant releases through target screening and metabolite identification using high resolution mass spectrometry: illicit drugs in raw sewage influents. Journal of Hazardous Materials, 2015, v. 282, pp. 41–50.

- Postigo C., Richardson S. D. Transformation of pharmaceuticals during oxidation/disinfection processes in drinking water treatment. Journal of Hazardous Materials, 2014, v. 279, pp. 461–475.

- Scheurer M., Godejohann M., Wick A., et al. Structural elucidation of main ozonation products of the artificial sweetener cyclamate and acesulfame. Environmental Science and Pollution Research International, 2012, v. 19, № 4, pp. 1107–1118.

- Negreira N., Canosa P., Rodriguez I., et al. Study of some UV filters stability in chlorinated water and identification of halogenated by-products by gas-chromatography – mass-spectrometry. Journal of Chromatography A, 2008, v. 1178, no. 1–2, рр. 206–214.

- Nakajima M., Kawakami T., Niino T., et al. Aquatic life of sunscreen agent octyl-4-methoxycinnamate and octyl-4-dimethylaminobenzoate in model swimming pools and the mutagenic assays of their chlorination by-products. Journal of Health Sciences, 2009, v. 55, no. 3, рр. 363–372.

- Eggen R. I. L., Behra R., Burkhardt-Holm P., et al. Challenges in ecotoxicology. Environmental Science and Technology, 2004, v. 38, pp. 58A–64A.

- Lu J., Wu J., Stoffella P. J., Wilson P. C. Uptake and distribution of bisphenol A and nonylphenol in vegetable crops irrigated with reclaimed water. Journal of Hazardous Materials, 2015, v. 283, pp. 865–870.

- Escher B. I., Bramaz N., Eggen R. I. L., Richter M. In vitro assessment of modes of toxic action of pharmaceuticals in aquatic life. Environmental Science and Technology, 2005, v. 39, pp. 3090–3100.

- Fuhrman V. F., Tal A., Arnon S. Why endocrine disrupting chemicals (EDCs) challenge traditional risk assessment and how to respond. Journal of Hazardous Materials, 2015, v. 286, pp. 589–611.

- Vasquez M. I., Lambrianides A., Schneider M., et al. Environmental side effects of pharmaceutical cocktails: what we know and what we should know. Journal of Hazardous Materials, 2014, v. 279, pp. 169–189.

- Gottschalk F., Sonderer T., Scholz R. W., Nowack B. Modeled environmental concentrations of engineered nanomaterials (TiO2, ZnO Ag, CNT, fullerenes) for different regions. Environmental Science and Technology, 2009, v. 43, pp. 9216–9222.

- Aschberger K., Micheletti C., Sokul-Kluttgen B., Christensen F. M. Analysis of currently available data for characterising the risk of engineered nanomaterials to the environment and human health. Lessons learned from four case studies. Environment International, 2011, v. 37, pp. 1143–1156.

- Simate G. S., Lyuke S. E., Ndlovu S., et al. Human health effects of residual carbon nanotubes and traditional water treatment chemicals in drinking water. Environment International, 2012, v. 39, pp. 38–49.

- Chen L., Hu P., Zhang L., et al. Toxicity of graphene oxide and multi-walled carbon nanotubes against human cells and zebrafish. Science of China: Chemistry, 2012, v. 55, pp. 2209–2216.

- Li Q., Mahendra S., Lyon D. Y., et al. Antimicrobial nanomaterials for water disinfection and microbial control: Potential application and implication. Water Research, 2008, v. 42, pp. 4591–4602.

- Jeong E., Im W.-T., Kim D.-H., et al. Different susceptibilities of bacterial community to silver nanoparticles in wastewater treatment systems. Journal of Environmental Science and Health, Part A: Toxic/Hazardous Substances and Environmental Engineering, 2014, v. 49, no. 6, pp. 685–693.

- Lambert S., Sinclair C., Boxall A. Occurence, degradation and effect of polymerbased materials in the environment. Review of Environmental Contamination Toxicology, 2014, v. 227, pp. 1–53.

- Krauss M., Singer H., Hollender J. LC–high resolution MS in environmental analysis: from target screening to the identification of unknowns. Analytical and Bioanalytical Chemistry, 2010, v. 397, pp. 943–951.

- Escher B. I., Fenner K. Recent advances in environmental risk assessment of transformation products. Environmental Science and Technology, 2011, v. 45, pp. 3835–3847.

ТРУБОПРОВОДНЫЕ СИСТЕМЫ

УДК 628.147:64.011.44

Долматов С. Ю., Галкин А. В. Математические модели износа трубопроводов систем водоснабжения 54

С. Ю. ДОЛМАТОВ1, А. В. ГАЛКИН2

1 Долматов Сергей Юрьевич, начальник отдела экономического аудита, ООО «Липецкая трубная компания «Свободный Сокол»

398007, Россия, г. Липецк, Заводская площадь, 1, тел.: (4742) 42-33-09, e-mail: s.dolmatov85@gmail.com

2 Галкин Александр Васильевич, кандидат технических наук, доцент, заведующий кафедрой прикладной математики, Липецкий государственный технический университет

398055, Россия, г. Липецк, Московская ул., 30, тел.: (4742) 32-80-50, e-mail: avgalkin82@mail.ru

В последние годы правительство РФ разрабатывает различные программы, направленные на модернизацию сферы ЖКХ и улучшение качества жизни населения России: капитальный ремонт жилищного фонда, благоустройство дворовой территории, а также реконструкция систем водоснабжения и водоотведения. Однако, несмотря на все эти благие начинания, в конце очередного финансового года жители российских городов все чаще задают вопрос о целесообразности финансовых вложений в данную сферу. Вопрос закономерен и вызван низким качеством произведенного ремонта, что не только не улучшает условия проживания населения, но и приносит новые трудности, связанные с необходимостью исправления ошибок, появившихся в результате работы недобросовестных подрядчиков. Рассматривается вопрос экономической целесообразности существующей модели реконструкции систем водоснабжения российских городов на примере Липецкой области. Описываются математические модели аварийности трубопроводов, анализируются эксплуатационные характеристики трубопроводов из различных материалов и их влияние на качество питьевой воды в регионе. На основании математических расчетов сделан вывод о возможности рационального инвестирования в процесс реновации водопроводных сетей при существенном улучшении их качественных характеристик.

Ключевые слова: реновация водопроводных сетей, математическая модель аварийности, трубы из высокопрочного чугуна с шаровидным графитом, модернизация системы водоснабжения, ограниченное финансирование.

СПИСОК ЛИТЕРАТУРЫ

- Пошлина на трубный полиэтилен может быть обнулена до конца года // Полимерные трубы. 2014. № 4 (46). С. 7.

- Серкова Д. В. Задача управления модернизацией водопроводной сети с целью минимизации аварийности // Управление большими системами. УБС–2017: материалы XIV Всероссийской школы-конференции молодых ученых. – Пермь: Издательство Пермского национального исследовательского политехнического университета, 2017. С. 316–327.

- Половко А. М., Гуров С. В. Основы теории надежности. – СПб.: БХВ-Петербург, 2006. 704 с.

PIPELINE SYSTEMS

Dolmatov S. Iu., Galkin A. V. Mathematical models of water supply pipeline deterioration 54

S. Iu. DOLMATOV1, A. V. GALKIN2

1 Dolmatov Sergei Iur'evich, Chief of the Economic Audit Department, Lipetsk Pipe Company «Svobodny Sokol» LLC

1 Zavodskaia Sq., 398007, Lipetsk, Russian Federation, tel.: +7 (4742) 42-33-09, e-mail: s.dolmatov85@gmail.com

2 Galkin Aleksandr Vasil'evich, Ph. D. (Engineering), Associate Professor, Head of the Subdepartment of Applied Mathematics, Lipetsk State Technical University

30 Moskovskaia St., 398055, Lipetsk, Russian Federation, tel.: +7 (4742) 32-80-50, e-mail: avgalkin82@mail.ru

Over recent years the RF Government has been developing different projects focused on the upgrade of public utilities and improvement of the quality of living of the Russian population: capital renovation of residential properties, improvement of the yard areas as well as reconstruction of water and sanitation systems. However, despite all these good initiatives at the end of the next financial year the residents of the Russian cities increasingly pose the question about the expediency of the financial investments in this sector. The question is legitimate and is caused by the low quality of the renovation made that does not only improve the living conditions but brings new difficulties related to the necessity to eliminate mistakes made by dishonest contractors. The problem of the economic expediency of the existing model of water supply system reconstruction adopted in the Russian cities through the example of the Lipetsk Area is considered. Mathematical models of pipeline failure rates are described; performance characteristics of pipelines made of different materials and their effect on the drinking water quality in the region are analyzed. Based on the mathematical calculations the conclusion is drawn about possible efficient investments in the process of water distribution pipeline renovation while providing for the significant improvement of their qualitative characteristics.

Kew words: water distribution network renovation, mathematical model of failure rate, pipes made of ductile iron, water supply system upgrade, limited financing.

REFERENCES

- [Pipe polyethylene duty can be slashed to zero before the end of the year]. Polimernye Truby, 2014, no. 4 (46), p. 7. (In Russian).

- Serkova D. V. [The task of water network upgrade management with the purpose of reducing the failure rate]. Large-scale system management. UBS–2017. Proceedings of XIV All-Russian School-Conference of Young Professionals. Perm, Izdatel'stvo Permskogo natsional'nogo issledovatel'skogo politekhnicheskogo universiteta Publ., 2017, pp. 316–327. (In Russian).

- Polovko A. M., Gurov S. V. Osnovy teorii nadezhnosti [Fundamentals of the theory of reliability. St. Petersburg, BKHV-Peterburg Publ., 2006, 704 p.].

СИСТЕМЫ ВЕНТИЛЯЦИИ

УДК 697.92

Рымаров А. Г., Агафонова В. В. Персонализированное воздухораспределительное устройство 60

А. Г. РЫМАРОВ1, В. В. АГАФОНОВА2

1 Рымаров Андрей Георгиевич, кандидат технических наук, доцент, Национальный исследовательский Московский государственный строительный университет

129337, Россия, Москва, Ярославское шоссе, 26, тел.: (495) 287-49-14, доб. 2365, e-mail: rymarov@yandex.ru

2 Агафонова Вера Валерьевна, ассистент, Национальный исследовательский Московский государственный строительный университет

129337, Россия, Москва, Ярославское шоссе, 26, тел.: (495) 287-49-14, доб. 2365, e-mail: AgafonovaVV@mgsu.ru

Рассматриваются вопросы развития способов организации воздухообмена в помещениях зданий гражданского назначения. Эти способы связаны с внедрением в практику проектирования систем вентиляции воздухораспределительных устройств нового типа. Проведен анализ эффективности организации воздухообмена в помещениях зданий гражданского назначения с помощью традиционных систем вентиляции. Сделан вывод о необходимости разработки нового типа воздухораспределителя, осуществляющего воздушное душирование рабочих мест быстрозатухающими микроструями. Представлена принципиальная схема разработанного персонализированного воздухораспределительного устройства на основе текстильного вкладыша с микроотверстиями (диаметр 0,2–0,6 мм), позволяющего подавать приточный воздух микроструями с малым импульсом в зону дыхания человека. Преимущество разработанного устройства заключается в возможности подачи требуемого объема воздуха в зону дыхания человека, создавая нормируемые и комфортные параметры микроклимата. Внедрение персонализированного воздухораспределительного устройства позволяет повысить эффективность и экономичность работы системы вентиляции.

Ключевые слова: системы вентиляции, воздухораспределение, вытесняющая вентиляция, персонализированное воздухораспределительное устройство, микроперфорированный текстильный вкладыш.

СПИСОК ЛИТЕРАТУРЫ

- Рымаров А. Г., Агафонова В. В. Особенности истечения воздуха микроструями // Приволжский научный журнал. 2015. № 1. С. 60–64.

- Рымаров А. Г., Агафонова В. В. Исследование возможности применения текстильных воздуховодов в системах вентиляции // Естественные и технические науки. 2015. № 2. С. 141–143.

- Nielsen P. V., Topp C., Sonnichsen M., et al. Air distribution in rooms generated by a textile terminal – comparison with mixing and displacement ventilation // ASHRAE Transaction. 2005. № 8 (1). P. 733–739.

- Nielsen P. V. Personal exposure between people in a room ventilated by textile terminals: with and without personalized ventilation // HVAC&R Research. 2007. № 13 (4). P. 635–644.

- Pinkalla C. Fabric duct air dispersion for HVAC systems // Construction Specifier. 2003. № 56 (6). P. 57–64.

- Наумов А. Л., Капко Д. В. Локальные системы кондиционирования воздуха в офисных зданиях // АВОК: Вентиляция, отопление, кондиционирование воздуха, теплоснабжение и строительная теплофизика. 2012. № 2. С. 14–21.

- Chen F., Chen H., Xie J., et al. Air distribution in room ventilated by fabric air dispersion system // Building and Environment. 2011. № 46 (11). P. 2121–2129.

- Melikov A. K., Cermak R., Majer M. Personalized ventilation: evaluation of different air terminal device // Energy and Buildings. 2002. № 34. P. 829–836.

- Brohus H., Nielsen P. V. Personal exposure in displacement ventilated rooms // Indoor Air. 1996. № 6. P. 157–167.

VENTILATION SYSTEMS

Rymarov A. G., Agafonova V. V. Personalized air terminal device 60

A. G. RYMAROV1, V. V. AGAFONOVA2

1 Rymarov Andrei Georgievich, Ph. D. (Engineering), Associate Professor, Moscow State National Research University of Civil Engineering

26 Yaroslavskoe Hwy., 129337, Moscow, Russian Federation, tel.: +7 (495) 287-49-14, ext. 2365, e-mail: rymarov@yandex.ru

2 Agafonova Vera Valer'evna, Assistant, Moscow State National Research University of Civil Engineering

26 Yaroslavskoe Hwy., 129337, Moscow, Russian Federation, tel.: +7 (495) 287-49-14, ext. 2365, e-mail: AgafonovaVV@mgsu.ru

The issues of developing the methods of designing air exchange in civil building premises are considered. These methods are related to the introduction of innovative air terminal devices in practical design of ventilation systems. The analysis of the efficiency of air exchange arrangement in the premises of civil buildings with the use of traditional ventilation systems was carried out. The conclusion was drawn about the necessity of designing an innovative air terminal that provided for spot cooling of the workplaces with rapidly damping micro jets. The basic configuration is presented of the designed personalized air terminal device based on a textile insert with microholes (0.2–0.6 mm diameter) that will allow supplying inlet air by low-pulse micro jets into the human breathing zone. The advantage of the designed device is in the possibility of supplying the required air amount into the human breathing zone. This will provide for the regulatory and comfortable parameters of the microclimate. The introduction of the personalized air terminal device allows improving the efficiency and cost effectiveness of the ventilation system.

Key words: ventilation systems, air distribution, displacement ventilation, personalized air terminal device, microperforated textile insert.

REFERENCES

- Rymarov A. G., Agafonova V. V. [Specific features of air micro jet escape]. Privolzhskii Nauchnyi Zhurnal, 2015, no. 1, pp. 60–64. (In Russian).

- Rymarov A. G., Agafonova V. V. [Study of the possibility of using textile air ducts in the ventilation systems]. Estestvennye i Tekhnicheskie Nauki, 2015, no. 2, pp. 141–143. (In Russian).

- Nielsen P. V., Topp C., Sonnichsen M., et al. Air distribution in rooms generated by a textile terminal – comparison with mixing and displacement ventilation. ASHRAE Transaction, 2005, no. 8 (1), pp. 733–739.

- Nielsen P. V. Personal exposure between people in a room ventilated by textile terminals: with and without personalized ventilation. HVAC&R Research, 2007, no. 13 (4), pp. 635–644.

- Pinkalla C. Fabric duct air dispersion for HVAC systems. Construction Specifier, 2003, no. 56 (6), pp. 57–64.

- Naumov A. L., Kapko D. V. [Local systems of air conditioning in office buildings]. AVOK: Ventiliatsiia, Otoplenie, Konditsionirovanie Vozdukha, Teplosnabzhenie i Stroitel'naia Teplofizika, 2012, no. 2, pp. 14–21. (In Russian).

- Chen F., Chen H., Xie J., et al. Air distribution in room ventilated by fabric air dispersion system. Building and Environment, 2011, no. 46 (11), pp. 2121–2129.

- Melikov A. K., Cermak R., Majer M. Personalized ventilation: evaluation of different air terminal device. Energy and Buildings, 2002, no. 34, pp. 829–836.

- Brohus H., Nielsen P. V. Personal exposure in displacement ventilated rooms. Indoor Air, 1996, no. 6, pp. 157–167.